| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0044v2 |
| --- | --- |

| Title: | [AHG9] Signalling of Green metadata and Video Decoding Interface SEI messages in VVC specification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard Francois, Yong He, Christian Herglotz, Youngkwon Lim | Tel: Email: | [phone contact number] edouard.francois@interdigital yonghe@qti.qualcomm.com christian.herglotz@fau.de young.L@samsung.com |
| Source: | InterDigital, Qualcom, FAU, Samsung |  |  |

_____________________________

# Abstract

This contribution proposes an update of the VVC specification for signalling the green metadata and video decoding interface (VDI) SEI messages, as specified in ISO/IEC 23001-11 and ISO/IEC 23090-13, respectively.

# Introduction

Green metadata SEI for VVC

SC 29/WG 03 has issued the CD text of ISO/IEC 23001-11 3rd edition in October 2021. ISO/IEC 23001-11 specifies metadata (Green Metadata) that facilitates reduction of energy usage during media consumption. Compared to the previous edition, the 3rd edition of 23001-11 adds the specification of a SEI message carrying green metadata related to Complexity metrics for VVC decoder-power reduction (section 5.2 of 23001-11). Other changes not related to SEI messages are also made.

New SEI specified in Video Decoding Interface (VDI) specification

SC 29/WG 03 has also issued the DIS text of ISO/IEC CD 23090-13 “Video Decoding Interface for Immersive Media” in October 2021. This document specifies the interfaces of a video decoding engine as well as the operations that can be performed by this video decoding engine on elementary streams and metadata. Those interfaces and operations come as extensions of existing video decoding engine specifications exposing hardware video decoding capabilities. The text specifies a new SEI message named VDI SEI envelope.

This contribution proposes an update of the VVC specification to signal these new SEI messages.

# VVC specification update

Replace 2.3 with the following:

2.3	Additional references

–	Rec. ITU-T T.35:2000, Procedure for the allocation of ITU-T defined codes for non standard facilities.

–	ISO/IEC 23001-11 (in force), Information Technology — MPEG Systems technologies — Part 11: Energy-efficient media consumption (green metadata).

–	ISO/IEC 23090-13 (in force), Information technology — Coded representation of immersive media — Part 13: Video Decoding Interface for Immersive Media.

Replace D2.1 with the following:

D.2.1	General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| else if( payloadType = = 4 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 19 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 45 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 56 ) /* specified in ISO/IEC 23001-11 */ |  |
| green_metadata( payloadsize ) |  |
| else if( payloadType = = 129 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| parameter_sets_inclusion_indication( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 137 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| mastering_display_colour_volume( payloadSize ) |  |
| else if( payloadType = = 144 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_light_level_info( payloadSize ) |  |
| else if( payloadType = = 145 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| dependent_rap_indication( payloadSize ) |  |
| else if( payloadType = = 147 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| alternative_transfer_characteristics( payloadSize ) |  |
| else if( payloadType = = 148 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| ambient_viewing_environment( payloadSize ) |  |
| else if( payloadType = = 149 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_colour_volume( payloadSize ) |  |
| else if( payloadType = = 150 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| equirectangular_projection( payloadSize ) |  |
| else if( payloadType = = 153 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| generalized_cubemap_projection( payloadSize ) |  |
| else if( payloadType = = 154 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sphere_rotation( payloadSize ) |  |
| else if( payloadType = = 155 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| regionwise_packing( payloadSize ) |  |
| else if( payloadType = = 156 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| omni_viewport( payloadSize ) |  |
| else if( payloadType = = 168 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_field_info( payloadSize ) |  |
| else if( payloadType = = 203 ) |  |
| subpic_level_info( payloadSize ) |  |
| else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sample_aspect_ratio_info( payloadSize ) |  |
| else if( payloadType = = 205 ) /* specified in ISO/IEC 23090-13 */ |  |
| vdi_sei_envelope( payloadsize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| sei_reserved_payload_extension_data | u(v) |
| sei_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| sei_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

In clause D2.2, replace Table 142 – Persistence scope of SEI messages (informative), with the following:

Table 142 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Picture timing | The AU containing the SEI message |
| DU information | The AU containing the SEI message |
| Scalable nesting | Depending on the scalable-nested SEI messages. Each scalable-nested SEI message has the same persistence scope as if the SEI message was not scalable-nested |
| Subpicture level information | The CVS containing the SLI SEI message and up to but not including the next CVS, in decoding order, that contains an SLI SEI message with different content |
| Green metadata | Specified by the syntax of the SEI message in ISO/IEC 23090-13 |
| VDI envelope | The CVS containing the SEI message |

Ed. Note: the persistence scope of the VDI SEI message has to be verified.

Rename clause D.8	Use of ITU-T H.274 | ISO/IEC 23002-7 VUI parameters

as clause  D.10	Use of ITU-T H.274 | ISO/IEC 23002-7 VUI parameters

Rename clause D.9	Use of ITU-T H.274 | ISO/IEC 23002-7 SEI messages

as clause  D.11	Use of ITU-T H.274 | ISO/IEC 23002-7 SEI messages

Add clause D.8 as follows:

D.8	Green metadata SEI message

D.8.1	Green metadata SEI message syntax

The syntax for green metadata SEI message is specified in ISO/IEC 23001-11 (Green metadata). Green metadata facilitates reduced power consumption in decoders, encoders, displays and in media selection.

D.8.2	Green metadata SEI message semantics

The semantics for green metadata SEI message is specified in ISO/IEC 23001-11 (Green metadata). Green metadata facilitates reduced power consumption in decoders, encoders, displays and in media selection.

Add clause D.9 as follows:

D.9	VDI envelope SEI message

D.9.1	VDI envelope SEI message syntax

The syntax for video decoding interface (VDI) envelope SEI message is specified in ISO/IEC 23090-13 (multi-decoder video interface for immersive media). Multi-decoder video interface for immersive media specifies the control, input, and output interfaces of a video decoding engine as well as the operations that can be performed by this video decoding engine: input formatting for elementary streams, time locking of decoded sequences and metadata streams, output formatting of decoded sequences and metadata streams, and the API for the application to control these operations.

D.9.1	VDI envelope SEI message semantics

The semantics for video decoding interface (VDI) envelope SEI message is specified in ISO/IEC 23090-13 (multi-decoder video interface for immersive media). Multi-decoder video interface for immersive media specifies the control, input, and output interfaces of a video decoding engine as well as the operations that can be performed by this video decoding engine: input formatting for elementary streams, time locking of decoded sequences and metadata streams, output formatting of decoded sequences and metadata streams, and the API for the application to control these operations.

# Patent rights declaration(s)

FAU does not have any current or pending patent rights relating to the technology described in this contribution.

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.

Samsung does not have any current or pending patent rights relating to the technology described in this contribution.

Alternate solution with signalling of SEI messages syntax and semantics in VSEI

VSEI specification update

Replace 2.3 with the following:

  - Additional references

–	ISO 11664-1 (in force), Colorimetry – Part 1: CIE standard colorimetric observers.

–	ISO/IEC 11578:1996, Information technology – Open Systems Interconnection – Remote Procedure Call (RPC).

–	Recommendation ITU-T T.35:2000, Procedure for the allocation of ITU-T defined codes for non standard facilities.

–	IETF RFC 1321 (in force), The MD5 Message-Digest Algorithm.

–	IETF RFC 5646 (in force), Tags for Identifying Languages.

–	ISO/IEC 23001-11 (in force), Information Technology — MPEG Systems technologies — Part 11: Energy-efficient media consumption (green metadata).

–	ISO/IEC 23090-13 (in force), Information technology — Coded representation of immersive media — Part 13: Video Decoding Interface for Immersive Media.

In clauses 8.1, replace Table 4 with the following:

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Filler payload | The PU containing the SEI message |
| User data registered by Rec. ITU-T T.35 | Unspecified |
| User data unregistered | Unspecified |
| Film grain characteristics | Specified by the syntax of the SEI message |
| Frame packing arrangement | Specified by the syntax of the SEI message |
| Display orientation | Specified by the syntax of the SEI message |
| Referenced parameter sets | The CLVS containing the SEI message |
| Decoded picture hash | The PU containing the SEI message |
| Mastering display colour volume | The CLVS containing the SEI message |
| Colour transform information | Specified by the syntax of the SEI message |
| Content light level information | The CLVS containing the SEI message |
| DRAP indication | The picture associated with the SEI message |
| Alternative transfer characteristics | The CLVS containing the SEI message |
| Ambient viewing environment | The CLVS containing the SEI message |
| Content colour volume | Specified by the syntax of the SEI message |
| Equirectangular projection | Specified by the syntax of the SEI message |
| Generalized cubemap projection | Specified by the syntax of the SEI message |
| Sphere rotation | Specified by the syntax of the SEI message |
| Region-wise packing | Specified by the syntax of the SEI message |
| Omnidirectional viewport | Specified by the syntax of the SEI message |
| Alpha channel information | Specified by the syntax of the SEI message |
| Frame-field information | The PU containing the SEI message |
| Depth representation information | Specified by the semantics of the SEI message |
| Multiview acquisition information | The CVS containing the SEI message |
| Multiview view position | The CVS containing the SEI message |
| Annotated regions | Specified by the syntax of the SEI message |
| Sample aspect ratio information | Specified by the syntax of the SEI message |
| Scalability dimension information | The CVS containing the SEI message |
| Extended DRAP indication | The picture associated with the SEI message |
| Green metadata | Specified by the syntax of the SEI message |
| VDI SEI envelope | The CVS containing the SEI message |

Ed. Note: the persistence scope of the VDI SEI message has to be verified.

Add clauses 8.27 as follows:

## 8.27 Green metadata SEI message

### 8.27.1	Green metadata SEI message syntax

The syntax for green metadata SEI message is specified in ISO/IEC 23001-11 (Green metadata). Green metadata facilitates reduced power consumption in decoders, encoders, displays and in media selection.

### 8.27.2	Green metadata SEI message semantics

The semantics for green metadata SEI message is specified in ISO/IEC 23001-11 (Green metadata). Green metadata facilitates reduced power consumption in decoders, encoders, displays and in media selection.

## 8.27 VDI envelope SEI message

### 8.27.1	VDI envelope SEI message syntax

The syntax for video decoding interface (VDI) envelope SEI message is specified in ISO/IEC 23090-13 (multi-decoder video interface for immersive media). Multi-decoder video interface for immersive media specifies the control, input, and output interfaces of a video decoding engine as well as the operations that can be performed by this video decoding engine: input formatting for elementary streams, time locking of decoded sequences and metadata streams, output formatting of decoded sequences and metadata streams, and the API for the application to control these operations.

### 8.27.2	VDI SEI envelope SEI message semantics

The semantics for video decoding interface (VDI) envelope SEI message is specified in ISO/IEC 23090-13 (multi-decoder video interface for immersive media). Multi-decoder video interface for immersive media specifies the control, input, and output interfaces of a video decoding engine as well as the operations that can be performed by this video decoding engine: input formatting for elementary streams, time locking of decoded sequences and metadata streams, output formatting of decoded sequences and metadata streams, and the API for the application to control these operations.