| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0045 |
| --- | --- |

| Title: | AhG11/EE1 viewing preparation report |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, M. Wien, A. Segall, J. Sauer | Email: | elena.alshina@huawei.com mathias.wien@lfb.rwth-aachen.de asegall@sharplabs.com johannes.sauer@huawei.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document reports results of discussion on preparation for viewing test to evaluate Neural Network-based Video Coding (NNVC) technologies under study in JVET.

# Introduction

Teleconference was organized by AhG11/EE1 and conducted at November, 16. Candidates for viewing have been discussed.

It was agreed

- to have 3 viewing sessions for: UHD, 1080p and 720p sequences;

- to include 4 video sequences into each session, 2 QPs for each;

- to include low-delay-B configuration into the viewing for 1080p and 720p sequences.

# Candidates suggested for viewing

Table 1 presents the list of video sequences, configurations and QPs which are recommended to be included into the viewing.

Table 1. Video sequences for the viewing.

| Category | Sequence Name | Resolution | fps | Frames | CFG | QP |
| --- | --- | --- | --- | --- | --- | --- |
| UHD | Tango | 3840x2160p | 60fps | 294 | RA | 39, 42 |
|  | Campfire(*) | 3840x2160p | 30fps | 300 | RA | 36, 39 |
|  | DaylightRoad2 | 3840x2160p | 60fps | 300 | RA | 39, 42 |
|  | CatRobot | 3840x2160p | 60fps | 300 | RA | 39, 42 |
| 1080p | Kimono1(**) | 1920x1080p | 24fps | 240 | LDB | 34, 39 |
|  | red_kayak(***) | 1920x1080p | 30fps | 300 | LDB | 34, 39 |
|  | BQTerrace | 1920x1080p | 60fps | 600 | LDB | 34, 39 |
|  | ArenaOfValor | 1920x1080p | 60fps | 600 | LDB | 34, 39 |
| 720p | KristenAndSara | 1280x720p | 60fps | 600 | LDB | 34, 39 |
|  | Johnny | 1280x720p | 60fps | 600 | LDB | 34, 39 |
|  | FourPeople | 1280x720p | 60fps | 600 | LDB | 34, 39 |
|  | Jets (****) | 1280x720p | 60fps | 550 | LDB | 34, 39 |
|  | Spincalendar (****) | 1280x720p | 60fps | 550 | LDB | 34, 39 |

(*) 300 frame to be encoded, frames 151~300 to be viewed

(**) sequences outside of AhG11 CTC, can be found at ftp://ftp.ient.rwth-aachen.de/ce/JVET-L1031_Deblocking/SRC/

Kimono1_1920x1080_24.yuv (md5sum   4a83005bc719012ac148dd3898e5e4ed)

(***) sequences outside of AhG11 CTC, can be found at ftp://ftp.ient.rwth-aachen.de/ce/JVET-P2025_CCALF/RedKayak/

red_kayak_1-300_1920x1080_30Hz_8bit_420.zip(md5sum  33b276220e7ea308d3fdf98e5fb5d491)

(****) optional sequences outside of AhG11 CTC, can be found at ftp://jvet@ftp.ient.rwth-aachen.de/ahg/candidates/720p

Jets_1280x720_60fps_bt709_420.yuv (md5sum  9b8c26f3ea0276832c41e0a4d0e6d364)

Spincalendar_1280x720_60fps_bt709_420.yuv(md5sum  ba830f17e1948c4134cfcc2de73cdf42)

For remote experts viewing, sections of 5s length will be taken out of the longer sequences for evaluation. The section to be used will be determined in preparation of the viewing sessions.

# EE1 tests suggested for viewing

Following EE1 tests were suggested to be included into viewing.

Table 2. EE1 tests suggested for the viewing.

| EE1 Test | Proposal | Viewing categories | Proponent | Contact |
| --- | --- | --- | --- | --- |
| 1.1 | JVET-X0055 | UHD, 1080p, 720p | Tencent | liqiangwang@tencent.com |
| 1.2.1 | JVET-X0066 | UHD, 1080p, 720p | Bytedance, Qualcomm | yue.li@bytedance.com hongtaow@qti.qualcomm.com |
| 2.1 | JVET-X0117 | UHD | AhG11 | johannes.sauer@huawei.com |
| 2.3.1 | JVET-X0064 | UHD | Bytedance | linchaoyi.cy@bytedance.com |

For tests in super-resolution category sequences are supposed to be encoded in quarter-size (enabling RPR with ScalingFactor = 2.0). Table 3 gives QPs suggested to be used if RPR is enabled (in order to ensure no more than 10% difference to target rate (all sequences 3840x2160p, all coded in RA cfg).

Table 3. Suggested QPs for sequences coded with RPR enabled.

| Sequence Name | CF | Anchor QPs | RPR QP |
| --- | --- | --- | --- |
| Tango | 3840x2160p | 39, 42 | 32, 35 |
| Campfire(*) | 3840x2160p | 36, 39 | 28, 32 |
| DaylightRoad2 | 3840x2160p | 39, 42 | 33, 36 |
| CatRobot | 3840x2160p | 39, 42 | 33, 36 |

# Timeline

7-Jan-2022 Viewing materials are uploaded.

10 and 11-Jan-2022 EE Viewing sessions (time and link to be announced in JVET reflector 2 weeks in advance).