| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0047-v1 |
| --- | --- |

| Title: | AHG4: first report of spatial scalability verification tests |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange F. Urban E. François W. Hamidouche | Email: | philippe.delagrange@Interdigital.com |
| Source: | Interdigital, IETR/INSA |  |  |  |

_____________________________

# Abstract

This contribution reports the results a formal visual quality assessment of VVC in spatial scalability mode, following preparation work described in JVET-X0202. With equal bitrate for base HD and enhancement UHD layer, PSNR-Y based performance is ‑24% vs simulcast, while MOS-based performance is around ‑15%; other layer bitrate allocations are tested in JVET-Y0048. Part of those results were used as a subjective evaluation report of VVC scalable profile in response to the TV 3.0 project Call for Proposals from the Brazilian Digital Terrestrial TV Forum (see JVET-U0128, JVET-V0167 and JVET-X0202). Beyond that, these tests can be used as a basis for JVET verification tests of VVC in scalable mode.

# Introduction

The Brazilian Digital Terrestrial TV Forum (SBTVD Forum) has released in July 2020 a publicly available Call for Proposals [1][2] for the specification of the next generation of the Brazilian Digital Terrestrial Television system, named "TV3.0" project. For the video coding part, VVC has been proposed by several organizations, namely DiBEG, Ateme, Fraunhofer HHI, and Interdigital, as a candidate addressing both single-layer coding and scalable dual-layer coding.

Objective and subjective tests results were requested in response to this CfP. Objective results based on the SBTVD CfP test conditions have already been reported. For visual tests, it was not requested to use the CfP test content. JVET SDR and HDR VVC verification tests reports have been provided for the single-layer coding configuration, while Interdigital and IETR/INSA have performed visual tests to address the dual-layer scalable coding case.

The focus here is on spatial scalability since SBTVD requested performance tests for spatial scalability, but only functional tests for SNR and temporal scalability.

This work can be factorized and re-used as a basis for JVET verification tests of VVC in spatial scalability mode.

# Test settings

The visual assessment is focused on 2x spatial scalability (HD+UHD), with settings similar to SHVC verification test [8].

## Layer settings

VTM is configured in spatial scalability mode, with 2x horizontal/vertical spatial ratio: HD resolution as base layer, and UHD resolution as enhancement layer. For enhancement layer, both intra and inter-layer prediction is allowed, for all pictures.

## Anchors

As in SHVC verification tests, the main anchor is simulcast (bitrates of single layer at HD and UHD resolutions are added), with single layer at UHD resolution as a secondary anchor to evaluate the overhead of multilayer vs single layer.

## Test content

Content with three different luminance transfer characteristics (SDR, HDR PQ, HDR HLG [9]) was initially considered, but HLG was finally skipped to reduce the duration of tests, just keeping the two extremes (SDR and PQ). To enable conducting those tests with a scope broader than SBTVD, e.g. under the umbrella of JVET, no TV3.0 CfP specific test sequence was used to avoid any licensing issue.

UHD content and rate points selected for previous JVET VVC verification tests were reused; this saved content selection time as these content items and rate points had already been identified as relevant for visual tests.

Additional SDR content, not used in VVC verification tests, were also selected.

- The two UHD clips from NetFlix ElFuente used for SHVC verification tests, so that the performance of SHVC and scalable VVC can be compared.

- More content with lower frame rate (25fps) was also considered, to evaluate the impact of shorter IRAP periods on spatial scalability coding performance. Since this is mostly redundant with 30fps content reused from VVC verification tests, some of those 25fps sequences were used for training (see test procedure in section 7) but not for actual evaluation.

### Content preparation process

Content from SHVC tests is actually re-extracted from the NetFlix ElFuente original ProRes source (SDR, BT.709 color) to get a 10-bit test sequence (SHVC test was using 8-bit):

ffmpeg -y -ss 00:03:20.2 -i NETFLIX_ElFuente_4096x2160_28592frames_5994fps.mov -f rawvideo -pix_fmt yuv420p10le -sws_flags lanczos+accurate_rnd+full_chroma_inp -vsync 0 -vf crop=3840:2160:128:0,scale=out_v_chr_pos=128:out_h_chr_pos=0:3840:2160,setsar=1/1 -frames 600 ElFuente-12000.yuv

ffmpeg -y -ss 00:04:08.5817 -i NETFLIX_ElFuente_4096x2160_28592frames_5994fps.mov -f rawvideo -pix_fmt yuv420p10le -sws_flags lanczos+accurate_rnd+full_chroma_inp -vsync 0 -vf crop=3840:2160:128:0,scale=out_v_chr_pos=128:out_h_chr_pos=0:3840:2160,setsar=1/1 -frames 600 ElFuente-14900.yuv

The base layer HD video source is then derived by downscaling the UHD source using the “TAppDownConvertStatic” software provided in the SHM. The downscale filters are exactly the same as those used in VVC RPR, and it is possible to control the chroma location.

Correctness of chroma location for both ffmpeg and TAppDownConvertStatic was carefully checked. Also, it was verified that TAppDownConvertStatic downscale and VVC upscale use the same luma phase (zero phase, meaning first pixel-center aligned, not image-centered), so that inter-layer reference (upscaled HD base layer) is exactly aligned with UHD layer.

For BT.709-color content (chroma location 0):

TAppDownConvertStatic 3840 2160 <input_UHD.yuv> 1920 1080 <output_HD.yuv> 10 10

For BT.2020-color content (chroma location 2):

TAppDownConvertStatic 3840 2160 <input_UHD.yuv> 1920 1080 <output_HD.yuv> 10 10 0 0 0 600 -phase 0 0 0 0

The downscale process described here does not change the colorspace, thus UHD BT.2020 content is downscaled to BT.2020 HD, which is not quite right from an industry standpoint, but allows to separate testing of spatial scalability and color gamut scalability. We did not test color gamut scalability.

## Encoding settings

Settings from SHVC visual tests were reused as much as possible, so that comparison is possible.

VVC scalable encodings were performed by the VTM in version 14.1, with an additional bug fix around Decoded Picture Buffer (DPB) management.

As in SHVC visual tests, a balanced distribution between the base and the enhancement layers bitrate was targeted (EL/BL ratio equal to 1). Note: other ratios are tested in JVET-Y0048, with a significant impact on coding performance, which may be relevant when comparing to other standards.

However, the 0.5s IRAP period used in SHVC visual tests was judged unrealistic in terms of real-life applications, so a 1s IRAP period was used instead, as commonly found in random access configuration. It is noted that a 1s IRAP period on 30fps content has the same effect in terms of Intra-coded picture occurrence as a 0.5 IRAP period on 60fps content.

The Motion-Compensated Temporal Filter (MCTF) pre-processing was not enabled, since its impact on scalable coding performance over simulcast is minor in terms of objective metrics (see section 5.2).

The rate points selection was based on the following approach:

- For content reused from SHVC tests, choose 2 or 3 QPs higher than those used for HEVC, targeting a luma PSNR slightly lower, as it has been observed on VVC visual tests that a PSNR slightly lower compared to HEVC is judged slightly better by viewers due to the properties of VVC. An offset of ‑0.3dB to ‑0.5dB seemed a good rule of thumb.

- For content reused from VVC verification tests, reuse the QPs applying them to the enhancement layer.

- The QP of the base layer is adjusted to be make its bitrate as close as possible to the enhancement layer. This required a bit of trial and error. The QP offset was found to be very often equal to -2 or -3.

# Test logistics and method

## Test lab

As GBTech, Vabtech and RWTH labs were busy with other tests, IETR/INSA kindly accepted to perform the visual tests. This activity was carried-out by IETR DIVAS platform.

## Test method

The tests were conducted using the degradation category rating (DCR) test method [3] with an 11-grade impairment scale as specified in Rec. ITU-R BT.500-14 [4], shown in Figure 1 below:

Figure 1: Meaning of the 11 grades numerical quality scale

Full details are given in section 7.

# Candidate sequences and rate points

## SDR tests sequences

Two sequences from SHVC verification tests [8] and three from VVC SDR verification tests [5] were used. Three additional sequences are used to train the test subjects before the viewing sessions.

Table 1 – SDR test sequences from SHVC VT

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| ElFuente-12000 (Procession) 3840×2160, 60fps, 600 frames | 44 39 33 26 | 46 40 34 28 |  |
| ElFuente-14900 (RiverBoat) 3840×2160, 60fps, 600 frames | 45 39 33 27 | 47 41 35 29 |  |

Table 2 – SDR test sequences from VVC VT

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| Marathon2 3840×2160, 30fps, 300 frames | 43 39 36 31 27 | 45 41 38 33 29 |  |
| MountainBay2 3840×2160, 30fps, 300 frames | 42 37 34 31 28 | 43 39 36 33 30 |  |
| TallBuildings2 3840×2160, 30fps, 300 frames | 42 38 33 28 23 | 44 40 36 32 28 |  |

Table 3 – Extra SDR test sequences from Arte, used for training

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| ChambordRoofs 3840×2160, 25fps, 250 frames | 42 38 34 30 25 | 44 40 36 32 28 |  |
| CorsaireNight 3840×2160, 25fps, 250 frames | 43 38 34 30 25 | 44 40 36 32 28 |  |
| TwoBirdsInDawn 3840×2160, 25fps, 250 frames | 42 38 34 30 26 | 44 40 36 32 28 |  |

## HDR PQ tests sequences

The video sequences and QP points used for HDR VVC verification tests [6] are reused. HDR versions of Arte sequences are used for training.

Table 4 – HDR PQ test sequences from VVC VT

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| Chimera HDR5 (DrivingPOV_HDR) 3840×2160, 60fps, 600 frames | 42 35 30 26 19 | 45 38 32 28 22 |  |
| Meridian HDR2 (Car) 3840×2160, 60fps, 600 frames | 35 31 28 24 18 | 38 34 30 26 22 |  |
| Meridian HDR5 (Beach) 3840×2160, 60fps, 600 frames | 35 31 28 23 18 | 38 34 30 26 22 |  |
| Sparks DirtLot 3840×2160, 60fps, 600 frames | 41 35 30 24 18 | 44 38 33 27 22 |  |
| Sparks Welding 3840×2160, 60fps, 600 frames | 41 35 31 26 20 | 44 38 33 27 22 |  |

# Results from objective metrics

## Bitrate balance between layers

The QP of enhancement layer (layer 1) is the same as UHD single layer, and is selected to span the full range of MOS. When the sequence has already been used for other visual tests, the QPs are just reused when available; this was not the case for SHVC tests where QPs have significantly changed between preparation work [7] and final tests [8], so the QPs were guessed from the bitrates reported in the rate-MOS curves (but some results presented here use the QPs from preparation work when direct comparison is relevant).

The QP of base layer (layer 0) is select to get bitrates that are roughly equal for base layer and enhancement layer (EL/BL ratio = 1). This means that the QP difference between layers may not be constant.

An example with Procession sequence is shown in Table 5, and illustrated in Figure 2, where individual layer bitrate and their sum are displayed in log scale using the quality of layer 1 for all – which is arbitrary for layer 0, but allows a direct comparison of the bitrates.

Table 5 – PSNR_Y results for Procession (no MCTF, RAP 1s)

| QP (L0) | Bitrate (L0) | PSNR_Y (L0) | QP (L1) | Bitrate (L1) | PSNR_Y (L1) | EL/BL | Bitrate (UHD) | PSNR_Y (UHD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | 5630 | 41.73 | 28 | 5216 | 41.05 | 0.93 | 10136 | 41.25 |
| 33 | 2111 | 37.99 | 34 | 2455 | 38.77 | 1.16 | 4190 | 39.01 |
| 39 | 958 | 34.80 | 40 | 1123 | 36.08 | 1.17 | 1934 | 36.43 |
| 44 | 471 | 32.05 | 46 | 393 | 32.89 | 0.83 | 861 | 33.45 |

Figure 2: EL/BL balance for Procession (no MCTF, RAP 1s)

It is noted that the enhanced layer bitrate is impacted in opposite direction by base layer bitrate (i.e. inter-layer prediction benefits from the quality of the base layer). Thus, considering layer 1 alone in comparison to single layer UHD is not sufficient; it is preferred to compare dual layer compared to simulcast. Speaking of simulcast, single layer HD QPs are aligned with those of the base layer, which leads to almost identical coding results: base layer and single layer HD can be considered the same thing.

Figure 3 illustrates the adjustment of base layer QP to reach the target EL/BL bitrate ratio, while keeping enhancement layer QP constant and identical to the single layer UHD reference. In this example, 4 QP offsets (-3 to 0) were tested; Figure 3 shows that the offset giving the bitrate ratio closest to the target (here 1.0) varies over the reference (EL) QP range. The ratio is shown in log scale, with [1/4 .. 4] range.

Figure 3: EL/BL QP offset impact on EL/BL bitrate ratio for CorsaireNight (no MCTF, RAP 1s)

## Impact of MCTF

MCTF does have a positive impact on R/D performance for both single layer coding and dual layer coding, but this impact is equivalent in both cases: very little difference in performance is seen when comparing dual layer coding to simulcast.

Table 6 – Dual-layer performance impact of MCTF (ElFuente sequences, RAP 1s)

| Name | BD-rate vs single layer (PSNR_Y) | BD-rate vs simulcast (PSNR_Y) | BD-rate vs single layer (SSIM_Y) | BD-rate vs simulcast (SSIM_Y) |
| --- | --- | --- | --- | --- |
| Procession (no MCTF) | +17.9% | -21.9% | +18.3% | -21.7% |
| Procession (MCTF) | +17.9% | -21.9% | +18.4% | -21.6% |
| RiverBoat (no MCTF) | +13.5% | -26.6% | +11.7% | -27.6% |
| RiverBoat (MCTF) | +13.6% | -26.6% | +11.7% | -27.6% |

Figure 4: Impact of MCTF on dual-layer coding performance (Procession).
Solid lines: without MCFT, dashed lines: with MCTF

## Impact of Random-Access Period and comparison with SHVC

Using the data from SHVC verification test preparation results [7], where the ElFuente sequences were used, the performance of dual-layer coding of VVC can be compared to SHVC. Initially, a significant difference was found, but after fixing a bug in VTM, the difference is minor (see Figure 7 and grayed cells Table 7), and is probably related to the lack of inter-layer motion prediction in VVC.

A different Random-Access Period was used in SHVC tests (0.5s) and scalable VVC (1s), which has an impact on coding performance: as expected, shorter RAPs, containing more Intra-coded pictures, benefit more from inter-layer prediction, thus improving performance compared to simulcast.

The conclusion is that the gains of dual-layer coding are mostly preserved from SHVC to VVC, which also means that the gains of VVC over HEVC are preserved in dual layer configuration: dual-layer VVC performs much better than SHVC dual layer in the same conditions (e.g. -45% PSNR_Y BD-Rate for Procession).

Table 7 – Dual-layer performance comparison of VVC vs SHVC (ElFuente sequences)

| Name | BD-rate vs single layer (PSNRY) | BD-rate vs simulcast (PSNRY) | BD-rate vs single layer (PSNRYUV) | BD-rate vs simulcast (PSNRYUV) |
| --- | --- | --- | --- | --- |
| Procession (SHVC) | +12.4% | -27.0% | +14.6% | -25.6% |
| Procession (VVC, RAP 0.5s) | +17.0% | -26.0% | +20.2% | -24.0% |
| Procession (VVC, RAP 1s) | +18.0% | -22.8% | +21.5% | -20.5% |
| RiverBoat (SHVC) | +12.2% | -28.4% | +14.5% | -27.0% |
| RiverBoat (VVC, RAP 0.5s) | +13.1% | -28.8% | +17.3% | -26.2% |
| RiverBoat (VVC, RAP 1s) | +14.3% | -26.6% | +18.9% | -23.7% |

Figure 5: Impact of random-access period on VVC EL/BL balance and R/D curves
(Procession). Solid = 1s, Dashed = 0.5s

Figure 6: Impact of random-access period on VVC EL/BL balance and R/D curves
(RiverBoat). Solid = 1s, Dashed = 0.5s

Figure 7: Comparison of VVC (left) and SHVC (right) R/D curves (Procession)

## Bitrates and metrics for SDR sequences

Table 8 – Dual-layer BD-Rate performance of SDR sequences

| Name | MCTF | BD-rate vs single layer (PSNR_Y) | BD-rate vs simulcast (PSNR_Y) | BD-rate vs single layer (SSIM_Y) | BD-rate vs simulcast (SSIM_Y) |
| --- | --- | --- | --- | --- | --- |
| Procession |  | +17.9% | -21.9% | +18.3% | -21.7% |
| Procession | x | +17.9% | -21.9% | +18.4% | -21.6% |
| RiverBoat |  | +13.5% | -26.6% | +11.7% | -27.6% |
| RiverBoat | x | +13.6% | -26.6% | +11.7% | -27.6% |
| Marathon2 |  | +18.8% | -21.1% | +17.6% | -21.6% |
| Marathon2 | x | +19.3% | -21.8% | +18.2% | -22.3% |
| MountainBay2 |  | +12.3% | -26.8% | +10.4% | -28.0% |
| MountainBay2 | x | +12.5% | -27.0% | +10.7% | -28.2% |
| TallBuildings2 |  | +19.4% | -22.9% | +15.6% | -24.2% |
| TallBuildings2 | x | +19.8% | -22.9% | +16.0% | -24.1% |
| ChambordRoofs |  | +24.5% | -21.1% | +24.3% | -21.1% |
| ChambordRoofs | x | +24.8% | -21.3% | +24.6% | -21.2% |
| CorsaireNight |  | +20.5% | -22.5% | +19.3% | -23.1% |
| CorsaireNight | x | +20.7% | -22.7% | +19.5% | -23.3% |
| TwoBirdsInDawn |  | +27.3% | -17.8% | +27.7% | -17.5% |
| TwoBirdsInDawn | x | +27.3% | -18.0% | +27.4% | -17.9% |

The corresponding EL/BL balance and R/D curves are shown in the following figures, with solid lines when coding without MCFT, and dashed lines with MCTF:

Figure 8: R/D curves for Procession

Figure 9: R/D curves for RiverBoat

Figure 10: R/D curves for Marathon2

Figure 11: R/D curves for MountainBay2

Figure 12: R/D curves for TallBuildings2

Figure 13: R/D curves for ChambordRoofs

Figure 14: R/D curves for CorsaireNight

Figure 15: R/D curves for TwoBirdsInDawn

## Bitrates and metrics for HDR PQ sequences

Table 9 – Dual-layer BD-Rate performance of HDR PQ sequences

| Name | MCTF | BD-rate vs single layer (PSNR_Y) | BD-rate vs simulcast (PSNR_Y) | BD-rate vs single layer (SSIM_Y) | BD-rate vs simulcast (SSIM_Y) |
| --- | --- | --- | --- | --- | --- |
| DrivingPOV_HDR |  | +22.2% | -21.9% | +22.0% | -22.8% |
| DrivingPOV_HDR | x | +21.8% | -22.2% | +21.5% | -23.3% |
| MeridianCar_HDR |  | +21.8% | -21.9% | +22.2% | -21.9% |
| MeridianCar_HDR | x | +21.4% | -21.3% | +21.5% | -21.2% |
| MeridianBeach_HDR |  | +18.6% | -23.8% | +17.3% | -24.7% |
| MeridianBeach_HDR | x | +18.5% | -23.0% | +17.4% | -23.6% |
| SparksDirtLot |  | +16.4% | -24.9% | +16.1% | -25.5% |
| SparksDirtLot | x | +16.9% | -25.4% | +16.4% | -25.9% |
| SparksWelding |  | +13.0% | -24.0% | +11.3% | -25.3% |
| SparksWelding | x | +13.2% | -24.6% | +10.9% | -26.4% |

Figure 16: R/D curves for DrivingPOV_HDR

Figure 17: R/D curves for MeridianCar_HDR

Figure 18: R/D curves for MeridianBeach_HDR

Figure 19: R/D curves for SparksDirtLot

Figure 20: R/D curves for SparksWelding

# Visual test report

A formal visual assessment was conducted in INSA/IETR labs with 18 test subjects for SDR category and 18 test subjects for HDR PQ; the mean opinion scores and related confidence intervals (CI95%) are reported in the following.

## MOS scores

Table 10 – Bitrates, MOS scores and confidence intervals (SDR)

| Sequence | Simulcast | Single UHD |  |  | Dual-layer (UHD) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rate [kbps] | Rate [kbps] | MOS | CI | Rate [kbps] | MOS | CI |
| Procession | 15765.74 | 10135.54 | 8.9 | 0.4 | 10846.23 | 8.6 | 0.4 |
|  | 6300.58 | 4189.67 | 8.2 | 0.5 | 4566.23 | 7.2 | 0.5 |
|  | 2892.02 | 1934.47 | 5.6 | 0.7 | 2080.98 | 4.4 | 0.7 |
|  | 1332.55 | 861.37 | 2.1 | 0.8 | 864.58 | 0.9 | 0.4 |
| RiverBoat | 12960.32 | 8285.84 | 8.7 | 0.5 | 9071.85 | 8.8 | 0.4 |
|  | 5508.65 | 3546.96 | 8.1 | 0.6 | 3813.50 | 8.3 | 0.6 |
|  | 2418.16 | 1567.84 | 6.1 | 0.7 | 1661.54 | 5.4 | 0.7 |
|  | 1024.10 | 667.20 | 4.4 | 1.1 | 696.30 | 3.6 | 0.9 |
| Marathon2 | 18582.55 | 12202.95 | 8.5 | 0.4 | 13414.62 | 7.3 | 0.7 |
|  | 9956.96 | 6529.65 | 7.1 | 0.4 | 7070.04 | 6.3 | 0.4 |
|  | 4975.80 | 3300.53 | 4.8 | 0.7 | 3459.18 | 3.7 | 0.9 |
|  | 3256.48 | 2182.04 | 3.1 | 0.8 | 2243.81 | 1.8 | 0.6 |
|  | 1792.65 | 1216.42 | 1.6 | 0.7 | 1220.68 | 0.8 | 0.5 |
| MountainBay2 | 5424.99 | 3561.93 | 7.8 | 0.6 | 3889.44 | 8.0 | 0.6 |
|  | 3227.39 | 2108.30 | 6.3 | 0.6 | 2264.17 | 6.6 | 0.8 |
|  | 1960.71 | 1273.81 | 5.3 | 0.8 | 1345.11 | 5.3 | 0.6 |
|  | 1185.00 | 762.83 | 3.8 | 0.6 | 789.95 | 3.2 | 0.8 |
|  | 569.09 | 384.97 | 1.6 | 0.7 | 400.86 | 1.2 | 0.6 |
| TallBuildings2 | 9298.92 | 5725.23 | 9.0 | 0.4 | 7148.84 | 8.6 | 0.6 |
|  | 5432.78 | 3399.42 | 8.1 | 0.5 | 3928.17 | 7.0 | 0.6 |
|  | 3136.05 | 2008.32 | 6.1 | 0.8 | 2203.02 | 5.1 | 0.9 |
|  | 1748.07 | 1163.00 | 4.0 | 0.9 | 1252.72 | 3.3 | 0.9 |
|  | 995.10 | 662.15 | 1.4 | 0.7 | 696.18 | 0.9 | 0.6 |

Table 11 – Bitrates, MOS scores and confidence intervals (HDR PQ)

| Sequence | Simulcast | Single UHD |  |  | Dual-layer (UHD) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rate [kbps] | Rate [kbps] | MOS | CI | Rate [kbps] | MOS | CI |
| DrivingPOV_HDR | 47386.48 | 30319.37 | 9.2 | 0.3 | 35383.98 | 9.4 | 0.4 |
|  | 14004.89 | 9135.84 | 8.5 | 0.5 | 10784.45 | 8.5 | 0.6 |
|  | 7292.85 | 4766.21 | 7.1 | 0.8 | 5585.26 | 6.7 | 0.8 |
|  | 3184.98 | 1984.10 | 5.5 | 0.9 | 2222.37 | 5.7 | 0.7 |
|  | 1239.89 | 790.59 | 1.8 | 0.8 | 888.13 | 1.4 | 0.7 |
| MeridianCar_HDR | 10664.87 | 6699.78 | 8.1 | 0.7 | 7227.86 | 8.2 | 0.6 |
|  | 3383.14 | 2230.30 | 7.2 | 0.5 | 2578.16 | 6.6 | 0.8 |
|  | 1672.84 | 1094.71 | 5.6 | 0.7 | 1245.55 | 5.0 | 0.7 |
|  | 930.86 | 584.33 | 3.8 | 0.7 | 632.14 | 3.8 | 0.7 |
|  | 511.12 | 323.22 | 1.7 | 0.7 | 354.40 | 1.4 | 0.6 |
| MeridianBeach_HDR | 13286.38 | 8611.28 | 8.6 | 0.4 | 9686.67 | 9.0 | 0.4 |
|  | 4677.98 | 2970.63 | 8.4 | 0.6 | 3220.25 | 7.8 | 0.6 |
|  | 2023.88 | 1354.05 | 7.1 | 0.6 | 1493.95 | 5.5 | 0.8 |
|  | 1071.41 | 679.35 | 4.1 | 0.8 | 716.36 | 3.2 | 0.9 |
|  | 566.03 | 359.91 | 1.6 | 0.7 | 384.10 | 1.1 | 0.6 |
| SparksDirtLot | 23173.83 | 14966.41 | 8.3 | 0.6 | 16912.95 | 8.7 | 0.4 |
|  | 6331.22 | 4187.56 | 7.6 | 0.6 | 4753.61 | 6.9 | 0.7 |
|  | 2074.95 | 1332.54 | 4.3 | 0.7 | 1523.60 | 4.6 | 0.7 |
|  | 931.68 | 594.68 | 2.7 | 0.8 | 687.16 | 2.1 | 0.6 |
|  | 360.26 | 230.37 | 0.2 | 0.3 | 272.19 | 0.4 | 0.3 |
| SparksWelding | 38626.16 | 25818.14 | 9.0 | 0.4 | 27000.52 | 8.9 | 0.5 |
|  | 15536.97 | 10529.79 | 8.1 | 0.6 | 11166.11 | 7.9 | 0.7 |
|  | 6381.50 | 4306.63 | 6.3 | 0.5 | 4479.96 | 6.4 | 0.7 |
|  | 2969.09 | 1963.40 | 3.9 | 0.8 | 1905.04 | 3.4 | 0.8 |
|  | 990.26 | 685.34 | 0.8 | 0.4 | 698.07 | 0.5 | 0.4 |

## Rate-MOS plots

The measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bit rate of the corresponding bitstream. The ±95% confidence intervals for the MOS values are indicated with a color shade enclosing each curve.

Figure 21: Rate-MOS plot for Procession	Figure 22: Rate-MOS plot for RiverBoat

Figure 23: Rate-MOS plot for Marathon2	Figure 24: Rate-MOS plot for MountainBay2

Figure 25: Rate-MOS plot for TallBuildings2 	Figure 26: Rate-MOS plot for DrivingPOV_HDR

Figure 27: Rate-MOS plot for MeridianCar_HDR	Figure 28: Rate-MOS plot for MeridianBeach_HDR

Figure 29: Rate-MOS plot for SparksDirtLot	Figure 30: Rate-MOS plot for SparksWelding

Though confidence intervals are often overlapping, the visual quality of dual-layer coding is consistently judged lower than the UHD from simulcast at the same QP. However, since the bitrate of dual-layer is also lower, it keeps an advantage. The scores seem to correlate rather well with the objective metrics, but with a larger penalty for dual-layer. Though very inaccurate because of relatively large confidence intervals, the average of MOS BD-rates is less favorable than objective metrics PSNR-Y BD-rates vs single layer / simulcast (+30/-15% for MOS and +17/-24% for PSNR-Y).

The lower visual scores for dual layer compared with single layer at the same QP point could indicate that the base layer quality is too low (base layer QP too high), and that other EL/BL bitrate ratios should be tried, e.g. ½ or ¼: dual-layer coding probably brings a higher benefit with a shallow enhancement layer: this is the object of JVET-Y0048.

# Detailed test procedure

An evaluation method very similar to VVC verification tests has been adopted. The following description is based on JVET-W2020 [6] with some adaptations.

## Test method

The test method adopted for this evaluation was degradation category rating (DCR) [3], also called DSIS method in [4], but with a 11-level quality scale.

### Degradation Category Rating (DCR)

This test method is commonly adopted when the material to be evaluated shows a range of visual quality that well distributes across all quality scales. All the video material used for these tests consist of video clips of 10 seconds duration.

This method was used under the schema of evaluation of the quality; for this reason, the 11 levels quality rating scale described in table 2-4 p.57 of [4] and reproduced in Figure 1 was adopted, ranging from "0" (lowest quality) to "10".

The structure of the basic test cell (BTC) of the DCR method was made of two consecutive presentations of the video clip under test; at first the original version of the video clip was displayed, immediately afterwards the coded version of the video clip was presented; then a message displayed for 6 seconds asking the viewers to vote. The presentation of the video clips was preceded by a two-seconds mid-grey screen displaying “Reference” for the original and “Test” for the coded version of the sequence under test.

Vote n

Reference video

Impaired video

2s

10s

2s

10s

6s

Figure 31: DCR basic test cell

Ten different randomizations of BTCs were generated, and assigned in turn to test subjects.

## How to express the visual quality opinion with DCR

The viewers were asked to express their vote by putting a mark on a voting sheet.

The voting sheet was made of a section for each BTC; each section having 11 boxes labelled 0 to 10, with rate meaning recalled on the side, and the viewer requested to tick one of them. By writing a score of “10”, the subject expressed an opinion of “best” quality, while by writing a score of “0” the subject expressed an opinion of “worst” quality, as shown in Figure 1.

The vote had to be written when the message "Vote x / N" appeared on the screen. The number "x" was a numerical progressive indication on the screen aiming to help the viewing subjects to use the appropriate box of the voting sheet.

## Training and stabilization phase

The outcome of a test (e.g. confidence intervals) is highly dependent on proper training of the test subjects.

For this purpose, each subject was trained by means of a short practice (training) session demonstrating the range of qualities to be expected in the test. The training session used 3 video sequences that were different from the actual test sessions; it consisted of 5 test cells demonstrating different levels of degradation. A quality rating could be suggested orally by the experimenter. The test subject could be given hints if he did not detect any impairments or had a very different opinion.

The subjects were then presented with the sequences under test, with one BTC per sequence, at mid-range quality.

As suggested in ITU-R BT.500-14, test sessions started with a stabilization phase made of three BTCs, containing one sample of best quality, one of the worst qualities and one of medium quality, which were duplicated at the beginning of the test session. This way, the test subjects had an immediate impression of the quality range they were expected to evaluate during that session.

The scores of the stabilization phase were discarded. The test subject was not made aware of the stabilization phase, but was told that in order to check the consistency of votes, some test items were repeated and that the reference was also tested.

## Laboratory setup

The laboratory for subjective assessments was arranged according to [3], except for the selection of the display and the video play-out system.

The display was an LG OLED55C1 model 2021 TV, capable of displaying HDR UHD. Video material for viewing sessions was encoded in HEVC high quality (using ffmpeg with crf=15 setting), packaged in a .ts file stored on a USB storage device, which was decoded directly by the TV. This ruled out potential HDR settings error by embedding signal format information into the encoded video stream, and allowed mixing SDR and HDR within the same session (the TV was detecting the change and reacting accordingly). Encoding quality was checked not to change the perception of reference or tested videos.

## Viewing environment

The viewing distance was 1.5H, where H is equal to the height of the active part of the screen.

The test laboratory was protected from external visual or audio pollution. Internal general light was low (just enough to allow the viewing subjects to fill out the scoring sheets) and a uniform light was placed behind the monitor, in a way no direct light hit the viewing subjects seated in front of the screen; the light behind the monitor was dimmed to an intensity as specified in Table 4 of Recommendation ITU-T P.911 (“Typical viewing and listening conditions as used in audio-visual quality assessment”). No other light source was admitted, and in particular any light source directed to the screen or creating reflections.

## Overall test effort and subjects’ involvement

Each viewing session did not run for more than 15 minutes and the same viewing subject did not participate to the test run for more than one hour in total.

Young people were hired as test subjects, selecting them for an age from 16 to 30, mostly students of scientific faculties, screened for normal visual acuity and color blindness using the Ishihara and Snellen vision tests [4]. Viewing subjects were compensated for their participation to the testing activities.

## Statistical analysis and presentation of the results

The data collected from the score sheets, filled out by the viewing subjects, were stored in an Excel spreadsheet. For each coding condition the Mean Opinion Score (MOS) and associated Confidence Interval (CI) values were computed in the spreadsheets.

The MOS and CI values were used to draw graphs. The graphs were drawn grouping the results for each video test sequence. No graph grouping results from different video sequences was considered.

The subject reliability was assessed from MOS correlation coefficient, and some outliers could be ignored (actually none for this specific test). A screening method is described in section A1-2.3 of [4].

# References

- Call for Proposals: TV 3.0 Project, Brazilian Digital Terrestrial Television System Forum, July 2020,  https://forumsbtvd.org.br/wp-content/uploads/2020/07/SBTVDTV-3-0-CfP.pdf

- CfP Phase2 / Testing and Evaluation: TV3.0 Project, Brazilian Digital Terrestrial Television System Forum, March 2021, https://forumsbtvd.org.br/wp-content/uploads/2021/03/SBTVD-TV_3_0-P2_TE_2021-03-15.pdf

- Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.

- M. Wien, V. Baroncini, “VVC Verification Test Report for Ultra High Definition (UHD) Standard Dynamic Range (SDR) Video Content” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JVET-T2020, 20th JVET meeting, teleconference, October 2020.

- M. Wien, V. Baroncini, “VVC Verification Test Report for High Dynamic Range Video Content” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JVET-W2020, 23rd JVET meeting, teleconference, July 2021.

- Y. He, Y. Ye, F. Henry, Y.K. Wang, V. Baroncini, “Proposed SHVC verification test draft plan” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JCTVC-V0069, 22nd meeting, Geneva, CH, October 2015.

- Y. He, V. Baroncini, Y.K. Wang, “SHVC verification test report” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 document JCTVC-W1004, 23rd meeting, San Diego, USA, February 2016.

- ITU-R, “Image parameter values for high dynamic range television for use in production and international program exchange”, Rec. ITU-R BT.2100-2, 2018.