| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0048-v1 |
| --- | --- |

| Title: | AHG10: study of layer bitrate allocation for spatial scalability in VTM |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange F. Urban G. Marquant | Email: | philippe.delagrange@interdigital.com |
| Source: | Interdigital |  |  |  |

_____________________________

# Abstract

First results of VVC spatial scalability visual performance tests have been reported in JVET-Y0047, with equal bitrate for HD base layer and UHD enhancement layer. However, it was noticed that the bitrate balance has a significant effect on coding performance, in favor of more bitrate in the base layer. This contribution reports experiments with various bitrate allocations, and reports gains and lower encoding time over single layer in some cases.

# Introduction

In JVET-Y0047, VVC spatial scalability was tested in conditions similar to SHVC verification test, to enable comparison. More specifically, base-layer (BL) and enhancement layer (EL) bitrates are matched (EL/BL ratio equals 1). However, exploring other bitrate allocation was welcome, as it was noted that it could have a significant impact on performance. This contribution attempts to explore the full range, between all bitrate on the HD base layer (i.e. RPR), and all bitrate on the UHD layer (i.e. single layer).

# Test settings

The same configuration based on CTCs as in JVET-Y0047 was used, with MCTF disabled, but varying bitrate allocation between HD base layer and UHD enhancement layer.

## Layer settings

Spatial scalability with horizontal/vertical spatial ratio 2x, with HD resolution as base layer (BL) and UHD resolution as enhancement layer (EL).

## Anchors

Contrary to JVET-Y0047, the only anchor considered here is UHD single layer, as we focus on spatial scalability as a coding performance tool, the two extremes of EL/BL bitrate allocation being single layer on one side (no bitrate on base layer), and pure upscale (RPR) on the other side (no bitrate on enhancement layer). Here we approximate RPR with maximum QP (63) on enhancement layer.

## Test content

The same test content as in JVET-Y0047 is used, so that results are consistent across the different tests.

## Encoding settings

VVC encodings were performed by the VTM in version 14.1, with an additional bug fix around Decoded Picture Buffer (DPB) management for correct dual-layer behaviour.

RA CTC configuration is used, except for MCTF which is disabled, as it has minor impact on coding performance of spatial scalability compared to single layer, as discussed in JVET-Y0047.

The EL/BL bitrate allocation is varied by keeping EL QP constant and equal to UHD single layer rate points, and varying BL QP. RPR is approximated by setting EL QP to 63, and varying BL QP on a wide scale.

The EL rate points are reused from JVET-Y0047, adding a lower bitrate point or two when relevant to show the benefit of spatial scalability.

# Sequences and rate points

As in JVET-Y0047, two sequences from SHVC verification tests and three from VVC SDR verification tests are used, plus three additional sequences from Arte.

Table 1 – SDR test sequences from SHVC VT (60fps)

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| ElFuente-12000 (Procession) 3840×2160, 60fps, 600 frames | 16 - 52 42 - 47 36 - 47 30 - 40 24 - 33 18 - 27 | 63 50 46 40 34 28 |  |
| ElFuente-14900 (RiverBoat) 3840×2160, 60fps, 600 frames | 16 - 52 35 - 47 37 - 48 31 - 41 25 - 34 19 - 28 | 63 50 47 41 35 29 |  |

Table 2 – SDR test sequences from VVC VT (30fps)

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| Marathon2 3840×2160, 30fps, 300 frames | 16-52 36-44 35-44 31-40 28-37 23-32 19-28 | 63 49 45 41 38 33 29 |  |
| MountainBay2 3840×2160, 30fps, 300 frames | 16-52 34-40 33-42 29-38 26-35 23-32 20-29 | 63 46 43 39 36 33 30 |  |
| TallBuildings2 3840×2160, 30fps, 300 frames | 16-52 36-47 32-43 28-39 20-35 20-30 16-25 | 63 48 44 40 36 32 28 |  |

Table 3 – SDR test sequences from Arte (25fps)

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| CorsaireJoy 3840×2160, 25fps, 250 frames | 16-48 38-43 38-43 38-66 34-40 30-36 26-32 22-28 | 63 52 48 44 40 36 32 28 |  |
| TwoBirdsInDawn 3840×2160, 25fps, 250 frames |  | 63 48 44 40 36 32 28 |  |
| AngelInSunrise 3840×2160, 25fps, 250 frames |  | 63 46 44 40 36 32 28 |  |

# Selection of QPs and bitrate balance between layers

In JVET-Y0047, it is noted that the enhanced layer bitrate is impacted in opposite direction by base layer bitrate (i.e. inter-layer prediction benefits from the quality of the base layer). For comparison with SHVC verification test, the same conditions (equal BL and EL bitrate) were used, and dual layer was compared with simulcast and single layer. However, it is noted that the bitrate allocation between layer has a significant impact on performance, and that exploring other settings is desirable. This is the purpose of the following.

The QP of enhancement layer (QPEL) is the same as UHD single layer (QPSL), and reused from JVET-Y0047, but adding one or two points in the low bitrate end when relevant.

Then, for each QPEL, the QP of base layer (QPBL) is varied to observe the impact on performance compared to single layer.

Figure 1 illustrates the balance of bitrates between layers explored for a specific sequence.

<TBC>

Figure 1: EL & BL bitrates explored for XXX

## Bitrate balance between layers: impact on quality

Figure 2 illustrates the impact on dual-layer coding performance of the bitrate balance between layers: it shows PSNRY and SSIMY RD curves of a specific video sequence (CorsaireJoy), where single layer is displayed as a continuous curve, and dual layer as “C” of “6”-shaped traces that follow the QPBL sweeps for each QPEL point (that matches QPSL). Additionally, the large QPBL sweep with QPEL=63, simulating RPR, is displayed as a dashed yellow curve. Each QPBL sweep is expected to start on the single layer curve (with high enough QPBL), and asymptotically end on the pseudo-RPR curve (with low enough QPBL).

Figure 3 is a zoom of figure 2 for QPEL = 44.

Figure 2: RD curves (CorsaireJoy)

Figure 3: zoom of figure2

It can be seen that for low bitrate, dual-layer coding can outperform both single-layer and RPR. This is visible on SSIM even at higher bitrates, which can indicate that the visual benefit is significant.

In Figure 2 and Figure 3, it can be observed that for a specific bitrate, two points with different distortion can be obtained (with a difference as large as 0.7 dB in PSNRY), depending on layer bitrate allocation.

We can also notice that, once inter-layer prediction is effective (i.e. BL has a sufficient quality), the UHD layer quality increases with the quality of the base layer.

Overall, we observe that, except for low-motion highly detailed sequences at high bitrate, dual-layer coding with optimized layer bitrate allocation can be on par or outperform RPR and single layer coding, while being faster to encode and providing the multi-layer flexibility (e.g. base layer compatibility with lower-profile decoders).

## Bitrates and metrics for SDR sequences

Table 3 shows BD-rate performance of dual layer vs single layer, for well-chosen pairs of QPs.

| Name | QPBL | QPEL = QPSL | BD-rate vs single layer (PSNR_Y) | BD-rate vs single layer (SSIM_Y) |
| --- | --- | --- | --- | --- |
| Procession | X, X, X, X | X, X, X, X | X% | X% |
| RiverBoat |  |  |  |  |
| Marathon2 |  |  |  |  |
| MountainBay2 |  |  |  |  |
| TallBuildings2 |  |  |  |  |
| CorsaireJoy |  |  |  |  |
| TwoBirdsInDawn |  |  |  |  |
| AngelInSunrise |  |  |  |  |

Table 3: BD-rate vs single layer