| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0050-v1 |
| --- | --- |

| Title: | AHG2/AHG9: On the alpha channel information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the semantics the alpha channel information SEI message, the alpha_channel_incr_flag is inferred to be equal to 0 when not present. It is asserted that this inference seems unnecessary or even not correct. The flag is not present only when alpha_channel_cancel_flag is equal to 1, in which case no information applies. It is further asserted that, if the absence of the SEI message is also considered, then the value of the flag should depend on the persistency scope of the previous SEI message of this type, and then this inference can be incorrect.

It is therefore proposed to remove the inference rule. Note that this asserted issue applies to both VSEI and HEVC.

# Introduction

## syntax and semantics of the alpha channel information SEI message

The syntax and semantics of the alpha channel information SEI message in JVET-X2006 are as follows:

| alpha_channel_info( payloadSize ) { | Descriptor |
| --- | --- |
| alpha_channel_cancel_flag | u(1) |
| if( !alpha_channel_cancel_flag ) { |  |
| alpha_channel_use_idc | u(3) |
| alpha_channel_bit_depth_minus8 | u(3) |
| alpha_transparent_value | u(v) |
| alpha_opaque_value | u(v) |
| alpha_channel_incr_flag | u(1) |
| alpha_channel_clip_flag | u(1) |
| if( alpha_channel_clip_flag ) |  |
| alpha_channel_clip_type_flag | u(1) |
| } |  |
| } |  |

The alpha channel information (ACI) SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA and one or more associated primary pictures.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with an ACI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and an ACI SEI message, the SDI SEI message shall precede the ACI SEI message in decoding order.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the ACI SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous ACI SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that ACI follows.

Let currPic be the picture that the ACI SEI message is associated with. The semantics of ACI SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an ACI SEI message is output that follows the current picture in output order.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2. Decoders shall ignore alpha channel information SEI messages in which alpha_channel_use_idc is greater than 2.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. alpha_channel_bit_depth_minus8 plus 8 shall be equal to the bit depth of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary picture luma sample value is equal to the decoded auxiliary picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary picture sample and any auxiliary picture luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used, without alteration, as the interpretation sample value for the decoded auxiliary picture sample value. When not present, the value of alpha_channel_incr_flag is inferred to be equal to 0. [Ed. (YK): This inference seems unnecessary or even not correct. It is not present only when alpha_channel_cancel_flag is equal to 1, in which case no information applies. If the absence of the SEI message is also considered, then its value should depend on the persistency scope of the previous SEI message of this type, and this inference can then be incorrect. The right course of action seems to be to just remove the inference rule.]

When alpha_channel_incr_flag is equal to 1, alpha_transparent_value shall not be equal to alpha_opaque_value and Log2( Abs( alpha_opaque_value − alpha_transparent_value ) ) shall have an integer value. A value of alpha_transparent_value that is equal to alpha_opaque_value indicates that the auxiliary coded picture is not intended for alpha blending purposes.

NOTE 1 – For alpha blending purposes, alpha_opaque_value may be greater than alpha_transparent_value, or it may be less than alpha_transparent_value. Interpretation sample values should be clipped to the range of alpha_opaque_value to alpha_transparent_value, inclusive.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the alpha_channel_clip_type_flag syntax element. When not present, the value of alpha_channel_clip_flag is inferred to be equal to 0.

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less or equal than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than alpha_opaque_value is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less than or equal to alpha_transparent_value is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE 2 – When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first followed by the alteration specified by alpha_channel_incr_flag to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE 3 – Alpha blending composition is normally performed with a background picture B, a foreground picture F, and a decoded auxiliary coded picture A, all of the same size. Assume for purposes of example illustration that the chroma resolution of B and F have been upsampled to the same resolution as the luma. Denote corresponding samples of B, F and A by b, f and a, respectively. Denote luma and chroma samples by subscripts Y, Cb and Cr.

Define the variables alphaRange, alphaFwt and alphaBwt as follows:

alphaRange = Abs( alpha_opaque_value − alpha_transparent_value )	(53)

alphaFwt = Abs( a − alpha_transparent_value )		(54)

alphaBwt = Abs( a − alpha_opaque_value )		(55)

Then, in alpha blending composition, samples d of the displayed picture D could be calculated as follows:

dY = ( alphaFwt * fY + alphaBwt * bY + alphaRange / 2 ) / alphaRange	(56)

dCb = ( alphaFwt * fCb + alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(57)

dCr = ( alphaFwt * fCr + alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(58)

The samples of pictures D, F and B could also represent red, green, and blue component values (see clause 7.3). Here the Y, Cb and Cr component values are assumed. Each component, e.g., Y, is assumed for purposes of example illustration above to have the same bit depth in each of the pictures D, F and B. However, different components, e.g., Y and Cb, need not have the same bit depth in this example.

If alpha_channel_use_idc is equal to 0, the following applies:

- F would be the decoded picture obtained from the decoded luma and chroma, and A would be the decoded picture obtained from the decoded auxiliary coded picture. In this case, the indicated example alpha blending composition involves multiplying the samples of F by factors obtained from the samples of A.

- A picture format that is useful for editing or direct viewing, and that is commonly used, is called pre-multiplied-black video. When the foreground picture was F, the pre-multiplied-black video S is given by

sY  = ( alphaFwt * fY ) / alphaRange		(59)

sCb = ( alphaFwt * fCb ) / alphaRange		(60)

sCr = ( alphaFwt * fCr ) / alphaRange		(61)

- Pre-multiplied-black video has the characteristic that the picture S will appear correct if displayed against a black background. For a non-black background B, the composition of the displayed picture D may be calculated as

dY  = sY + ( alphaBwt * bY + alphaRange / 2 ) / alphaRange	(62)

dCb = sCb + ( alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(63)

dCr = sCr + ( alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(64)

Otherwise, when alpha_channel_use_idc is equal to 1, alpha blending composition does not involve multiplication of the samples of S by factors obtained from the samples of A, S would be the decoded picture obtained from the decoded luma and chroma, and A would again be the decoded picture obtained from the decoded auxiliary coded picture.

## Problem description and proposal summary

In the semantics the alpha channel information SEI message, the alpha_channel_incr_flag is inferred to be equal to 0 when not present. As described in the editing note, this inference seems unnecessary or even not correct. The flag is not present only when alpha_channel_cancel_flag is equal to 1, in which case no information applies. If the absence of the SEI message is also considered, then the value of the flag should depend on the persistency scope of the previous SEI message of this type, and then this inference can be incorrect.

It is therefore proposed to just remove the inference rule. Note that this asserted issue applies to both VSEI and HEVC.

# Proposed changes

The proposed change is as follows (deleted parts are highlighted in yellow strikethrough in red fonts):

...

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary picture luma sample value is equal to the decoded auxiliary picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary picture sample and any auxiliary picture luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used, without alteration, as the interpretation sample value for the decoded auxiliary picture sample value. When not present, the value of alpha_channel_incr_flag is inferred to be equal to 0. [Ed. (YK): This inference seems unnecessary or even not correct. It is not present only when alpha_channel_cancel_flag is equal to 1, in which case no information applies. If the absence of the SEI message is also considered, then its value should depend on the persistency scope of the previous SEI message of this type, and this inference can then be incorrect. The right course of action seems to be to just remove the inference rule.]

...

# Patent rights declaration(s)

Bytedance Inc. does not have any current or pending patent rights relating to the technology proposed in this contribution (to the extent of the personal awareness of the authors).