| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0053-v1 |
| --- | --- |

| Title: | AHG9/AHG13: film grain blending process for film grain characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a film grain blending process for film grain characteristics SEI message. The blending process is identical to the process specified in SMPTE RDD5 except the modifications to reduce line buffers and on-chip memory storage.

The contribution proposes two independent modifications:

- The grain block size is changed from fixed 8x8-size in RDD5 to 8x1 for picture resolutions less than or equal to 1920x1080, 16x1 for resolution greater than 1920x1080 and up to 3840x2160, and 32x1 for resolutions greater than 4K including 8K. Using Nx1 block size eliminates the need of line buffers to compute block average in the existing methods.

- Add note to limit the number of film grain patterns per picture to 1 to reduce on-chip memory requirement.

# Problem Statement

SMPTE RDD5 [1] specifies a fixed-point blending process and is already referenced in VSEI. The process computes the 8x8 sample block average value to select the film grain parameters, which needs 8-line buffers for the hardware implementation. While AV1 [3] film synthesis process implements the grain synthesis with just 2-line buffers for luma and 1 line buffer for each chroma component.

JVET-W0095 [2] proposed 16x16 and 32x32 grain block size for higher resolution which further increases the number of line buffers up to 32. Here is the note from the 24th JVET meeting on FGS CE:

"It is pointed out that bigger block size could require more line buffers, which might not be implementation friendly in hardware. Run time may not be the only aspect of complexity."

This contribution proposes the method to eliminate the need of multiple line buffers to compute block average in the existing methods

SMPTE RDD5 constrains that "combining all the color components c and intensity intervals i in an SEI message, the number of different pairs (comp_model_value[ c ][ i ][ 1 ], comp_model_value[ c ][ i ][ 2 ]) shall be equal or smaller than 10", which implies up to 10 film grain patterns may be applied to a picture.

Given 64x64 film grain pattern size, the on-chip memory storage for 10 patterns is around 40KB. In today's "artificial" film grain practice, it is mostly common to have one pattern over a picture or even over a complete sequence. This contribution proposes to constrain one film grain pattern per picture to reduce on-chip memory to 4KB.

# Proposal

The proposed film grain synthesis process is identical to the SMPTE RDD5 process except for two modifications:

- The grain block size is changed from fixed 8x8-size in RDD5 to 8x1 for picture resolutions less than or equal to 1920x1080, 16x1 for resolution greater than 1920x1080 and up to 3840x2160, and 32x1 for resolutions greater than 4K including 8K.

- Add a note to constrain the number of film grain pattern used per picture "combining all the color components c and intensity intervals i in film grain characteristics SEI message, the number of different pairs (comp_model_value[ c ][ i ][ 1 ], comp_model_value[ c ][ i ][ 2 ]) shall be equal to 1".

The modification of grain block size was implemented on the film grain synthesis CE test software. The subjective test on CE subjective test bitstreams shows no difference between RDD5 and the proposed modification. Additional subjective test was also conducted on the CTC class B 1080P test sequences with film grain configuration setting, FGC_SEI_I10C3, and the result is the same.

# Acknowledgment

We would like to thank InterDigital for cross-checking the software.

# References

- “RDD 5:2006 - SMPTE Registered Disclosure Doc - Film Grain Technology — Specifications for H.264 | MPEG-4 AVC Bitstreams,” RDD 5:2006, pp. 1–18, Mar. 2006, doi: 10.5594/SMPTE.RDD5.2006.

- Sean McCarthy, Peng Yin, Walt Husak, Fangjun Pu, Taoran Lu, Tao Chen, Edouard François, Miloš Radosavljević, Vijayakumar G R, Kaustubh Patankar, Shireesh Kadaramandalgi, Ajayshyam, " AHG9: Fixed-point grain blending process for film grain characteristics SEI message", JVET-W0095, Teleconference, 7-16 July, 2021.

- Peter de Rivaz, Jack Haughton, "AV1 bitstream & decoding process specification", 2019

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).