| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0056 |
| --- | --- |

| Title: | AHG2: High tier for lower levels |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Adrian Browne Karl Sharman | Email: | steve.keating@sony.com adrian.browne@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This contribution proposes to remove the defined MinCrBase values for high tier at levels below 4.0 to make clear that high tier is not supported at these levels.

# Introduction

For high tier at levels below 4.0 MaxBR is not defined but MinCrBase is defined.  It is asserted that this creates an ambiguity as to whether high tier is supported for these lower levels. It is proposed to remove the defined MinCrBase values for high tier at levels below 4.0 to make clear that high tier is not supported for levels below 4.0.

# Proposed Changes

In clause A.4.2, Table 136 is changed to:

Table 136 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1.0 | 552 960 | 128 | - | 2 | - |
| 2.0 | 3 686 400 | 1 500 | - | 2 | - |
| 2.1 | 7 372 800 | 3 000 | - | 2 | - |
| 3.0 | 16 588 800 | 6 000 | - | 2 | - |
| 3.1 | 33 177 600 | 10 000 | - | 2 | - |
| 4.0 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5.0 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.0 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |

# Conclusion

It is proposed to remove the defined MinCrBase values for high tier at levels below 4.0 to make clear that high tier is not supported for levels below 4.0.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies do not have any current or pending patent rights relating to the technology described in this contribution.

# References

- “ Versatile Video Coding Editorial Refinements on Draft 10)” JVET-T2001-v1, B Bross, J Chen, S Liu, Y-K Wang