| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0057 |
| --- | --- |

| Title: | AHG2: MinCr for still picture profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Adrian Browne Karl Sharman | Email: | steve.keating@sony.com adrian.browne@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This contribution proposes defining a minimum compression ratio for still picture profiles.  It is asserted that this would be useful for encoder and decoder design to set maximum buffer sizes etc.

# Introduction

This proposal is to define a minimum compression ratio for Still Picture profiles.  Currently this is only defined for motion picture sequences.  Defining such a limit would be useful for encoder and decoder design to set maximum buffer sizes etc.

It is also proposed to define this according to the maximum luma picture size, MaxLumaPs, of the level (defined in table 135) rather than according to the input picture size, PicSizeMaxInSamplesY (defined in clause A.4.1).  The advantage of doing this is that choosing a higher level to encode a given size of picture would allow a lower compression ratio to be used.

It is also now necessary to define HbrFactor and MinCrBase for Still Picture profiles; it is also necessary to keep the definitions for FormatCapabilityFactor and MinCrScaleFactor (defined in Table 5) rather than remove them as proposed in JVET-X2005.

# Proposed Changes

Changes to JVET-X2005 are shown highlighted in yellow.

In clause A.4.2, replace the following:

The variable HbrFactor is defined as follows:

- If the bitstream is indicated to conform to the Main 10, Main 10 4:4:4, Multilayer Main 10, or Multilayer Main 10 4:4:4 profile, HbrFactor is set equal to 1.

with the following:

The variable HbrFactor is defined as follows:

- If the bitstream is indicated to conform to the Main 10, Main 10 4:4:4, Multilayer Main 10, or Multilayer Main 10 4:4:4, Main 10 Still Picture, Main 10 4:4:4 Still Picture, Main 12, Main 12 4:4:4, Main 16, or Main 16 4:4:4 profile, HbrFactor is set equal to 1.

- Otherwise, if the bitstream is indicated to conform to Main 12, Main 12 Intra, Main 12 Still Picture, Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture, Main 16 4:4:4, Main 16 4:4:4 Intra or Main 16 4:4:4 Still Picture, HbrFactor is set equal to 1 for Main tier or equal to 2 for High tier.

In clause A.4.2, add the following (additions are yellow-highlighted):

j)	The difference between consecutive CPB removal times of AUs n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in AU n is less than or equal to Min( Max( 1, MaxTilesPerAu * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTilesPerAu ), where MaxTilesPerAu is the value specified in Table 135 that apply to AU n.

Bitstreams conforming to the Main 12 Still Picture, Main 12 4:4:4 Still Picture or Main 16 4:4:4 Still Picture profile at a specified tier and level shall obey the following constraint for each bitstream conformance test as specified in Annex C:

- The sum of the NumBytesInNalUnit variables for AU 0 shall be less than or equal to FormatCapabilityFactor * MaxLumaPs ÷ MinCr, where FormatCapabilityFactor and MaxLumaPs are the values specified in Table 135 and Table 137, respectively, that apply to AU 0.

In Table 137, keep the entries for FormatCapabiltyFactor and MinCrScaleFactor for Still Picture profiles, with 12-bit and 16-bit Still Picture profiles using the same values as Intra profiles:

| Profiles | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10, Main 10 Still Picture, Multilayer Main 10 | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 10 4:4:4, Main 10 4:4:4 Still Picture, Multilayer Main 10 4:4:4 | 2 500 | 2 750 | 3.750 | 0.75 |
| Main 12 | 1 200 | 1 320 | 1.875 | 1.00 |
| Main 12 Intra, Main 12 Still Picture | 2 400 | 2 640 | 1.875 | 1.00 |
| Main 12 4:4:4 | 3 000 | 3 300 | 3.750 | 0.75 |
| Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture | 6 000 | 6 600 | 3.750 | 0.75 |
| Main 16 4:4:4 | 4 000 | 4 400 | 6.000 | 0.75 |
| Main 16 4:4:4 Intra, Main 16 4:4:4 Still Picture | 8 000 | 8 800 | 6.000 | 0.75 |

# Additional aspect

Include changes to Main 10 Still Picture, Main 10 4:4:4 Still Picture as an ammendment to the version 1 specification.

# Conclusion

It is proposed to add a minimum compression ratio, MinCr, for still picture profiles.  It is also proposed that this should be relative to the maximum luma picture size of the level rather than according to the input picture size.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “VVC operation range extensions (Draft 5)” JVET-X2005-v1, F Bossen, B Bross, T Ikai, D Rusanovskyy, G Sullivan, Y-K Wang

- “ ITU-T Rec. H.265v8 High Efficiency Video Coding (08/2021)”