| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting, by teleconference, 12-21 Jan 2022 | Document: JVET-Y060-v1 |
| --- | --- |

| Title: | AhG-7 Refined Low Latency Controlled Complexity (LLCC) configurations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | G. Martin-Cocher, K. Nasser, T. Poirier, S. Puri | Tel: Email: | first.last @ interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution complements the Low Latency Controlled Complexity (LLCC) baseline configurations defined in  JVET-Y0043 by enabling and disabling further tools according to their latency and complexity impacts.

The following additional constraints are proposed for both LLCC-1 (LD-P) and LLCC-2 (LD-B): Tools off: ALF, DQ, RDOQ and MTS; Tools on: Sign-hiding, IBC, MTSImplicit.

For LLCC-1 the following additional constraints are proposed: MTT=0, min QT size = 4. For Class B (1080p) the Encoding Time to Real Time (ET/RT) ratio is reduced from 8538/1 to 1524/1 for VTM and from xxx to 10248/1 for ECM.  The encoding runtime is reduced to yyy% for VTM and to 45% for ECM.

For LLCC-2 the following additional constraints are proposed:  MTT=1, min QT size = 4. For Class B (1080p) the Encoding Time to Real Time (ET/RT) ratio is reduced from 12090/1 to 2850/1 for VTM and from 36108/1 to yyy for ECM. The encoding runtime is reduced from to yyy% for VTM and to yyy% for ECM.

A software patch for ECM AffineAMVP is available, enabling to test it independently from Affine. It is further proposed to turn OFF AffineAMVP in the LLCC configurations to further reduce the encoding runtime.

- ECM LD-P (class E,F, DMV) Affine Off:            9,53% Y, 8,33% U,  8,19%V,  87%EncT

- ECM LD-P (class E,F, DMV) AffineAMVP off: 2,88% Y, 2,46% U,  2,70%V,  88% EncT

# Discussion

In JVET-X0138  four Low Latency Controlled Complexity (LLCC) configurations were first proposed with an analysis of market practices and tools usage. The “low latency centric” and “Low latency and encoder complexity centric” options are reproduced below:

| Config | TEST | Class | BDR Y | BDR U | BDR V | Enc | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LLCC option 1 (latency centric) | TEST 1 - Sign-hiding ON - IBC ON - DQ, RDOQ, ALF OFF | Class F | 9.58% | 17.21% | 16.37% | 74.34% | 71.72% |
|  |  | Class TGM | 7.55% | 13.30% | 13.81% | 76.78% | 66.69% |
| LLCC option 3 (encoder complexity) | TEST 3 - TEST 1 - MTT=0, min QT size = 4 - MTSImplicit ON - MTS OFF | Class F | 33.67% | 51.37% | 51.02% | 30.28% | 76.77% |
|  |  | Class TGM | 31.73% | 43.77% | 44.30% | 41.43% | 80.47% |

Table 1: JVET X0138 :  LLCC options 1&3 versus ECM 2.0-LDP+SCC

In JTCVC-B054 [1] similar observations and proposals were made: “low complexity video encoder settings are proposed …. tools should be tested for both high complexity and low complexity encoder environments so that the … design can satisfy the requirements of a wide range of video applications.”  ALF and RDOQ were proposed to be turned off.

# Proposed LLCC CTCs

VTM LLCC-1:

- Based on LD-P with flat QP

- Tools OFF: DQ, RDOQ, ALF, MTS, LMCS chroma scaling

- ON: Sign-hiding, IBC, MTSImplicit,

- GDR  withGdrInterval set to 1second and  a GdrPeriod = GdrInterval+ 4P frames

- MTT=0, min QT size = 4

ECM LLCC-1:

- Based on LD-P with flat QP

- Tools OFF: DQ, RDOQ, ALF, MTS

- ON: Sign-hiding, IBC, MTSImplicit,

- One IDR every second

- MTT=0, min QT size = 4

VTM LLCC-2:

- Based on LD-B with maximum 2 reference frames

- Tools OFF: DQ, RDOQ, ALF, MTS, LMCS chroma scaling

- ON: Sign-hiding, IBC, MTSImplicit

- MTT=1, min QT size = 4

ECM LLCC-2:

- Based on LD-B with maximum 2 reference frames

- Tools OFF: DQ, RDOQ, ALF, MTS, LMCS chroma scaling

- ON: Sign-hiding, IBC, MTSImplicit

- MTT=1, min QT size = 4

QP42 is added and BD-rate calculations are proposed to be performed on QP22-37 and on QP 27-42.

# results:

Anchors: VTM 15 and ECM 3.1 with baseline configurations defined in JVET-Y0043

ECM partial results:

|  | LLCC-2 (LDB) (QP22,27,32,37) |  |  |  |  |  |  | LLCC-2 (LDB) (QP27,32,37,42) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM3.1 -baseline LLCC-2 |  |  |  |  |  |  | BD-rate Over ECM3.1 -baseline LLCC-2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |  |  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  | Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | 10,87% | 28,11% | 14,84% | 58% | 71% |  | Class E | 11,28% | 19,55% | 10,89% | 61% | 78% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 5,28% | 12,72% | 12,30% | 43% | 72% |  | Class D | 5,39% | 10,46% | 8,30% | 46% | 72% |
| Class F | 6,89% | 15,82% | 14,69% | 50% | 68% |  | Class F | 8,39% | 13,48% | 13,39% | 53% | 70% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | 70% |
|  | LLCC-1 (LDP) (QP22,27,32,37) |  |  |  |  |  |  | LLCC-1 (LDP) (QP27,32,37,42) |  |  |  |  |
|  | BD-rate Over ECM3.1 - baseline LLCC-1 |  |  |  |  |  |  | BD-rate Over ECM3.1 -baseline LLCC-1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |  |  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 23,06% | 52,85% | 42,87% | 43% | 58% |  | Class B | 19,77% | 30,64% | 25,09% | 48% | 56% |
| Class C | 15,05% | 23,76% | 25,61% | 35% | 71% |  | Class C | 15,74% | 20,74% | 23,07% | 38% | 69% |
| Class E | 23,79% | 39,79% | 26,06% | 45% | 65% |  | Class E | 21,35% | 28,70% | 20,28% | 47% | 73% |
| Overall | 20,57% | 39,89% | 32,91% | 40% | 64% |  | Overall | 18,82% | 26,85% | 23,21% | 44% | 64% |
| Class D | 13,22% | 20,50% | 19,26% | 31% | 73% |  | Class D | 13,55% | 18,32% | 15,17% | 33% | 74% |
| Class F | 20,16% | 30,26% | 29,85% | 41% | 67% |  | Class F | 21,02% | 28,22% | 28,05% | 43% | 68% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | 68% |

VTM partial results:

|  | LLCC-2 (LDB) (QP22,27,32,37) |  |  |  |  |  |  | LLCC-2 (LDB) (QP27,32,37,42) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM15 baseline LLCC-2 |  |  |  |  |  |  | BD-rate Over VTM 15 baseline LLCC-2 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |  |  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 15,32% | 28,29% | 25,01% | 19% | 93% |  | Class C | 15,48% | 25,97% | 21,78% | 23% | 89% |
| Class E | 13,93% | 35,57% | 18,85% | 36% | 87% |  | Class E | 13,86% | 23,42% | 14,75% | 43% | 86% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 15,79% | 22,45% | 20,39% | 18% | 88% |  | Class D | 14,44% | 20,09% | 15,48% | 22% | 86% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | LLCC-1 (LDP) (QP22,27,32,37) |  |  |  |  |  |  | LLCC-1 (LDP) (QP27,32,37,42) |  |  |  |  |  |
|  | BD-rate Over VTM15 baseline LLCC-1 |  |  |  |  |  |  | BD-rate Over VTM15 baseline LLCC-1 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |  |  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 20,74% | 28,87% | 29,27% | 8% | 94% |  | Class C | 23,09% | 35,53% | 35,60% | 10% | 96% |
| Class E | 35,44% | 72,33% | 52,71% | 15% | 95% |  | Class E | 40,11% | 66,60% | 55,64% | 20% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 21,32% | 26,90% | 26,47% | 7% | 95% |  | Class D | 22,86% | 35,76% | 32,58% | 9% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  | ClassTGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

ET/RT ratio (partial results):

| VTM (5QP) | VTM 15 |  | VTM 15 JVET-Y0043 cfg |  | VTM 15 proposed cfg |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | ET/RT LD-P | ET/RT LD-B | ET/RT baseline LLCC1 | ET/RT baseline LLCC2 | ET/RT LLCC1 | ET/RT LLCC2 |
| B | 8538/1 | 12090/1 |  |  | 1524/1 | 2850/1 |
| C | 2010/1 | 2850/1 | 2946/1 | 2424/1 | 444/1 | 246/1 |
| D | 612/1 | 720/1 | 864/1 | 828/1 | 108/1 | 216/1 |
| E | 1532/1 | 2376/1 | 2628/1 | 1896/1 | 474/1 | 954/1 |
| F | 2232/1 | 2706/1 |  |  | 858/1 | 798/1 |
| TGM | 5628/1 | 6879/1 |  |  | 1902/1 | 1956/1 |
| ECM (5QP) | ECM 3.0 anchor |  | ECM JVET Y0043 cfg |  | ECM proposed cfg |  |
| Class | ET/RT LD-P | ET-RT LD-B | ET/RT baseline LLCC1 | ET/RT baseline LLCC2 | ET/RT LLCC1 | ET/RT LLCC2 |
| B |  | 36108/1 | 22500/1 |  | 10248/1 |  |
| C | 5568/1 | 8700/1 | 5100/1 | 7752/1 | 2136/1 |  |
| D | 1644/1 | 2436/1 | 1596/1 | 1908/1 | 660/1 | 792/1 |
| E | 4428/1 | 7212/1 | 4764/1 | 5688/1 | 2268/1 | 3360/1 |
| F |  | 6252/1 | 5220/1 | 5472/1 | 2448/1 | 3108/1 |
| TGM |  |  |  |  |  |  |

The ET/RT ratio is still quite fare from 1000/1 for 1080p resolution.

# tools consideration:

- ALF

ALF implementation providing more granularity (per CTU) than currently available would be beneficial from a latency perspective. ALF is switched OFF in all the options.

- RDOQ

RDOQ is turned off: Different tools than RDOQ are often used (e.g. see [2]).

- MTT

As some encoders do not perform a full MTT under latency/real-time and complexity constraints, MTT depth is modified, MTS is switched off, MTS implicit is enabled. Minimum QT size is set to 4. These options are somehow close to an HEVC configuration in terms of splitting complexity, while still providing 10% gain above HEVC.  Enabling MTT=0 and MTT=1 in two different configurations allow for a broader test coverage for individual tools and combination of tools, while corresponding to existing usage.

# AffineAMVP

We further modified the current affine implementation to enable/disable AffineAMVP independently from Affine as complexity of this tool can be a concern.

A software patch is made available that defines AffineAMVP option in the software. AffineAMVP can be disabled on encoder either through command line (--AffineAmvp=0) or through config file option (AffineAmvp : 0).

Results for VTM:

|  |  |  | LD-P Affine Off |  |  |  |  | LD-P AffineAMVP Off |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 14.0 |  |  |  |  |  | Over VTM 14.0 |  |  |  |  |  |
|  | Y | U | V | EncT |  | DecT | Y |  | U | V | EncT |  | DecT |
| Class E | 3,89% | 1,89% | 2,53% | 76% |  | 102% | 1,68% |  | 1,18% | 1,08% | 79% |  | 104% |
| Class F | 4,46% | 3,50% | 2,81% | 92% |  | 107% | 3,50% |  | 2,71% | 2,51% | 97% |  | 111% |
| Class DMV HD | 8,38% | 8,46% | 7,32% | 82% |  | 91% | 3,73% |  | 3,59% | 3,20% | 87% |  | 100% |
| Class DMV 4K | 23,31% | 21,09% | 20,82% | 78% |  | 80% | 10,94% |  | 9,58% | 9,37% | 84% |  | 93% |
| Overall (E & F) | 4,21% | 2,81% | 2,69% | 85% |  | 104% | 2,72% |  | 2,05% | 1,90% | 89% |  | 108% |
| All | 8,88% | 8,01% | 7,34% | 82% |  | 95% | 4,47% |  | 3,93% | 3,60% | 87% |  | 103% |

|  |  |  | LD-B Affine Off |  |  |  |  | LD-B AffineAMVP Off |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 14.0 |  |  |  |  |  | Over VTM 14.0 |  |  |  |  |  |
|  | Y | U | V | EncT |  | DecT | Y |  | U | V | EncT |  | DecT |
| Class E | 2,94% | 1,79% | 2,33% | 70% |  | 99% | 1,80% |  | 1,15% | 2,11% | 74% |  | 104% |
| Class F | 4,46% | 3,66% | 4,03% | 85% |  | 105% | 3,82% |  | 3,95% | 3,52% | 84% |  | 99% |
| Class DMV HD | 8,74% | 8,97% | 7,94% | 75% |  | 88% | 4,35% |  | 4,29% | 3,93% | 79% |  | 99% |
| Class DMV 4K | 23,89% | 21,58% | 21,49% | 77% |  | 94% | 11,79% |  | 10,35% | 10,40% | 74% |  | 94% |
| Overall (E & F) | 3,81% | 2,86% | 3,30% | 78% |  | 102% | 2,95% |  | 2,75% | 2,92% | 80% |  | 101% |
| All | 8,95% | 8,28% | 8,02% | 77% |  | 95% | 4,96% |  | 4,70% | 4,51% | 79% |  | 99% |

Results for ECM

|  |  |  |  | LD-P Affine Off |  |  |  |  |  |  |  |  |  | LD-P AffineAMVP Off |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM 3.1 |  |  |  |  |  |  |  |  |  |  | Over ECM 3.1 |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U | V |  |  | EncT |  |  | DecT |  | Y |  |  |  | U | V |  | EncT |  |  |  | DecT |  |
| Class E | 3,58% |  | 1,59% | 2,29% |  |  | 82% |  |  | 92% |  | 1,54% |  |  |  | 0,17% | 1,80% |  | 84% |  |  |  | 98% |  |
| Class F | 6,43% |  | 5,59% | 5,26% |  |  | 92% |  |  | 94% |  | 3,25% |  |  |  | 2,92% | 2,31% |  | 91% |  |  |  | 98% |  |
| Class DMV HD | 9,69% |  | 9,71% | 9,01% |  |  | 86% |  |  | 86% |  | 2,39% |  |  |  | 2,37% | 2,64% |  | 88% |  |  |  | 100% |  |
| Class DMV 4K | 16,75% |  | 14,12% | 14,38% |  |  | 82% |  |  | 83% |  | 3,85% |  |  |  | 3,62% | 3,51% |  | 85% |  |  |  | 95% |  |
| Overall (E & F) | 5,21% |  | 3,87% | 3,99% |  |  | 87% |  |  | 93% |  | 2,52% |  |  |  | 1,75% | 2,09% |  | 88% |  |  |  | 98% |  |
| All | 9,53% |  | 8,33% | 8,19% |  |  | 87% |  |  | 93% |  | 2,88% |  |  |  | 2,46% | 2,70% |  | 88% |  |  |  | 97% |  |
|  |  |  |  |  | LD-B Affine Off |  |  |  |  |  |  |  | LD-B AffineAMVP Off |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Over ECM 3.1 |  |  |  |  |  |  |  |  |  | Over ECM 3.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | U |  | V | EncT |  |  |  |  | DecT | Y |  |  | U |  |  | V |  | EncT |  |  |  | DecT |
| Class E |  | 2,78% | 2,33% |  | 1,66% | 83% |  |  |  |  | 96% | 1.42% |  |  | 1.49% |  |  | 0.87% |  | 85% |  |  |  | 100% |
| Class F |  | 6,66% | 4,38% |  | 4,71% | 92% |  |  |  |  | 97% | 3.64% |  |  | 2.35% |  |  | 2.36% |  | 92% |  |  |  | 100% |
| Class DMV HD |  | 9,89% | 9,96% |  | 9,31% | 86% |  |  |  |  | 79% | 2.50% |  |  | 2.71% |  |  | 2.66% |  | 88% |  |  |  | 99% |
| Class DMV 4K |  | - | - |  | - | - |  |  |  |  | - | - |  |  | - |  |  | - |  | - |  |  |  | - |
| Overall (E & F) |  | 4,99% | 3,50% |  | 3,40% | 88% |  |  |  |  | 97% | 2.69% |  |  | 1.98% |  |  | 1.72% |  | 89% |  |  |  | 100% |
| All |  | - | - |  | - | - |  |  |  |  | - | - |  |  | - |  |  | - |  | - |  |  |  | - |

# Proposal

Adopt the LLCC configurations defined in section 2.

Approve the AffineAMVP software patch and further turn off AffineAMVP in the LLCC configurations.

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

| [1] | M. Zhou, "JTCVC-B054," [Online]. Available: http://phenix.int-evry.fr/jct/doc_end_user/documents/2_Geneva/wg11/JCTVC-B054.zip. |
| --- | --- |
| [2] | "Nvidia Enc API programming guide," [Online]. Available: https://docs.nvidia.com/video-technologies/video-codec-sdk/nvenc-video-encoder-api-prog-guide/index.html#recommended-nvenc-settings__table-nvenc-settings. |