| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0063-v1 |
| --- | --- |

| Title: | AHG2: On Main 10 4:4:4 Still Picture profile for VVC v1 and v2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VVC v1 there is a requirement for decoders conforming to the Main 10 4:4:4 Still Picture profile that they shall also be capable of decoding the first picture of a Main 10 4:4:4 bitstream. However, there is no requirement that such decoder shall also be capable of decoding the first picture of a Main 10 bitstream.

The proponents assess that this is an oversight and proposes to add to the errata of VVC v1, and consequently also to VVC v2, that a Main 10 4:4:4 Still Picture decoder shall also be capable of decoding the first picture of a Main 10 bitstream.

The following text for the Main 10 4:4:4 Still picture decoder is proposed with added text on top of the current VVC v1 errata text in X1004-v1 marked in italics:

Decoders conforming to the Main 10 4:4:4 Still Picture profile at a specific level of a specific tier shall also be capable of decoding of the first picture of a bitstream when both of the following conditions apply:

  - That bitstream is indicated to conform to the Main 10 or Main 10 4:4:4 profile, to conform to a tier that is lower than or equal to the specified tier, to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

  - That picture is an IRAP picture or is a GDR picture with ph_recovery_poc_cnt equal to 0, is in an output layer, and has ph_pic_output_flag equal to 1.

The proponents note that decoders conforming to the Main 10 4:4:4 Still Picture profile shall be capable of decoding Main 10 Still Picture bitstreams which means that they are capable of decoding 10-bit 4:2:0 pictures already.

The proponents note that the text for the Main 12 Still Picture, Main 12 4:4:4 Still Picture, and Main 16 4:4:4 Still Picture profiles in the VVC v2 draft JVET-X2005 states that those profiles shall be capable of decoding the first picture of lower video profiles. The proponents see the current Main 10 4:4:4 Still Picture text as an exception in this regard and this proposal as a correction.

# Introduction

In section A.3.2 of the VVC v1 specification there is the following decoder requirement for the Main 10 4:4:4 Still Picture profile:

Decoders conforming to the Main 10 4:4:4 Still Picture profile at a specific level of a specific tier shall also be capable of decoding of the first picture of a bitstream when both of the following conditions apply:

- That bitstream is indicated to conform to the Main 10 4:4:4 profile, to conform to a tier that is lower than or equal to the specified tier, to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

- That picture is an IRAP picture or is a GDR picture with ph_recovery_poc_cnt equal to 0, is in an output layer, and has ph_pic_output_flag equal to 1.

Thus, a Main 10 4:4:4 Still Picture decoder shall be capable to decode the first picture of a Main 10 4:4:4 bitstream, but it is not required to be able to decode the first picture of a Main 10 (4:2:0) bitstream.

It seems like an oversight that not also Main 10 bitstreams would have to be supported, for the following two reasons:

- VVC v1 states that decoders conforming to the Main 10 4:4:4 Still Picture profile shall also be capable of decoding Main 10 Still Picture bitstreams:

Decoders conforming to the Main 10 4:4:4 Still Picture profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 10 4:4:4 Still Picture or Main 10 Still Picture profile.

- …

This means that Main 10 4:4:4 Still Picture profile decoders are capable of decoding 10-bit 4:2:0 pictures already.

- The corresponding text for the Main 12 Still Picture, Main 12 4:4:4 Still Picture, and Main 16 4:4:4 Still Picture profiles in the VVC v2 draft JVET-X2005 lists the lower video profiles that are supported, e.g. there is the following requirement for Main 12 4:4:4 Still Picture decoders:

Decoders conforming to the Main 12 4:4:4 Still Picture profile at a specific level of a specific tier shall also be capable of decoding of the first picture of a bitstream when both of the following conditions apply:

- That bitstream is indicated to conform to the Main 10, Main 10 4:4:4, Main 12, Main 12 Intra, Main 12 4:4:4, or Main 12 4:4:4 Intra profile, to conform to a tier that is lower than or equal to the specified tier, and to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

- That picture is a GDR picture with ph_recovery_poc_cnt equal to 0 or an IRAP picture, is in an output layer, and has ph_pic_output_flag equal to 1.

Consequently, it seems logical that a Main 10 4:4:4 Still picture decoder should also be required to decode the first picture of a Main 10 bitstream.

# Proposal

It is proposed to add to the errata of VVC v1, and consequently also to VVC v2, that a Main 10 4:4:4 Still Picture decoder shall also be capable of decoding the first picture of a Main 10 bitstream.

The following text for the Main 10 4:4:4 Still picture decoder is proposed with added text on top of the current VVC v1 errata text in X1004-v1 marked in italics:

Decoders conforming to the Main 10 4:4:4 Still Picture profile at a specific level of a specific tier shall also be capable of decoding of the first picture of a bitstream when both of the following conditions apply:

  - That bitstream is indicated to conform to the Main 10 or Main 10 4:4:4 profile, to conform to a tier that is lower than or equal to the specified tier, to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

  - That picture is an IRAP picture or is a GDR picture with ph_recovery_poc_cnt equal to 0, is in an output layer, and has ph_pic_output_flag equal to 1.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).