| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0068-v1 |
| --- | --- |

| Title: | EE1-2.1-related: RPR encoder with multiple scale factors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes RPR encoder control using multiple scale factors, such as x2.0 (half size) and x1.5 (2/3 size). At each GOP, the encoder can select a scale factor based on initial QP and a PSNR value which is calculated between the source and down-up sampling pictures for the first picture of each GOP. The experimental results are reportedly shown as below,

- For the range of QP 22 to 42 case,

  - AI: -0.57% / 3.14% / 2.73% / 98% / 83% for Y/U/V/Enc. Time/Dec. Time

  - RA: -0.98% / 2.62% / 2.50% / 92% / 80% for Y/U/V/Enc. Time/Dec. Time

- For the range of QP 27 to 47 case,

  - AI: -1.67% / 6.18% / 4.87% / 102% / 73% for Y/U/V/Enc. Time/Dec. Time

  - RA: -2.42% / 5.82% / 5.03% / 89% / 71% for Y/U/V/Enc. Time/Dec. Time

Introduction

EE1-2.1 shows the VVC RPR with x2.0 scale factor as anchor of NN-based super resolution experiments. It is reported that coding performance of RPR could be improved with adaptive enabling based on QP and resolution. As shown in JVET-W0082 [1], RPR encoder control at GOP level provided better coding performance. Generally, if an RPR encoder could find the optimal resolution based on predefined factors such as QP and frame complexity, coding gain could also be achieved. This contribution proposes RPR encoder control using multiple scale factors.

Proposed method

In this proposal, the RPR encoder for multiple resolutions uses two kinds of scale factors; one is x2.0 (half size), the other is x1.5 (2/3 size). A scale factor is selected at GOP level, the scale factor is applied all pictures in the GOP. To determine the scale factor, a PSNR value and initial QP for the first picture of each GOP are exploited. As similar with JVET-W0082, the first picture is downscaled to quarter resolution and then resampled to the source resolution. The PSNR is calculated between original picture and down-up scaled picture. One of scale factors, {x2.0 (half), x1.5 (2/3) and x1.0 (origin)}, is selected according to a threshold which is adaptively varied based on the initial QP. RPR scale factor (RPR_SF) of i-th GOP is derived as follows,

RPR_SFGOPi=2.0,DnUpPSNRi>(BasePSNR)-QPi-BaseQP*0.5                               1.5,DnUpPSNRi>BasePSNR-OffsetPSNR-QPi-BaseQP*0.51.0, Oherwise

where DnUpPSNRi represents aforementioned down-up scaled PSNR for the first picture of i-th GOP. BasePSNR and BaseQP stand for predefined criteria for PSNR and QP, respectively. OffsetPSNR is a gap of PSNR to determine one of half and 2/3 resolutions. QPi is the initial QP for the first picture of i-th GOP. The threshold is adjusted in multiples of 0.5 for the difference between the initial QP and the BaseQP.

Experimental Results

The proposed method is implemented on top of VTM-13.0 [2] and the tests are conducted by VTM CTC [3]. The experimental results are provided by using template in NNVC CTC [4]. In this test, QP ranges are used with 22 to 42 (5 points). To evaluate the performance, output pictures for both half (ScalingRatioHor=2.0 and ScalingRatioVer=2.0) and 2/3 (ScalingRatioHor=1.5 and ScalingRatioVer=1.5) resolutions are scaled to the source resolution (UpscaledOutput=2). BaseQP and OffsetPSNR are set to 37 and 3. BasePSNR for RA and AI are 45 and 42, respectively. When half or 2/3 resolutions are encoded, QP value is adjusted with offset which is defined to -6 for half and -4 for -2/3 resolutions. LD tests are skipped because most coding gains are came from Class A1 and Class A2.

Table 1. Performance of the proposed method in AI (QP 22 – QP42)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.55% | 2.38% | 2.60% | 108% | 65% |
| Class A2 | -2.81% | 12.66% | 9.69% | 94% | 74% |
| Class B | 0.52% | 2.13% | 2.27% | 96% | 87% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Class E | 0.09% | 0.25% | 0.27% | 96% | 94% |
| Overall | -0.57% | 3.14% | 2.73% | 98% | 83% |
| Class D | 0.02% | 0.06% | 0.05% | 97% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 95% | 98% |

Table 2. Performance of the proposed method in RA(QP 22 – QP42)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3.48% | 2.23% | 3.71% | 78% | 54% |
| Class A2 | -1.55% | 7.82% | 6.70% | 86% | 73% |
| Class B | 0.08% | 1.83% | 1.27% | 97% | 89% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.98% | 2.62% | 2.50% | 92% | 80% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |

Additionally, tests on higher QP ranges with 27 to 47 are conducted. The experimental results shows better coding performance than anchor QP ranges as follows,

Table 3. Performance of the proposed method in AI (QP 27 – QP47)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -4.87% | 5.83% | 4.06% | 114% | 51% |
| Class A2 | -5.66% | 25.61% | 18.95% | 101% | 60% |
| Class B | 0.00% | 3.39% | 3.66% | 102% | 78% |
| Class C | 0.00% | -0.05% | -0.03% | 98% | 93% |
| Class E | 0.50% | 0.05% | 0.18% | 98% | 80% |
| Overall | -1.67% | 6.18% | 4.87% | 102% | 73% |
| Class D | 0.02% | 0.07% | 0.05% | 97% | 95% |
| Class F | 0.01% | -0.01% | 0.01% | 96% | 95% |

Table 4. Performance of the proposed method in RA(QP 27 – QP47)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -7.22% | 3.40% | 3.85% | 76% | 43% |
| Class A2 | -4.17% | 19.43% | 16.12% | 79% | 57% |
| Class B | -0.43% | 3.75% | 3.11% | 95% | 82% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -2.42% | 5.82% | 5.03% | 89% | 71% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 99% |

Conclusion

In this contribution, RPR encoder control using multiple resolutions including half and 2/3 of the original resolution is proposed and tested. The proposed method enables a smart encoder to select the best resolution among various candidates for the better coding performance.

References

[1] K. Andersson, J. Ström, R. Yu, “AHG10: GOP-based RPR encoder control”, JVET-W0082, Meeting by Teleconference, July 2021.

[2] VTM-13.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-13.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

[4] S. Liu, A. Segall, E. Alshina, R-L. Liao,	“Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, Meeting by Teleconference, October 2021.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).