| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0074-v1 |
| --- | --- |

| Title: | AHG9: Data conversion description for NNR post-filter SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Takeshi Chujoh Keiichiro Takada Tomohiro Ikai | Tel: Email: | yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp keiichiro.takada@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a post-filter SEI message based on JVET-X0112 to include input and output tensor format information of neural network post-filter and conversion process description. It is argued that the neural network models (input and output shape) do not have association information, i.e., which tensor element corresponds to which component of samples to apply the post-filter. It is also argued that even if NNC carries topology information by either NNR compression, PyTorch, TensorFlow, NNEF, or ONNX specification, the input and output tensor format information is fully dependent on each specific specification and not trivial. It is asserted that thus the format of an input/output tensor shall be specified.

Introduction

In JVET-X0112[1], an NNR post-filter SEI message is proposed for the carriage of an MPEG NNR bitstream that specifies a neural-network-based post-filter. This contribution proposes additional information describing the format of the input and output tensor of a neural network model to correctly convert the sample data from a decoded picture and to an output picture.

Although an NNR post-filter SEI message contains the topology information of the neural network (NN) model in the format of NNR compression, PyTorch, TensorFlow, NNEF, or ONNX specification, it does not specify the process of converting each component of picture data to input and from output tensor of the NN model. Moreover, it is not a trivial process to know the input/output format from the described model because each specification needs a specific parser and the specification may be changed in versions. For example, NNEF has the “external” operation for input to define parameters of a graph but it seems no explicit operation for output (thus parsing the model is necessary to know the output format). In addition to this, association information, i.e., the correspondence between picture data and an input/output tensor, is not defined. The association and format of an input/output tensor shall be specified to make this correspondence clear. In this proposal, the association is defined as a conversion process.

Proposal

The contribution proposes to include syntax on the input and output of the tensor or the post-filter neural network model and specifies a conversion process. The additional relative to JVET-X0112 are highlighted as yellow marker.

NNR post-filter SEI message syntax

| nnr_post_filter( payloadSize ) { | Descriptor |
| --- | --- |
| nnrpf_id | u(21) |
| nnrpf_mode_idc | u(2) |
| if( nnrpf_mode_idc > 0 ) { |  |
| nnrpf_persistence_flag | u(1) |
| nnrpf_input_tensor_format_idc | u(4) |
| nnrpf_output_tensor_format_idc | u(4) |
| } |  |
| if( nnrpf_mode_idc = = 1 ) { |  |
| i = 0 |  |
| do { |  |
| nnrpf_uri[ i ] | b(8) |
| while( nnrpf_uri[ i++ ] != 0 ) |  |
| } |  |
| if( nnrpf_mode_idc = = 2 ¦ ¦ nnrpf_mode_idc = = 3 ) { |  |
| for( i = 0; i < payloadSize − 3 ; i++ ) |  |
| nnrpf_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

- NNR post-filter SEI message semantics

This SEI message either specifies a neural network that may be used as a post-processing filter or cancels the persistence of the previous post-processing filter with the same identifier value (i.e., the same value of nnrpf_id).

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	An indicator for components to be updated, denoted herein by PfpComponentIdc, as specified in subclause X.X (post-filter purpose SEI message proposed in JVET-Y0073)

–	A chroma format indicator, denoted herein by ChromaFormatIdc and chroma subsampling ratio relative to luma denoted as SubWidthC and SubHeightC as specified by subclause 7.3.

–	A chroma format indicator for input data, denoted herein by ChromaFormatIdc, as specified in subclause 7.3

–	A 2-dimension array of decoded sample values of the cIdx-th component of a decoded picture, denoted herein by ComponentSample[ cIdx ].

A 3-dimension array of input tensor of the neural network, denoted herein inputTensor.

A 3-dimension array of output tensor of the neural network, denoted herein outputTensor.

–	A chroma format indicator for output data, denoted herein by OutChromaFormatIdc and chroma subsampling ratio relative to luma, denoted herein by outSubWidthC and outSubHeightC, as specified in subclause X.X (post-filter purpose SEI message proposed in JVET-Y0073)

A picture width and picture height in units of luma samples, denoted herein by OutWidthInLumaSamples and OutHeightInLumaSamples, respectively.

nnrpf_id contains an identifying number that may be used to identify a post-processing filter.

Values of nnrpf_id from 0 to 220 − 1, inclusive, may be used as determined by the application. Values of nnrpf_id from 220 to 221 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC.

nnrpf_mode_idc equal to 0 indicates that no post-processing filter associated with the nnrpf_id value is specified in this SEI message. nnrpf_mode_idc equal to 1 specifies that the post-processing filter associated with the nnrpf_id value is a neural network identified by a given Uniform Resource Identifier (URI). nnrpf_mode_idc equal to 2 specifies that the post-processing filter associated with the nnrpf_id value is a neural network represented by the ISO/IEC 15938-17 bitstream contained in this SEI message. nnrpf_mode_idc equal to 3 specifies that the post-processing filter is the neural network identified by the previous NNR post-filter SEI message, in decoding order, with nnrpf_mode_idc equal to 1 or 2 and updated by the ISO/IEC 15938-17 bitstream contained in this SEI message.

nnrpf_persistence_flag specifies the persistence of this SEI message for the current layer.

nnrpf_persistence_flag equal to 0 specifies that the this SEI message applies to the current decoded picture only.

nnrpf_persistence_flag equal to 1 specifies that this SEI message applies to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	This SEI message has nnrpf_mode_idc equal to 1 or 2 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and nnrpf_mode_idc value equal to 0, 1, or 2 is output.

–	This SEI message has nnrpf_mode_idc equal to 3 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and any nnrpf_mode_idc value is output.

The neural network that may be used as a post-processing filter for a cropped decoded output picture is the filter that i) is identified by the NNR post-filter SEI message that has a particular nnrpf_id value, has nnrpf_mode_idc equal to 1 or 2, and pertains to the picture and ii) is updated by the NNR post-filter SEI message, if any, that has the same nnrpf_id value, has nnrpf_mode_idc equal to 3, and pertains to the picture.

nnrpf_input_tensor_format_idc specifies the conversion process from a decoded picture samples to an input tensor of the neural network post-filter, as specified in Table X1

Prior to applying the neural network, the decoded picture data are arranged into a three-dimensional tensor inputTensor[ channel ][y][x].

When PfPComponentIdc is equal to 0, nnrpf_input_tensor_format_idc shall be 0.

When PfPComponentIdc is equal to 1, nnrpf_input_tensor_format_idc shall be 1.

Table X1 – Conversion process from decoded picture samples to input tensor

| nnrpf_input_ tensor_format_ idc | Conversion process |
| --- | --- |
| 0 | inputTensor[0][y][x] = ComponentSample[ 0 ][x][y] with x = 0..PicWidthInLumaSamples − 1 and y = 0..PicHeightInLumaSamples − 1 |
| 1 | inputTensor[0][y][x] = ComponentSample[ 1 ][x][y] inputTensor[1][y][x] = ComponentSample[ 2 ][x][y] with x = 0..PicWidthInLumaSamples/SubWidthC − 1 and y = 0..PicHeightInLumaSamples/SubHeightC − 1 |
| 2 | SW = SubWidthC SH = SubHeightC inputTensor[ 0 ][y][x] = ComponentSample[ 0 ][x][y] inputTensor[ 1 ][y][x] = ComponentSample[ 1 ] [x / SW][y / SH] inputTensor[ 2 ][y][x] = ComponentSample[ 2 ] [x / SW][y / SH] with x = 0..PicWidthInLumaSamples − 1 and y = 0..PicHeightInLumaSamples − 1 |
| 3 | SW = SubWidthC SH = SubHeightC inputTensor[ 0 ][y/2][x/2] = ComponentSample[ 0 ][x/2*2][y/2*2] inputTensor[ 1 ][y/2][x/2] = ComponentSample[ 0 ][x/2*2+1][y/2*2] inputTensor[ 2 ][y/2][x/2] = ComponentSample[ 0 ][x/2*2][y/2*2+1] inputTensor[ 3 ][y/2][x/2] = ComponentSample[ 0 ][x/2*2+1][y/2*2+1] inputTensor[ 4 ][y/2][x/2] = ComponentSample[ 1 ][x / SW][y / SH] inputTensor[ 5 ][y/2][x/2] = ComponentSample[ 2 ][x / SW][y / SH] with x = 0..PicWidthInLumaSamples/2 − 1 and y = 0..PicHeightInLumaSamples/2 − 1 |
| 4..15 | Reserved |

nnrpf_output_tensor_format_idc specifies the conversion process from an output tensor of the neural network post-filter to a picture samples.

After applying the neural network, the output three-dimensional tensor outputTensor[ channel ][ y ][ x ] are arranged into an output picture samples outSamplesL, outSamplesCb and outSamplesCr.

For the samples of outSamplesL, outSamplesCb or outSamplesCr in which the colour component is required by OutChromaFormatIdc but not indicated to be updated by PfpComponentIdc, the corresponding samples of ComponentSamples are used.

Table X2 – Conversion process from output tensor to output picture samples

| nnrpf_output_ tensor_format_ idc | Conversion process |
| --- | --- |
| 0 | outSamplesL[x][y] = outputTensor[ 0 ][y][x] with x = 0..OutWidthInLumaSamples − 1 and y = 0..OutHeightInLumaSamples − 1 |
| 1 | outSamplesL[x][y] = ComponentSamples[ 0 ][y][x] outSamplesCb[x][y] = outputTensor[ 0 ][y][x] outSamplesCr[x][y] = outputTensor[ 1 ][y][x] with x = 0..OutWidthInLumaSamples%outSubWidthC − 1 and y = 0..OutHeightInLumaSamples/outSubHeightC − 1 |
| 2 | outSW = OutSubWidthC outSH = OutSubHeightC outSamplesL[x][y] = outputTensor[ 0 ][y][x] outSamplesCb[x][y] = outputTensor[ 1 ][y / outSH][x / outSW] outSamplesCr[x][y] = outputTensor[ 2 ][y / outSH][x / outSW] with x = 0..OutWidthInLumaSamples − 1 and y = 0..OutHeightInLumaSamples − 1 |
| 3 | outSW = OutSubWidthC outSH = OutSubHeightC outSamplesL[x/2*2][y/2*2] = outputTensor[ 0 ][y/2][x/2] outSamplesL[x/2*2+1][y/2*2] = outputTensor[ 1 ][y/2][x/2] outSamplesL[x/2*2][y/2*2+1] = outputTensor[ 2 ][y/2][x/2] outSamplesL[x/2*2+1][y/2*2+1] = outputTensor[ 3 ][y/2][x/2] outSamplesCb[x / outSW][y / outSH] = outputTensor[ 4 ][y / outSH][x / outSW] outSamplesCr[x / outSW][y / outSH] = outputTensor[ 5 ][y / outSH][x / outSW] with x = 0..OutWidthInLumaSamples − 1 and y = 0..OutHeightInLumaSamples − 1 |
| 4..15 | Reserved |

nnrpf_uri[ i ] contains the i-th byte of a NULL-terminated UTF-8 character string that contains a URI identifying the neural network used as the post-processing filter.

nnrpf_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnrpf_payload_byte[ i ] for all values of i in the range of 0 to payloadSize − 3, inclusive, shall be a complete bitstream that conforms to ISO/IEC 15938-17.

H.266 text

D.9 Use of ITU-T H.274 | ISO/IEC 23002-7 SEI messages

D.X Use of the NNR post-filter SEI message

For purposes of interpretation of the NNR post-filter SEI message, the following variables are specified:

–	PicWidthInLumaSamples and PicHeightInLumaSamples are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	ComponentSample[ cIdx ] is set to be the 2-dimension array of decoded sample values of the cIdx-th component of a decoded picture.

References

[1]	M. M. Hannuksela, E. B. Aksu, F. Cricri, H. R. Tavakoli and M. Santamaria, “AHG9: On post-filter SEI,” Joint Video Experts Team Document, JVET-V0112, teleconference, Oct. 2021.

[2]	T. Chujoh, E. Sasaki and T. Ikai, “AHG9/AHG11: Neural network based super resolution SEI,” Joint Video Experts Team Document, JVET-T0092, teleconference, Oct. 2020.

[3]	B. Choi, Z. Li, W. Wang, W. Jiang, X. Xu, S. Wenger and S. Liu, "AHG9/AHG11: SEI message for carriage of neural network information for post filtering", Joint Video Experts Team Document, JVET-U0091, teleconference, Jan. 2021.

[4]	“Text of ISO/IEC DIS 15938-17 Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29 Documents, MDS19764_WG04_N00016, Online, Oct. 2020.

[5]	T. Chujoh, Y. Yasugi, K. Takada and T. Ikai, “AHG9: Colour component description for post-filter purpose SEI message,” Joint Video Experts Team Document, JVET-Y0073, teleconference, Jan. 2022.

[6]	K. Takada, Y. Yasugi, T. Chujoh and T. Ikai, “AHG9: Complexity description for NNR post-filter SEI message,” Joint Video Experts Team Document, JVET-Y0075, teleconference, Jan. 2022.

[7]	T. Suzuki, E. Sasaki, T. Chujoh, T. Ikai and H. Watanabe, “Coded Video Transmission using Super-Resolution and MPEG-NNR Deep Learning Metadata”, the 35th Picture Coding Symposium of Japan/the 25th Image Media Processing Symposium (PCSJ/IMPS 2020), Nov. 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).