| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0075-v1 |
| --- | --- |

| Title: | AHG9: Complexity description for NNR post-filter SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichiro Takada Yukinobu Yasugi Takeshi Chujoh Tomohiro Ikai | Tel: Email: | keiichiro.takada@sharp.co.jp yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a NNR post-filter SEI message based on JVET-X0112. This proposal suggests including syntax element for information of complexity and precision to decide applicability on the decoder side without having to analyze network information specified by the URI or payload in the SEI.

Introduction

At the last meeting, JVET-X0112 proposed an NNR post-filter SEI message using MPEG NNR specification, which carries neural network topology and parameters for a neural-network-based post-filter. This contribution presents additional information about the complexity of a neural-network-based post-filter.

Proposed text

The additional relative to JVET-X0112 are highlighted as yellow marker.

NNR post-filter SEI message syntax

| nnr_post_filter( payloadSize ) { | Descriptor |
| --- | --- |
| nnrpf_id | u(21) |
| nnrpf_mode_idc | u(2) |
| if( nnrpf_mode_idc > 0 ) { |  |
| nnrpf_persistence_flag | u(1) |
| nnrpf_parameter_type_idc | u(4) |
| nnrpf_num_parameters_idc | u(8) |
| nnrpf_num_kmac_operations_idc | ue(v) |
| while( !byte_aligned( ) ) |  |
| nnrpf_alignment_zero_bit /* equal to 0 */ | f(1) |
| } |  |
| if( nnrpf_mode_idc = = 1 ) { |  |
| i = 0 |  |
| do { |  |
| nnrpf_uri[ i ] | b(8) |
| while( nnrpf_uri[ i++ ] != 0 ) |  |
| } |  |
| if( nnrpf_mode_idc = = 2 ¦ ¦ nnrpf_mode_idc = = 3 ) { |  |
| for( i = 0; i < payloadSize − 3 ; i++ ) |  |
| nnrpf_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

This SEI message either specifies a neural network that may be used as a post-processing filter or cancels the persistence of the previous post-processing filter with the same identifier value (i.e., the same value of nnrpf_id).

nnrpf_id contains an identifying number that may be used to identify a post-processing filter.

Values of nnrpf_id from 0 to 220 − 1, inclusive, may be used as determined by the application. Values of nnrpf_id from 220 to 221 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC.

nnrpf_mode_idc equal to 0 indicates that no post-processing filter associated with the nnrpf_id value is specified in this SEI message. nnrpf_mode_idc equal to 1 specifies that the post-processing filter associated with the nnrpf_id value is a neural network identified by a given Uniform Resource Identifier (URI). nnrpf_mode_idc equal to 2 specifies that the post-processing filter associated with the nnrpf_id value is a neural network represented by the ISO/IEC 15938-17 bitstream contained in this SEI message. nnrpf_mode_idc equal to 3 specifies that the post-processing filter is the neural network identified by the previous NNR post-filter SEI message, in decoding order, with nnrpf_mode_idc equal to 1 or 2 and updated by the ISO/IEC 15938-17 bitstream contained in this SEI message.

nnrpf_persistence_flag specifies the persistence of this SEI message for the current layer.

nnrpf_persistence_flag equal to 0 specifies that the this SEI message applies to the current decoded picture only.

nnrpf_persistence_flag equal to 1 specifies that this SEI message applies to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	This SEI message has nnrpf_mode_idc equal to 1 or 2 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and nnrpf_mode_idc value equal to 0, 1, or 2 is output.

–	This SEI message has nnrpf_mode_idc equal to 3 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and any nnrpf_mode_idc value is output.

The neural network that may be used as a post-processing filter for a cropped decoded output picture is the filter that i) is identified by the NNR post-filter SEI message that has a particular nnrpf_id value, has nnrpf_mode_idc equal to 1 or 2, and pertains to the picture and ii) is updated by the NNR post-filter SEI message, if any, that has the same nnrpf_id value, has nnrpf_mode_idc equal to 3, and pertains to the picture.

nnrpf_parameter_type_idc specifies the most complex variable type of neural network parameters.
When nnrpf_parameter_type_idc is equal to 0, the network uses only integer parameters. 
When nnrpf_parameter_type_idc is equal to 1, the network uses floating point parameters and possibly integer parameters.
Other values of nnrpf_parameter_type_idc are reserved for future use.

nnrpf_num_parameters_idc specifies the maximum number of neural network parameters for the post processing filter in units of a power of 2048. nnrpf_num_parameters_idc equal to 0 specifies that the maximum number of neural network parameters is not specified.

The variable MaxNumParameters is derived as follows:
	MaxNumParameters = (2048 << nnrpf_num_parameters_idc) − 1

It is a requirement of bitstream conformance that the number of neural network parameters of the post-processing filter shall be equal to or less than MaxNumParameters.

nnrpf_num_kmac_operations_idc specifies the maximum number of operations per pixel of the post-processing filter.

It is a requirement of bitstream conformance that the number of MAC operation (multiply-accumulate) per pixel for the post processing filter is indicated to be equal to or less than nnrpf_num_kmac_operations_idc * 1000.

nnrpf_num_kmac_operations equal to 0 specifies that the number of kMAC operations of the network is not specified.

nnrpf_alignment_zero_bit shall be equal to 0.

nnrpf_uri[ i ] contains the i-th byte of a NULL-terminated UTF-8 character string that contains a URI identifying the neural network used as the post-processing filter.

nnrpf_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnrpf_payload_byte[ i ] for all values of i in the range of 0 to payloadSize − 3, inclusive, shall be a complete bitstream that conforms to ISO/IEC 15938-17.

References

- Miska M. Hannuksela, Emre B. Aksu, Francesco Cricri, Hamed. R. Tavakoli, Maria Santamaria, "AHG9: On post-filter SEI", JVET-X0112, Oct. 2021.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).