| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0076-v3 |
| --- | --- |

| Title: | Non-EE2: Template Matching-based OBMC Design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhuoyi Lv Chuan Zhou Jinrong zhang No.168 Weiwo Rd., Chang'an, Dongguan, Guangdong | Email: | zhuoyi.lv@vivo.com chuan.zhou@vivo.com jinrong.zhang@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents a method of template matching-based OBMC design, where the prediction value of CU boundary samples derivation approach is decided according to the template matching costs. Instead of directly using the weighted prediction, the proposed method chooses from three approaches for each block, including using current block’s motion information only, or using neighboring block’s motion information as well with one of the two blending modes.

On top of ECM-3.1, the simulation results of the proposed method are reported as below:

Test 1. RA: -0.13%, -0.18%, -0.10%, 114%, 118%.

Test 2. RA: -0.13%, -0.21%, -0.16%, 100%, 102%.

Introduction

In ECM, when OBMC is applied, the top and left boundary samples of a CU are refined using neighboring block’s motion information with a weighted prediction as described in JVET-L0101[1], JVET-U0100[2], JVET-K0258[3]. The OBMC sample blending operation is processed for each top/left block at the top/left CU boundaries (top-direction OBMC of top 4×4 row, and left-direction OBMC of left 4×4 column). The original MC result using current block’s motion information (denoted as Pixel1) and the additional MC result using neighboring block’s motion information (denoted as Pixel2) are blended to obtain the new prediction result (denoted as NewPixel).

For luma blocks, the number of blending pixel rows/columns is 4 and the corresponding weighting factors are:

NewPixeli,j=26×Pixel1i,j+6×Pixel2i,j+16≫5

NewPixeli,j=7×Pixel1i,j+Pixel2i,j+4≫3

NewPixeli,j=15×Pixel1i,j+Pixel2i,j+8≫4

NewPixeli,j=31×Pixel1i,j+Pixel2i,j+16≫5

For chroma blocks, the number of blending pixel rows/columns is 1 and the weighting factors are:

NewPixeli,j=26×Pixel1i,j+6×Pixel2i,j+16≫5

Proposal

In this contribution, a template matching-based OBMC scheme is proposed. As shown in Fig. 1, for each top block with a size of 4×4 at the top CU boundary, the above template size equals to 4×1. If N adjacent blocks have the same motion information, then the above template size is enlarged to 4N×1 since the MC operation can be processed at one time, which is in the same manner in ECM-OBMC. For each left block with a size of 4×4 at the left CU boundary, the left template size equals to 1×4 or 1×4N.

Figure 1. Template

For each 4×4 top block (or N 4×4 blocks group), the prediction value of boundary samples is derived following the below steps.

Take block A as the current block and its above neighboring block AboveNeighbor_A for example. The operation for left blocks is conducted in the same manner.

First, three template matching costs (Cost1, Cost2, Cost3) are measured by SAD between the reconstructed samples of a template and its corresponding reference samples derived by MC process according to the following three types of motion information:

Cost1 is calculated according to A’s motion information.

Cost2 is calculated according to AboveNeighbor_A’s motion information.

Cost3 is calculated according to weighted prediction of A’s and AboveNeighbor_A’s motion information with weighting factors as 34 and 14 respectively.

Figure 2. Current block and its neighboring block

Second, choose one out of three approaches to calculate the final prediction results of boundary samples by comparing Cost1, Cost2 and Cost 3.

The original MC result using current block’s motion information is denoted as Pixel1, and the MC result using neighboring block’s motion information is denoted as Pixel2. The final prediction result is denoted as NewPixel.

- If Cost1 is minimum, then NewPixeli,j= Pixel1i,j.

- If Cost2 is minimum, then blending mode 1 is used.

For luma blocks, the number of blending pixel rows is 4.

NewPixeli,0=26×Pixel1i,0+6×Pixel2i,0+16≫5

NewPixeli,1=7×Pixel1i,1+Pixel2i,1+4≫3

NewPixeli,2=15×Pixel1i,2+Pixel2i,2+8≫4

NewPixeli,3=31×Pixel1i,3+Pixel2i,3+16≫5

For chroma blocks, the number of blending pixel rows is 1.

NewPixeli,0=26×Pixel1i,0+6×Pixel2i,0+16≫5

- If Cost3 is minimum, then blending mode 2 is used.

For luma blocks, the number of blending pixel rows is 2.

NewPixeli,0=15×Pixel1i,0+Pixel2i,0+8≫4

NewPixeli,1=31×Pixel1i,1+Pixel2i,1+16≫5

For chroma blocks, the number of blending pixel rows/columns is 1.

NewPixeli,0=15×Pixel1i,0+Pixel2i,0+8≫4

Simulation results

The template matching-based OBMC method was implemented on top of the ECM-3.1 and tested using the common test conditions as defined in JVET-X2017 [4]. Table 1 shows the overall BD-rate performance.

It has to be clarified that, due to our carelessness the results in the first uploaded version were not run according to CTC. For RA configuration, per-class RA configurations have not been used, and for LDB configuration, the test duration of 10sec was set. Just for reference, these results are also reported as nonCTC.

Table 1.1. BD-rate performance of Test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | -0.10% | -0.01% |  |  |
| Class C | -0.22% | -0.21% | -0.11% | 115% | 115% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.21% | -0.24% | 0.03% | 114% | 118% |
| Class F |  |  |  |  |  |

Table 1.2. BD-rate performance of Test 1 (nonCTC)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.09% | -0.11% |  |  |
| Class A2 | -0.15% | -0.11% | -0.12% |  |  |
| Class B | -0.12% | -0.23% | -0.04% |  |  |
| Class C | -0.16% | -0.25% | -0.16% | 114% | 118% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.18% | -0.10% |  |  |
| Class D | -0.19% | -0.06% | -0.11% | 112% | 106% |
| Class F | -0.14% | -0.26% | -0.38% |  |  |

Further optimization to reduce the complexity was tested, in which integer precision is used while 2-tap bilinear filter is used to generate the reference template. Table 2 shows the BD-rate performance.

Table 2.1. BD-rate performance of Test 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.19% | -0.08% | -0.10% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.23% | -0.04% | -0.03% | 101% | 103% |
| Class F | -0.11% | -0.24% | -0.17% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.22% | -0.11% | -0.18% |  |  |
| Class E | -0.05% | 0.98% | -0.15% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.06% | 0.05% | -1.10% |  |  |
| Class F |  |  |  |  |  |

Table 2.2. BD-rate performance of Test 2 (nonCTC)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.24% | -0.17% |  |  |
| Class A2 | -0.15% | -0.21% | -0.21% |  |  |
| Class B | -0.10% | -0.11% | -0.05% |  |  |
| Class C | -0.18% | -0.31% | -0.25% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.21% | -0.16% |  |  |
| Class D | -0.18% | -0.12% | -0.21% | 100% | 101% |
| Class F | -0.12% | -0.24% | -0.17% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.18% | -0.17% | -0.15% | 101% | 101% |
| Class E | -0.21% | 0.60% | -0.31% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.09% | 0.53% | -0.17% | 101% | 105% |
| Class F |  |  |  |  |  |

In ECM, OBMC is indicated for each CU by a CU-level flag. For inter modes except for Merge mode, the value of the flag is set by comparing the RD cost in the encoder and transmitted to the decoder. Considering that this proposed method may decide whether use OBMC or not based on template, one more test was conducted, in which RD cost comparison is skipped and the CU-level OBMC flag is not transmitted. Table 3 shows the BD-rate performance.

Table 3.1. BD-rate performance of Test 3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | -0.10% | -0.07% |  |  |
| Class C | -0.19% | -0.10% | -0.15% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.23% | -0.06% | 0.04% | 101% | 102% |
| Class F |  |  |  |  |  |

Table 3.2. BD-rate performance of Test 3 (nonCTC)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.11% | -0.11% |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.17% | -0.13% |  |  |
| Class C | -0.19% | -0.24% | -0.24% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.17% | -0.06% | -0.10% |  |  |
| Class F |  |  |  |  |  |

Conclusion

The contribution proposes a template matching-based OBMC scheme to derive the prediction value for CU boundary samples. Simulation results show -0.13%, -0.21%, -0.16% coding gain in RA configuration with 100% and102% complexity for encoder and decoder sides. It is recommended to investigate the proposed method in the next EE2 meeting.

References

- Z.-Y. Lin, C.-C. Chen, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), “CE10.2.1: OBMC”, JVET-L0101, Sep. 2018.

- Y.-J. Chang, C.-C. Chen, J. Chen, J. Dong, H. E. Egilmez, N. Hu, H. Huang, M. Karczewicz, J. Li, B. Ray, K. Reuze, V. Seregin, N. Shlyakhov, L. Pham Van, H. Wang, Y. Zhang, Z. Zhang (Qualcomm), “Compression efficiency methods beyond VVC”, JVET-U0100, January 2021.

- C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), “CE10-related: OBMC complexity reduction and parallel blending”, JVET-K0258, July 2018.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).