| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0077-v1 |
| --- | --- |

| Title: | AHG10: Block importance mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. Wennersten J. Enhorn C. Hollmann J. Ström | Email: | per.wennersten@ericsson.com jack.enhorn@ericsson.com christopher.hollmann@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes an updated version of the method in contribution JVET-V0057.

The proposed algorithm for VTM and HM signals CTU QP delta values in pictures that will be used for reference. The QP value to use for each CTU is based on the estimated importance of a given CTU for future pictures and the QP selected is in the range of −2 to +2 relative to the picture QP.

Presented SDR CTC results Y/U/V over VTM-15.0 are for RA -1.92%/-2.98%/-2.83%, for LDB -2.24%/-0.07%/-0.45% and for LDP -2.28%/0.12%/-0.31%. Respectively over HM-16.24 presented results for RA are -2.2%/-7.3%/-6.9%, for LDB -2.6%/-4.3%/-3.4% and for LDP -2.4%/-3.9%/-3.1%.

Presented results on HDR CTC RA over VTM-15.0 are DE100 -7.86%, PSNR-L100 -3.55%, wPSNR Y -1.23%, U -8.14%, V -7.09%, PSNR Y -1.33%, U -6.85%, V -6.63%.

It is proposed to adopt the algorithm into VTM and HM software.

# Introduction

This contribution proposes an updated version of the method in contribution JVET-V0057 [1].

Motivation of the method is if a current picture is followed by an identical picture, the coded blocks in the current picture would be re-used identically in the following picture. In this case, rate*lambda+2*distortion should be minimized instead. VTM approximates this for RA and LD by setting different QP values for pictures depending on position in the GOP.

In practice, the similarity between pictures varies for different sequences, and for different parts of the picture. Because of this, rather than using a fixed QP for each temporal layer, we would like to select individual QP values for each CTU based on how important we think they will be to future pictures. We propose to estimate this importance by measuring the difference between motion compensated pictures.

# Proposal

## Method as presented in JVET-V0057

The estimation of the difference between pictures is based on calculations performed in the temporal filter. QP is only modified for frames where the temporal filter is active, i.e., where POC is divisible by eight.

After motion compensation for an 8x8 block, the temporal filter computes the variance of the original block and SSD, the sum of squared differences between the original block and the corresponding block in the reference picture after motion compensation. Based on this, an error E is computed as

E=0.2∙SSD+5V+5+SSD3200

This formula will give a low E-value (i.e., high importance) to blocks that have a low SSD value, as well as a low relative error (SSD/variance).

The E-value is currently computed per 8x8 block for each reference picture in the temporal filter. For this method, the average E-value in a CTU is calculated for the pictures immediately before and after the current picture. These two values are then averaged to form a value E1 for each CTU in the current picture, and the process is repeated for both pictures at a distance of two, yielding E2 for every CTU. These values are extrapolated to yield an E4 value for each CTU:

E4=maxE1,E2+abs(E2-E1)*3

The E4 value is designed to take into account how quickly the importance fades. If E1 indicates high importance, but E2 indicates low importance, the importance evaporates quickly, and the block is not so important to encode well. However, if both E1 and E2 signal high importance, this indicates that the importance stays for longer and it is more important to code the block well.

For the highest temporal layer that is filtered, where POC is divisible by 8 but not 16, E4 is modified by

E4 = 0.6*E4 + 0.4*Ccenter

This sets the importance closer to medium importance to compensate for these pictures being used as reference for fewer future pictures. Finally, a thresholding is performed to decide a delta QP relative to the picture QP for each CTU.

## Updates to the method

The updated version of the method has corrected a few bugs in the software, one of which were the cause for the low performance on HM in JVET-V0057.

Additionally, in the case where references are only present in one direction, such as first picture of a sequence or Low Delay coding, the only available values for E1 and E2 are used multiplied by two.

The final thresholding was re-tuned according to the following table and Ccenter set to 45.

| E4 | delta QP |
| --- | --- |
| 0-14 | -2 |
| 15-29 | -1 |
| 30-60 | 0 |
| 61-74 | +1 |
| >75 | +2 |

# Results

The proposed method was implemented over VTM-15.0 [2] and HM-16.24 [3]. Only RA and LD results are presented as the method is not applicable for All Intra.

Note that the encoding and decoding times are not reliable. The complexity of the method is low as all heavy calculations (motion compensation, SSD etc.) are already calculated by the temporal filter.

Following are the SDR CTC [4] BD-rate results over VTM-15.0:

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-15.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.07% | -1.53% | -2.61% | 92% | 91% |
| Class A2 | -1.96% | -2.93% | -1.95% | 94% | 92% |
| Class B | -2.48% | -3.85% | -4.20% | 91% | 91% |
| Class C | -1.82% | -3.01% | -1.93% | 113% | 117% |
| Class E |  |  |  |  |  |
| Overall | -1.92% | -2.98% | -2.83% | 97% | 98% |
| Class D | -1.24% | -1.46% | -1.32% | 102% | 97% |
| Class F | -2.72% | -2.97% | -1.90% | 90% | 88% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-15.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.96% | -1.29% | -1.81% | 91% | 94% |
| Class C | -2.93% | 0.33% | 0.59% | 94% | 98% |
| Class E | -0.12% | 1.44% | 0.45% | 93% | 94% |
| Overall | -2.24% | -0.07% | -0.45% | 92% | 95% |
| Class D | -1.33% | 1.08% | 1.54% | 86% | 80% |
| Class F | -2.95% | -2.16% | -1.54% | 92% | 98% |

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-15.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.92% | -1.10% | -1.83% | 92% | 96% |
| Class C | -2.91% | 0.40% | 0.17% | 88% | 91% |
| Class E | -0.37% | 1.78% | 1.58% | 90% | 93% |
| Overall | -2.28% | 0.12% | -0.31% | 90% | 93% |
| Class D | -1.41% | 0.96% | 1.33% | 90% | 87% |
| Class F | -2.97% | -2.33% | -2.61% | 89% | 91% |

Following are the HDR CTC [5] BD-rate results over VTM-15.0:

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-15.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -7.86% | -3.55% | -1.23% | -8.14% | -7.09% | -0.76% | -7.65% | -6.76% | 87% | 99% |
| Class H2 |  |  |  |  |  | -2.32% | -5.45% | -6.39% | 92% | 106% |
| Overall | -7.86% | -3.55% | -1.23% | -8.14% | -7.09% | -1.33% | -6.85% | -6.63% | 89% | 101% |

Following are the SDR CTC results over HM-16.24:

|  | Random Access Main 10 |  |  | Low delay B Main 10 |  |  | Low delay P Main 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -0.7% | -4.5% | -4.7% |  |  |  |  |  |  |
| Class A2 | -1.6% | -6.7% | -6.2% |  |  |  |  |  |  |
| Class B | -3.0% | -9.2% | -9.1% | -3.1% | -4.8% | -4.5% | -2.8% | -4.4% | -3.9% |
| Class C | -2.7% | -7.4% | -6.3% | -3.7% | -5.7% | -4.3% | -3.4% | -5.1% | -3.7% |
| Class E |  |  |  | -0.6% | -1.5% | -0.6% | -0.3% | -1.4% | -0.8% |
| Overall | -2.2% | -7.3% | -6.9% | -2.6% | -4.3% | -3.4% | -2.4% | -3.9% | -3.1% |
|  | -2.1% | -7.1% | -6.8% | -2.6% | -4.2% | -3.4% | -2.3% | -3.8% | -3.0% |
| Class D | -1.7% | -6.7% | -6.2% | -1.7% | -3.1% | -3.1% | -1.6% | -3.8% | -3.4% |
| Class F | -3.3% | -6.6% | -5.6% | -3.6% | -3.9% | -3.8% | -3.5% | -5.4% | -4.9% |
| Enc Time[%] | 97% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  | 91% |  |  |

# Conclusion

In this document an updated version of the method in JVET-V0057 is presented which provides improved compression efficiency with no discernable impact on complexity. It is proposed to adopt the proposed method into VTM and HM software.

# References

[1] P. Wennersten, C. Hollmann, J. Ström, “[AHG10] Block importance mapping”. JVET-V0057, Meeting by Teleconference, April 2021.

[2] VTM-15.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

[3] HM-16.24 reference software, https://vcgit.hhi.fraunhofer.de/jvet/HM.git

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

[5] A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-V2011, Meeting by Teleconference, April 2021

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).