| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting: by teleconference, 12 – 21 January 2022 | Document: JVET-Y0081-v1 |
| --- | --- |

| Title: | AHG11: Transformer based in-loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tong Ouyang Huairui Wang Han Zhu Zhenzhong Chen | Tel: Email: | oyjiyu@whu.edu.cn wanghr827@whu.edu.cn zhuhanlyx@whu.edu.cn zzchen@whu.edu.cn |
| Source: | Wuhan University |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of transformer based in-loop filtering, which employs attention modules for image restoration instead of the convolutional neural network as backbone. The performance is evaluated using PSNR and MS-SSIM based BD-rate reduction in VTM-11.0 + new MCTF, which reports an average gain of {-6.77%, -14.62%, -14.97%} on PSNR and {-6.88%, -17.24%, -17.50%} on MS-SSIM under all intra configuration.

Introduction

Among all the contributions of AHG11, many convolutional neural networks (CNN) based in-loop filter are proposed to improve the coding efficiency of VVC. However, few studies attempted to explore the application of deep learning technologies other than CNN. Although many researches show that there are other technologies with better performance compared with residual CNN, the employment of the up-to-date deep learning technologies in codec is still lacking at present.

Motivated by this, the coding efficiency of transformer based in-loop filter is investigated to give a new perspective on the work of AHG11.

Methodology

In this contribution, we propose to introduce Swin Transformer for Image Restoration (SwinIR) [1] to in-loop filter. SwinIR is adopted to reduce the compression artifacts of reconstructed frames, while theoretically improving the performance of video coding through setting a few control flags. Without modifying the network architecture and training strategy, SwinIR is deployed as an additional filter before ALF. The model structure is illustrated briefly in section 2.1, more detailed information is available in [1, 2, 3]. Moreover, the information of training and test configuration are described in Section 2.2 – 2.3.

Architecture

The architecture of the SwinIR is shown in Fig. 1. SwinIR consists of three modules: shallow feature extraction, deep feature extraction and high-quality (HQ) image reconstruction modules.

Fig. 1 The architecture of SwinIR.

Shallow feature extraction

Given a low-quality image (ILQ) as input, a 3 × 3 convolutional layer (HSF) is adopted to extract shallow feature F0 as

F0=HSFILQ.#1

The convolution layer is good at early visual processing, leading to more stable optimization and better results.

Deep feature extraction

The deep feature extraction module (HDF) contains K residual Swin Transformer blocks (RSTB) and a 3 × 3 convolutional layer. As shown in Fig. 1(a), RSTB is a residual block with Swin Transformer layers (STL) and convolutional layers. STL [2] is based on the standard multi-head self-attention (MSA) and Multi-layer perceptron (MLP) of the original Transformer layer [3] with two LayerNorm layer and several residual connections.

Multi-head self attention

The main differences lie in local attention and the shifted window mechanism compared with the original MSA. Given an input of size H×W×C, Swin Transformer first reshapes the input to a (HW/M2)×M2×C feature by partitioning the input into non-overlapping M×M local windows, where (HW/M2) is the total number of windows. Then, it computes the standard self-attention separately for each window. For a local window feature X, the query, key and value matrices Q, K and V are computed as

Q=XPQ,  K=XPK,  V=XPV,#2

where PQ, PK and PV are projection matrices that are shared across different windows. The attention matrix is thus computed by the self-attention mechanism in a local window as

AttentonQ,K,V=SoftMaxQKTd+B⋅V,#3

where B is the learnable relative positional encoding. In practice, the attention function is performed several times in parallel and concatenates the results for multi-head self-attention (MSA) following the instruction of [3].

Multi-layer perceptron

It has two fully connected (FC) layers with GELU non-linearity between them and is used for further feature transformations. The default value of drop rate is set as 0 in SwinIR.

| - Multi-head self attention (MSA) | - Multi-layer perceptron (MLP) |
| --- | --- |

Fig 2. The architecture of original MSA and MLP in [3].

Image reconstruction

A single convolution layer is used for image reconstruction. Besides, residual learning is adopted to reconstruct the residual between the LQ and the HQ image instead of the HQ image, which can be formulated as

IRHQ=HSwinIRILQ+ILQ,#4

where HSwinIR(⋅) denotes the function of SwinIR.

Loss function

For the task of image restoration, Charbonnier loss is adopted as loss function, where ϵ is set to 10-3.

L=IRHQ-IHQ2+ϵ2.#5

Training

The proposed network is trained with DIV2K dataset, there are 800 images in its training set and 100 images in its validation set. All the images are compressed under the common test conditions (AI configuration and QP= {22, 27, 32, 37, 42}) with VTM-11.0. More information related with the network architecture are reported in Table 2.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3060 12GB |
|  | Framework: | PyTorch v1.10 |
|  | Number of GPUs per Task | 8 |
|  | Epoch: | 1799 |
|  | Batch size: | 8 |
|  | Training time: | 290h |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {22, 27, 32, 37, 42} |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 2e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Inference

The SwinIR model is tested with 5 different QPs= {22, 27, 32, 37, 42} under AI configuration.

Table 3. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | PyTorch v1.10 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 11.51M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 46M |
|  | MAC (Giga) | 11384K/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 9 |
|  | Total FC Layers | 72 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Experimental results

The SwinIR model based in-loop filtering is tested on top of VTM-11.0 + new MCTF [6] according to the common test conditions. Deblocking filter and SAO are disabled while the model is placed before ALF. The experimental results are shown in Table 1. It can be observed that the SwinIR model could bring on average {6.77%, 14.62%, 14.97%} BD-rate savings under AI configuration. There are no test results for other configurations due to insufficient test equipment.

Table 1 Experimental results (All Intra)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.21% | -5.63% | -12.38% | -14.07% | -7.36% | -16.32% | -15.82% | 4938% | 3257020% |
| Class A2 | -7.32% | -5.89% | -16.08% | -13.89% | -6.36% | -17.44% | -13.35% | 2719% | 2465264% |
| Class B | -6.62% | -5.70% | -15.84% | -15.42% | -5.55% | -18.65% | -19.19% | 2337% | 2423351% |
| Class C | -8.09% | -7.50% | -12.96% | -14.87% | -6.37% | -16.92% | -18.97% | 1339% | 1677933% |
| Class E | -10.29% | -9.61% | -15.60% | -16.31% | -9.80% | -16.02% | -18.54% | 2998% | 2835443% |
| Overall | -7.61% | -6.77% | -14.62% | -14.97% | -6.88% | -17.24% | -17.50% | 2500% | 2415181% |
| Class D | -8.06% | -7.57% | -11.60% | -13.35% | -5.97% | -17.44% | -18.32% | 1237% | 1846443% |
| Class F | -4.60% | -3.81% | -8.97% | -8.27% | -3.16% | -12.77% | -13.09% | 1115% | 1470967% |

Conclusion

This contribution provides valuable information on Swin transformer based in-loop filtering. Experimental results show that considerable BD-rate reduction is achieved without any side information or model optimization.

References

[1] J. Liang, J. Cao, G. Sun, K. Zhang, L. Van Gool and R. Timofte, SwinIR: Image Restoration Using Swin Transformer, International Conference on Computer Vision Workshops, 2021, pp. 1833-1844.

[2] Z. Liu, Y. Lin, Y. Cao, H. Hu, Y. Wei, Z. Zhang, S. Lin, and B. Guo. Swin transformer: Hierarchical vision transformer using shifted windows. arXiv preprint arXiv:2103.14030, 2021.

[3] A. Vaswani, N. Shazeer, N. Parmar, J. Uszkoreit, L. Jones, A. N. Gomez, L. Kaiser, and I. Polosukhin. Attention is all you need. arXiv preprint arXiv:1706.03762, 2017.

[4] JVET-T0069, “AHG 11: SSIM based CNN model for in-loop filtering”, T. Ouyang, F. Liu, H. Zhu, Z. Chen, X. Xu, S. Liu, teleconference, 7-16 Oct. 2020.

[5] JVET-U0074, “EE: SSIM based CNN model for in-loop filtering”, T. Ouyang, H. Zhu, Z. Chen, X. Xu, S. Liu, teleconference, 6-15 Jan. 2021.

[6] JVET-W0062, “EE: SSIM based CNN model for in-loop filtering”, T. Ouyang, Y. Guo, Z. Chen, X. Xu, S. Liu, teleconference, 7-16 Jul. 2021.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.