| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0083-v1 |
| --- | --- |

| Title: | AhG10: Report of Teleconference on Viewing Session Preparation for Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Name(s) Mathias Wien Han Zhang Xiang Li | Tel: Email: | wien@lfb.rwth-aachen.de zhhanzhang@tencent.com xlxiangli@tencent.com |
| Source: | AHG4, Tencent |  |  |

_____________________________

- Abstract

This document reports the teleconference on viewing session preparation for deblocking on 2021-11-17 16:00 – 17:15 UTC. Testing sequences and testing conditions were discussed.

Introduction

During JVET-X meeting, JVET-X0063 on deblocking filter setting for VTM was discussed [1]. A visual investigation of the proposal was requested, as recorded in meeting notes under JVET-X0063. Therefore, a teleconference was held to prepare the viewing session.

Discussions

Testing sequences

- Deblocking and ALF proposals have been studied visually on 1080p and 720p content during the VVC development phase. It is suggested to follow this approach and use the candidates suggested by EE1 viewing preparation as listed in the appendix [2].

- If possible, providing encodings of the 10sec versions would be best

- It is very likely, that only 5sec will be evaluated for all cases, to limit the duration of the visual tests, probably last 5sec for the LDB sequences to capture the largest amount of differences.

- The length to be used will be determined in preparation of the viewing sessions.

QPs to be tested

- In the AHG11, QP 39, 42 are suggested for UHD sequences, and 34,39 for the lower resolutions

- It seems that below QP37, there is not much visual difference to be observed

- Suggested approach: prepare QP37, 39, 42

Length of sequences

- See above (2.1) for the length

- Use LDB configuration for the assessment since artifacts accumulate over the whole length of the sequence

- It was also agreed to have viewing sessions for ECM to investigate the visual quality impact.

Scoring

- In current JVET viewing approach, there are 4 scales, i.e., “much better”, “slightly better”, “slightly worse”, “much worse”. There is no “similar quality”

- If there are differences, the established JVET scale should be applicable

- Using that scale is preferred since JVET people are used to it

- If there are only minor or very minor differences, a different scale should be used

  - Proposal to set up a dedicated test for this purpose for/at the meeting.

Schedule of viewing

- AHG11 currently plans for viewing tests for Neural Network-based Video Coding technologies on 10 and 11-Jan-2022.

- Would be nice to integrate the viewing of this proposal into these viewing sessions

- Potentially, an additional experiment could be planned during the first days of the meeting where a different assessment approach with a different scale would be used

- 7-Jan-2022 Viewing materials are uploaded.

Appendix

The list of video sequences, configurations and QPs which are recommended to be included into the viewing for EE1 are copied below:

| Category | Sequence Name | Resolution | fps | Frames | CFG | QP |
| --- | --- | --- | --- | --- | --- | --- |
| UHD | Tango | 3840x2160p | 60fps | 294 | RA | 39, 42 |
|  | Campfire(*) | 3840x2160p | 30fps | 300 | RA | 36, 39 |
|  | DaylightRoad2 | 3840x2160p | 60fps | 300 | RA | 39, 42 |
|  | CatRobot | 3840x2160p | 60fps | 300 | RA | 39, 42 |
| 1080p | Kimono1(**) | 1920x1080p | 24fps | 240 | LDB | 34, 39 |
|  | red_kayak(***) | 1920x1080p | 30fps | 300 | LDB | 34, 39 |
|  | BQTerrace | 1920x1080p | 60fps | 600 | LDB | 34, 39 |
|  | ArenaOfValor | 1920x1080p | 60fps | 600 | LDB | 34, 39 |
| 720p | KristenAndSara | 1280x720p | 60fps | 600 | LDB | 34, 39 |
|  | Johnny | 1280x720p | 60fps | 600 | LDB | 34, 39 |
|  | FourPeople | 1280x720p | 60fps | 600 | LDB | 34, 39 |
|  | Jets (****) | 1280x720p | 60fps | 550 | LDB | 34, 39 |
|  | Spincalendar (****) | 1280x720p | 60fps | 550 | LDB | 34, 39 |

(*) 300 frame to be encoded, frames 151~300 to be viewed

(**) sequences outside of AhG11 CTC, can be found at ftp://ftp.ient.rwth-aachen.de/ce/JVET-L1031_Deblocking/SRC/

Kimono1_1920x1080_24.yuv (md5sum   4a83005bc719012ac148dd3898e5e4ed)

(***) sequences outside of AhG11 CTC, can be found at ftp://ftp.ient.rwth-aachen.de/ce/JVET-P2025_CCALF/RedKayak/

red_kayak_1-300_1920x1080_30Hz_8bit_420.zip(md5sum  33b276220e7ea308d3fdf98e5fb5d491)

(****) optional sequences outside of AhG11 CTC, can be found at ftp://jvet@ftp.ient.rwth-aachen.de/ahg/candidates/720p

Jets_1280x720_60fps_bt709_420.yuv (md5sum  9b8c26f3ea0276832c41e0a4d0e6d364)

Spincalendar_1280x720_60fps_bt709_420.yuv(md5sum  ba830f17e1948c4134cfcc2de73cdf42)

- Reference

- H. Zhang, X. Li, S. Liu, “AHG10: Deblocking filter setting for VTM”, JVET-X0063, Oct. 2021.

- E. Alshina, M. Wien, A. Segall, “AhG11/EE1 viewing preparation report”, JVET-Y0045, Jan. 2022.