| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0089-v2 |
| --- | --- |

| Title: | Non-EE2: DMVR with BCW enabled |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, A.Robert, Y.Chen, F.Galpin, | Email: | philippe.bordes@interdigital.com antoine.robert@interdigital.com ya.chen@interdigital.com franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes enabling BCW with DMVR. It is reported the simulation results of the proposed methods on top of the ECM-3.1 is {-0.03%, -0.01%, -0.04%} for YUV in RA configuration and the complexity (encoding/decoding times) is unchanged.

# Introduction

In ECM-3.1 the building of the list for BM-HMVP candidates excludes candidates with non-equal BCW weights (addBMMergeHMVPCand) and in the list of BM candidates the BCW index is unforced to equal weights (getInterBMCandidates).

# Proposal

One proposes to enable BCW with DMVR. The list of BM candidates contains the inherited BCW weights and DMVR process is unchanged except the computation of the distortion is made using MRSAD or MRSATD if the weights are non-equal and the bi-prediction is weighted with BCW weights.

# Tests

The proposed method is tested on ECM-3.1 [1] with CTC and Random-Access configuration (Table 1).

Table 1: DMVR with BCW enabled (RA).

|  | ECM-3.1 - Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,08% | -0,10% | -0,16% | 100% | 100% |
| Class A2 | -0,05% | -0,01% | -0,03% | 101% | 99% |
| Class B | -0,02% | 0,05% | -0,01% | 100% | 101% |
| Class C | 0,01% | -0,02% | 0,02% | 97% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,01% | -0,04% | 100% | 99% |
| Class D | 0,01% | -0,02% | 0,17% | 98% | 100% |
| Class F | 0,05% | 0,03% | 0,01% | 101% | 100% |

# Conclusion

In this contribution, a modification of DMVR to include candidates with non-equal BCW weights is proposed. It is reported a BD-rate gain of {-0.03%, -0.01%, -0.04%} for YUV in RA configuration is achieved without additional encoding and decoding time increase. We therefore suggest including this change in ECM.

# References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, July. 2021

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).