| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Online, 12–19 Jan 2022 | Document: JVET-Y0099-v1 |
| --- | --- |

| Title: | VVC level 4.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | G. Martin-Cocher | Email: | first.last @ interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution proposes to discuss whether a VVC level 4.2 is needed to address use-cases such as Cloud/Online Gaming as results on some non-CTC 1080p sequences indicates that a bitrate above the main and high tier limit is reached. It may be desirable to rely on a “Main Tier” to address this use-case rather than on a “high tier”.

# Introduction

The current VVC Levels and tiers are defined in table A of VVC / H.266 [1]:

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 1.0 | 16 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2.0 | 32 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 35 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3.0 | 48 | 552 960 | 6 000 | - | 30 | 4 | 2 |
| 3.1 | 51 | 983 040 | 10 000 | - | 40 | 9 | 3 |
| 4.0 | 64 | 2 228 224 | 12 000 | 30 000 | 75 | 25 | 5 |
| 4.1 | 67 | 2 228 224 | 20 000 | 50 000 | 75 | 25 | 5 |
| 5.0 | 80 | 8 912 896 | 25 000 | 100 000 | 200 | 110 | 10 |
| 5.1 | 83 | 8 912 896 | 40 000 | 160 000 | 200 | 110 | 10 |
| 5.2 | 86 | 8 912 896 | 60 000 | 240 000 | 200 | 110 | 10 |
| 6.0 | 96 | 35 651 584 | 80 000 | 240 000 | 600 | 440 | 20 |
| 6.1 | 99 | 35 651 584 | 120 000 | 480 000 | 600 | 440 | 20 |
| 6.2 | 102 | 35 651 584 | 180 000 | 800 000 | 600 | 440 | 20 |
| * The level numbers in this table are in the form of "majorNum.minorNum", and the value of general_level_idc for each of the levels is equal to majorNum * 16 + minorNum * 3. |  |  |  |  |  |  |  |

# Cloud Gaming and streaming

As illustrated in contribution JVET-Y0043, Cloud gaming services have a typical bitrate ranging from 10 to 35 Mbps, with a max peak bandwidth at up to 50Mbps for PC/TV terminals for 1080p 60fps. It may be desirable to rely on a “Main Tier” to address this use-case rather than on a “high tier”.

GforceNow allows customisation by a user: [2]

The “default” max bitrate for 1080p 60fps is set at 27Mbps. It allows for customisation at up to 50Mbps.

Apple HLS [3]:

- 1.6a. For maximum compatibility, some HEVC variants SHOULD be less than or equal to Main 10 Profile, Level 4.0, Main Tier.

- 1.6b. Profile, Level, and Tier for HEVC MUST be less than or equal to Main 10 Profile, Level 5.1, High Tier.

# Example test results on some gaming sequences:

3GPP SA4 [4]

VTM-SCC 11 (LD-B)

|  | sequence | QPISlice |  | kbps |  |  | Y psnr |  | U psnr |  | V psnr |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baolei-Man | 22 |  |  | 20895.93 | 41.74 |  | 42.80 |  | 44.22 |  |  |  |
| Class Gaming | 1920x1080_60Hz_8bit_420 | 27 |  |  | 8720.23 | 37.92 |  | 39.93 |  | 41.47 |  |  |  |
| HD Tencent |  | 32 |  |  | 3973.14 | 34.65 |  | 37.77 |  | 39.26 |  |  |  |
|  |  | 37 |  |  | 1922.01 | 31.80 |  | 35.93 |  | 37.65 |  |  |  |
|  | jianling-Temple | 22 |  |  | 65386.34 | 39.39 |  | 40.93 |  | 42.38 |  |  |  |
|  | 1920x1080_60Hz_8bit_420 | 27 |  |  | 28277.70 | 34.56 |  | 38.13 |  | 39.96 |  |  |  |
|  |  | 32 |  |  | 11610.23 | 30.77 |  | 36.39 |  | 38.37 |  |  |  |
|  |  | 37 |  |  | 4217.91 | 27.88 |  | 34.97 |  | 37.10 |  |  |  |
| Class Gaming | MINECRAFT 1920x1080_60Hz_8bit_420 | 22 |  |  | 41769.47 | 43.06 |  | 44.86 |  | 45.53 |  |  |  |
| HD |  | 27 |  |  | 14578.54 | 36.74 |  | 41.89 |  | 42.85 |  |  |  |
| DERF_TWITCH |  | 32 |  |  | 5527.24 | 33.22 |  | 40.03 |  | 41.02 |  |  |  |
|  |  | 37 |  |  | 2504.42 | 30.78 |  | 38.31 |  | 39.50 |  |  |  |

DMV sequence JVET-Y0041:

|  |  |  |  |  | VTM 15 LD-P |  |  |  |  | VTM 15 LD-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr |  | U psnr |  | V psnr |
| DMV | Level1 | 22 | 43058,33 | 39,22 | 41,69 | 41,06 | 41833,99 |  | 39,22 |  | 41,76 |  |
| HD |  | 27 | 13479,21 | 34,93 | 39,45 | 38,68 | 13289,18 |  | 34,96 |  | 39,51 |  |
|  |  | 32 | 5574,08 | 31,99 | 37,75 | 36,99 | 5527,72 |  | 32,01 |  | 37,77 |  |
|  |  | 37 | 2478,73 | 29,44 | 35,18 | 35,46 | 2466,36 |  | 29,45 |  | 35,20 |  |
|  | Level2 | 22 | 12046,02 | 40,79 | 42,97 | 44,83 | 11637,88 |  | 40,84 |  | 43,04 |  |
|  |  | 27 | 4450,85 | 37,56 | 40,58 | 42,42 | 4347,58 |  | 37,60 |  | 40,64 |  |
|  |  | 32 | 1891,87 | 34,88 | 38,65 | 40,38 | 1863,68 |  | 34,91 |  | 38,67 |  |
|  |  | 37 | 878,66 | 32,61 | 37,03 | 38,93 | 868,64 |  | 32,63 |  | 37,08 |  |
|  | DarkTree | 22 | 38640,25 | 39,38 | 43,43 | 44,51 | 35648,57 |  | 39,37 |  | 43,51 |  |
|  |  | 27 | 10146,97 | 35,81 | 42,18 | 43,38 | 9411,50 |  | 35,86 |  | 42,27 |  |
|  |  | 32 | 3101,93 | 33,32 | 41,04 | 42,31 | 2913,05 |  | 33,37 |  | 41,12 |  |
|  |  | 37 | 1042,35 | 31,37 | 39,80 | 41,12 | 1003,96 |  | 31,42 |  | 39,86 |  |
|  | FontainebleauCinematic | 22 | 26142,80 | 41,72 | 43,00 | 42,86 | 25385,31 |  | 41,72 |  | 43,04 |  |
|  |  | 27 | 8096,77 | 37,86 | 40,23 | 40,40 | 7826,23 |  | 37,85 |  | 40,27 |  |
|  |  | 32 | 2470,47 | 34,92 | 38,57 | 38,98 | 2418,51 |  | 34,93 |  | 38,60 |  |
|  |  | 37 | 860,88 | 32,68 | 37,06 | 37,71 | 854,00 |  | 32,69 |  | 37,07 |  |
|  | FontainebleauFPV | 22 | 122034,67 | 38,45 | 38,63 | 38,43 | 120910,36 |  | 38,45 |  | 38,65 |  |
|  |  | 27 | 53134,00 | 33,59 | 34,67 | 34,66 | 52608,76 |  | 33,59 |  | 34,68 |  |
|  |  | 32 | 23513,10 | 29,77 | 32,67 | 33,10 | 23202,19 |  | 29,76 |  | 32,69 |  |
|  |  | 37 | 9423,02 | 26,58 | 30,81 | 31,82 | 9257,73 |  | 26,56 |  | 30,83 |  |

# Proposed level 4.2

Table A.1 – General tier and level limits

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 1.0 | 16 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2.0 | 32 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 35 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3.0 | 48 | 552 960 | 6 000 | - | 30 | 4 | 2 |
| 3.1 | 51 | 983 040 | 10 000 | - | 40 | 9 | 3 |
| 4.0 | 64 | 2 228 224 | 12 000 | 30 000 | 75 | 25 | 5 |
| 4.1 | 67 | 2 228 224 | 20 000 | 50 000 | 75 | 25 | 5 |
| 4.2 | 70 | 2 228 224 | 50 000 | 75 000 | 75 | 25 | 5 |
| 5.0 | 80 | 8 912 896 | 25 000 | 100 000 | 200 | 110 | 10 |
| 5.1 | 83 | 8 912 896 | 40 000 | 160 000 | 200 | 110 | 10 |
| 5.2 | 86 | 8 912 896 | 60 000 | 240 000 | 200 | 110 | 10 |
| 6.0 | 96 | 35 651 584 | 80 000 | 240 000 | 600 | 440 | 20 |
| 6.1 | 99 | 35 651 584 | 120 000 | 480 000 | 600 | 440 | 20 |
| 6.2 | 102 | 35 651 584 | 180 000 | 800 000 | 600 | 440 | 20 |
| * The level numbers in this table are in the form of "majorNum.minorNum", and the value of general_level_idc for each of the levels is equal to majorNum * 16 + minorNum * 3. |  |  |  |  |  |  |  |

Table A.2 – Tier and level limits for the video profiles

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 1.0 | 16 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2.0 | 32 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 35 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3.0 | 48 | 552 960 | 6 000 | - | 30 | 4 | 2 |
| 3.1 | 51 | 983 040 | 10 000 | - | 40 | 9 | 3 |
| 4.0 | 64 | 2 228 224 | 12 000 | 30 000 | 75 | 25 | 5 |
| 4.1 | 67 | 2 228 224 | 20 000 | 50 000 | 75 | 25 | 5 |
| 4.2 | 70 | 2 228 224 | 50 000 | 75 000 | 75 | 25 | 5 |
| 5.0 | 80 | 8 912 896 | 25 000 | 100 000 | 200 | 110 | 10 |
| 5.1 | 83 | 8 912 896 | 40 000 | 160 000 | 200 | 110 | 10 |
| 5.2 | 86 | 8 912 896 | 60 000 | 240 000 | 200 | 110 | 10 |
| 6.0 | 96 | 35 651 584 | 80 000 | 240 000 | 600 | 440 | 20 |
| 6.1 | 99 | 35 651 584 | 120 000 | 480 000 | 600 | 440 | 20 |
| 6.2 | 102 | 35 651 584 | 180 000 | 800 000 | 600 | 440 | 20 |
| * The level numbers in this table are in the form of "majorNum.minorNum", and the value of general_level_idc for each of the levels is equal to majorNum * 16 + minorNum * 3. |  |  |  |  |  |  |  |

# References

| [1] | "H.266," [Online]. Available: https://www.itu.int/rec/T-REC-H.266. |
| --- | --- |
| [2] | "GeforceNow," [Online]. Available: https://play.geforcenow.com/mall/#/layout/settings. |
| [3] | "HLS HEVC," [Online]. Available: https://developer.apple.com/documentation/http_live_streaming/hls_authoring_specification_for_apple_devices. |
| [4] | SA4, "SA4-S5," [Online]. Available: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-5-Gaming/VTM/. |

# Patent rights declaration(s)

Interdigital does not have any current or pending patent rights relating to the technology described in this contribution.