| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0102-v3 |
| --- | --- |

| Title: | On the balance of ECM coding gains between luma and chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, P. Andrivon, E. Thomas | Tel: Email: | fabricel@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

Thanks to the introduction of new coding tools, in particular cross-component tools, ECM coding gain over VVC is larger in chroma than in luma.

This contribution proposes to modify ECM encoder configurations in AI, RA and LB to balance the chroma and luma coding gains, through modified chroma QP mapping tables. Two encoder configurations are tested, leading to the following results (for Y/Cb/Cr) over VTM-11.0+V0056.

- ECM-3.1: AI: -6.8% / -12.3% / -13.2% ; RA: -14.8%/-18.6%/-19.4%     ; LB : -12.3% / -21% / -21%

- config. 1:  AI: -8.1% /-2.2% /-3.4% ;       RA: -15.6% / -11.5% / -12.8% ; LB: -13% / -6.3% / -6.2%

- config. 2:  AI: -7.3%/-7.9%/-8.7%;          RA: -15.2% / -17.1% / -18.1% ; LB : -12.9% / -9.8% / -9.8%

It is proposed to adopt configuration 1 for AI and RA, and configuration 2 for LB.

# Introduction

Some coding tools introduced in ECM, like CCSAO, improved CCALF, MMLM, bring coding gain especially for chroma. As a results, ECM gains over VTM are significantly larger in chroma than in luma.

During last JVET meeting, it was orally mentioned that it would make sense to balance the luma and chroma gains in a finer way than with current ECM-3.1 encoder configurations.

This contribution proposes modified AI, RA and LDB encoder configuration files, that shift some chroma coding gains towards the luma component.

# Description

To improve the coding efficiency balance between luma and chroma, it is proposed to modify the chroma QP mapping tables for AI, RA and LDB, through the “--QpOutValCb” encoding parameter.

Two alternative configurations are proposed for AI, RA and LDB, as shown by the following Table 1.

Table 1 : proposed modifications to the chroma QP mappings tables (QpOutValCb encoder parameter)

|  | AI | RA | LDB |
| --- | --- | --- | --- |
| ECM-3.1 | 17 29 34 41 | 17 23 35 39 | 17 23 35 39 |
| proposed config 1 | 17 30 35 42 | 17 23 36 40 | 17 24 36 40 |
| proposed config 2 | 17 29 35 42 | 17 23 35 41 | 17 23 36 40 |

# Experimental results

This section shows the results obtained with each proposed configuration, in Ahg12 testing conditions [1]. Table 2, Table 3 and Table 4 respectively show the performances of ECM-3.1 with current ECM configuration, proposed configuration 1 and proposed configuration 2. VTM-11.0+V0056 is used as the anchor.

As can be seen, the results of ECM over VTM-11.0+V0056 are (Y/Cb/Cr):

AI: -6.8% / -12.3% / -13.2% ; RA: -14.8% / -18.6% / -19.4% ; LB : -12.3% / -21% / -21.1%

With tested configuration 1, ECM-3.1 performances over VTM-11.0+V0056 are:

AI: -8.1% / -2.2% / -3.4% ; RA: -15.6% / -11.5% / -12.8% ; LB: -13% / -6.3% / -6.2%

And with tested configuration 2, ECM-3.1 performances over VTM-11.0+V0056 are:

AI: -7.3% / -7.9%/ -8.7% ; RA:  -15.18% / -17.08% / -18.05% ;  LB : -12.9% / -9.8% / -9.5%

Furthermore, configurations 1 and 2 do not change the coding and decoding times compared to current ECM configuration. In addition, Table 5 and Table 6 respectively give the comparative results of proposed configurations 1 and 2 over ECM-3.1. These are summarized as follows.

With tested configuration 1, ECM-3.1 performances over the current configuration are (Y/Cb/Cr):

AI: -1.4% / 11.9% / 11.4% ; RA: -1.00% / 8.47% / 7.93% ; LB: -0.8% / 18.9% / 19.5%

And with tested configuration 2, ECM-3.1 performances over VTM-11.0+V056 are:

AI: -0.6% / 4.5% / 4.6% ; RA:  -0.45% / 1.60% / 1.43%;  LB : -0.64% / 13.55% / 13.74%

From the obtain results, it can be seen the proposed configurations allow shifting some chroma gain towards the luma gain. Moreover, configuration 1 brings at least -1.4%  and -1% luma gain for AI and RA, with 11% and 8% loss in chroma, thus seems the most interesting among the two tested configs.

For LDB, configuration 2 seems a better trade-off.

Table 2: ECM-3.1 performances over VTM-11.0+V0056

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0+V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.98% | -12.44% | -16.63% | 290% | 248% |
| Class A2 | -6.44% | -13.13% | -11.99% | 277% | 235% |
| Class B | -6.05% | -13.75% | -14.21% | 276% | 240% |
| Class C | -7.06% | -10.21% | -10.67% | 269% | 225% |
| Class E | -7.58% | -11.55% | -12.41% | 263% | 259% |
| Overall | -6.75% | -12.28% | -13.16% | 275% | 240% |
| Class D | -5.86% | -8.45% | -8.23% | 265% | 225% |
| Class F | -10.80% | -15.69% | -15.66% | 193% | 285% |
| Class TGM | -15.62% | -18.73% | -18.25% | 265% | 431% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0+V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -14.95% | -16.42% | -21.85% | 347% | 541% |
| Class A2 | -15.84% | -20.38% | -20.42% | 339% | 604% |
| Class B | -13.57% | -19.94% | -19.32% | 315% | 540% |
| Class C | -15.35% | -17.27% | -16.98% | 321% | 487% |
| Class E |  |  |  |  |  |
| Overall | -14.77% | -18.61% | -19.42% | 327% | 538% |
| Class D | -16.10% | -17.65% | -16.99% | 319% | 501% |
| Class F | -13.54% | -18.02% | -17.76% | 274% | 429% |
| Class TGM | -14.65% | -18.96% | -19.20% | 302% | 323% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0+V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -12.28% | -24.89% | -24.11% | 258% | 373% |
| Class C | -12.51% | -18.81% | -18.94% | 264% | 329% |
| Class E | -12.11% | -17.24% | -19.15% | 248% | 372% |
| Overall | -12.31% | -20.95% | -21.14% | 257% | 357% |
| Class D | -14.09% | -20.17% | -19.36% | 254% | 342% |
| Class F | -12.15% | -18.99% | -18.67% | 246% | 389% |
| Class TGM | -14.02% | -21.79% | -22.13% | #NOMBRE! | #NOMBRE! |

Table 3: VTM-11.0+ V0056 vs. proposed configuration 1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 + V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -8.49% | -0.75% | -8.01% | 287% | 248% |
| Class A2 | -9.03% | -2.60% | -2.27% | 272% | 233% |
| Class B | -7.01% | -3.62% | -3.22% | 274% | 240% |
| Class C | -8.08% | -0.75% | -1.03% | 265% | 225% |
| Class E | -8.34% | -2.99% | -3.44% | 259% | 266% |
| Overall | -8.05% | -2.23% | -3.41% | 271% | 241% |
| Class D | -6.82% | 1.17% | 1.58% | 262% | 228% |
| Class F | -11.86% | -8.95% | -9.01% | 192% | 287% |
| Class TGM | -17.28% | -16.59% | -16.04% | 196% | 321% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0 + V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -16.44% | -8.48% | -16.84% | 346% | 544% |
| Class A2 | -17.29% | -14.32% | -14.90% | 338% | 602% |
| Class B | -14.01% | -12.66% | -11.70% | 317% | 543% |
| Class C | -15.85% | -10.22% | -9.63% | 322% | 490% |
| Class E |  |  |  |  |  |
| Overall | -15.64% | -11.51% | -12.82% | 328% | 540% |
| Class D | -16.52% | -10.82% | -9.94% | 320% | 506% |
| Class F | -14.38% | -12.93% | -12.52% | 273% | 430% |
| Class TGM | -15.96% | -16.59% | -16.80% | 302% | 346% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0 + V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -12.98% | -10.74% | -8.64% | 259% | 372% |
| Class C | -13.43% | -3.76% | -3.75% | 264% | 356% |
| Class E | -12.47% | -2.40% | -5.23% | 248% | 405% |
| Overall | -13.00% | -6.33% | -6.16% | 258% | 374% |
| Class D | -14.68% | -5.15% | -4.00% | 254% | 347% |
| Class F | -13.48% | -9.28% | -8.63% | 246% | 419% |
| Class TGM | -15.69% | -17.99% | -18.55% | 276% | 352% |

Table 4: VTM-11.0+ V0056 vs. proposed configuration 2

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0+V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -7.65% | -7.87% | -13.30% | 289% | 249% |
| Class A2 | -7.40% | -9.55% | -8.78% | 275% | 235% |
| Class B | -6.41% | -9.39% | -8.90% | 275% | 241% |
| Class C | -7.50% | -5.69% | -5.77% | 268% | 225% |
| Class E | -7.98% | -6.94% | -7.64% | 262% | 260% |
| Overall | -7.29% | -7.93% | -8.71% | 273% | 241% |
| Class D | -6.29% | -3.72% | -3.28% | 265% | 227% |
| Class F | -11.26% | -12.41% | -12.38% | 193% | 287% |
| Class TGM | -16.28% | -17.74% | -17.30% | 196% | 321% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0+V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -16.05% | -13.78% | -20.74% | 346% | 542% |
| Class A2 | -16.28% | -19.47% | -19.42% | 337% | 601% |
| Class B | -13.73% | -18.53% | -17.70% | 316% | 544% |
| Class C | -15.51% | -15.95% | -15.43% | 322% | 490% |
| Class E |  |  |  |  |  |
| Overall | -15.18% | -17.08% | -18.05% | 328% | 539% |
| Class D | -16.24% | -16.63% | -15.78% | 319% | 505% |
| Class F | -13.87% | -16.59% | -16.11% | 273% | 431% |
| Class TGM | -15.38% | -17.83% | -18.18% | 294% | 329% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0+V0056 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -12.84% | -13.60% | -11.86% | 260% | 371% |
| Class C | -13.24% | -7.13% | -7.08% | 266% | 329% |
| Class E | -12.45% | -6.96% | -10.05% | 249% | 372% |
| Overall | -12.88% | -9.79% | -9.81% | 259% | 357% |
| Class D | -14.58% | -8.31% | -7.89% | 256% | 344% |
| Class F | -13.18% | -11.16% | -10.26% | 247% | 391% |
| Class TGM | -15.38% | -18.73% | -19.19% | 278% | 345% |

Table 5: proposed configuration 1 vs. ECM-3.1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.62% | 13.97% | 10.58% | 99% | 100% |
| Class A2 | -2.78% | 12.84% | 11.29% | 98% | 99% |
| Class B | -1.03% | 12.27% | 12.98% | 99% | 100% |
| Class C | -1.10% | 10.62% | 10.81% | 99% | 100% |
| Class E | -0.83% | 9.79% | 10.57% | 99% | 103% |
| Overall | -1.40% | 11.87% | 11.42% | 99% | 100% |
| Class D | -1.03% | 10.42% | 10.52% | 99% | 101% |
| Class F | -1.20% | 8.14% | 8.05% | 99% | 101% |
| Class TGM | -2.03% | 2.77% | 2.88% | 74% | 75% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.70% | 9.21% | 6.39% | 100% | 101% |
| Class A2 | -1.67% | 7.66% | 6.78% | 100% | 100% |
| Class B | -0.50% | 8.73% | 9.06% | 101% | 101% |
| Class C | -0.58% | 8.21% | 8.53% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.00% | 8.47% | 7.93% | 100% | 100% |
| Class D | -0.49% | 7.82% | 7.97% | 101% | 101% |
| Class F | -0.95% | 6.09% | 6.24% | 100% | 100% |
| Class TGM | -1.52% | 3.03% | 3.14% | 100% | 107% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.80% | 19.45% | 21.21% | 100% | 100% |
| Class C | -1.06% | 18.68% | 18.91% | 100% | 108% |
| Class E | -0.42% | 18.31% | 17.48% | 100% | 109% |
| Overall | -0.79% | 18.91% | 19.51% | 100% | 105% |
| Class D | -0.67% | 18.96% | 19.05% | 100% | 101% |
| Class F | -1.52% | 12.31% | 12.70% | 100% | 108% |
| Class TGM | -1.97% | 5.43% | 5.43% | #DIV/0! | #DIV/0! |

Table 6: proposed configuration 2 vs. ECM-3.1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | 4.92% | 3.57% | 100% | 100% |
| Class A2 | -1.00% | 3.81% | 3.23% | 99% | 100% |
| Class B | -0.37% | 4.56% | 5.51% | 100% | 100% |
| Class C | -0.46% | 4.51% | 4.88% | 100% | 100% |
| Class E | -0.43% | 4.72% | 5.03% | 100% | 101% |
| Overall | -0.56% | 4.51% | 4.58% | 100% | 100% |
| Class D | -0.45% | 4.60% | 4.78% | 100% | 101% |
| Class F | -0.50% | 3.51% | 3.53% | 100% | 101% |
| Class TGM | -0.76% | 1.16% | 1.11% | 74% | 75% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | 2.83% | 1.20% | 100% | 100% |
| Class A2 | -0.50% | 1.05% | 1.04% | 100% | 100% |
| Class B | -0.18% | 1.42% | 1.68% | 100% | 101% |
| Class C | -0.18% | 1.34% | 1.57% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | 1.60% | 1.43% | 100% | 100% |
| Class D | -0.15% | 1.02% | 1.27% | 100% | 101% |
| Class F | -0.37% | 1.57% | 1.77% | 100% | 101% |
| Class TGM | -0.83% | 1.28% | 1.23% | 97% | 102% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.64% | 14.39% | 15.44% | 100% | 100% |
| Class C | -0.83% | 13.68% | 13.91% | 101% | 100% |
| Class E | -0.40% | 11.98% | 10.68% | 100% | 100% |
| Overall | -0.64% | 13.55% | 13.74% | 101% | 100% |
| Class D | -0.55% | 14.08% | 13.42% | 101% | 100% |
| Class F | -1.14% | 9.20% | 9.89% | 101% | 100% |
| Class TGM | -1.54% | 3.96% | 4.06% | #DIV/0! | #DIV/0! |

# Conclusion

It is proposed to improve the balance of ECM coding gains over VTM between chroma and luma channels, by shifting some chroma gains towards luma gains.

Among the proposed tested configuration, the most interesting trade-offs are obtained with configuration 1 for AI and RA, and configuration 2 for LDB.

It is proposed to adopt these selected encoder configurations, which would lead to following BD-rate difference with current ECM configuration:

AI: -1.4% / 11.9% / 11.4% ; RA: -1% ; 8.5% ; 7.9% ; LB: -0.64% / 13.55% / 13.74%

# References

[1] JVET-W2017-v1, “Common test conditions and evaluation procedures for enhanced compression tool testing”, Marta Karczewicz, Yan Ye, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6–15 October 2021.

# Patent rights declaration(s)

Xiaomi does not have any current or pending patent rights relating to the technology described in this contribution.