| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0107-v1 |
| --- | --- |

| Title: | AHG9: Text improvement for the film grain SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas | Tel: Email: | thomase@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

It is asserted that the description text and formula of the film grain SEI message may be improved regarding the parameter fg_blending_mode_id. The value ranges for the variables x and y may not be correct for all the values of c from 0 to 2.

A possible the text modification is proposed to the align the text with the original intent.

# Introduction

This contributions suggests a text improvement for the description text and formula of the film grain SEI message. In the formula, it appears that the value ranges for the variable x and y may not be correct for all the values of c from 0 to 2.

This contribution suggests to modify the text of the align with the original intent.

# Proposed changes (yellow highlighted)

fg_blending_mode_id identifies the blending mode used to blend the simulated film grain with the decoded images as specified in Table 6. fg_blending_mode_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for fg_blending_mode_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with fg_blending_mode_id equal to 2 or 3.

Table 6 – fg_blending_mode_id values

| Value | Description |
| --- | --- |
| 0 | Additive |
| 1 | Multiplicative |

Depending on the value of fg_blending_mode_id, the blending mode is specified as follows:

–	If fg_blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Idecoded[ c ][ x ][ y ] + G[ c ][ x ][ y ] )	(18)

–	Otherwise (fg_blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Idecoded[ c ][ x ][ y ] +	(19)
			Round( ( Idecoded[ c ][ x ][ y ] * G[ c ][ x ][ y ] ) ÷ ( ( 1  <<  fgBitDepth[ c ] ) − 1 ) ) )

where Idecoded[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the decoded image Idecoded, G[ c ][ x ][ y ] is the simulated film grain value at the same position and colour component, and fgBitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays Igrain[ c ][ x ][ y ], Idecoded[ c ][ x ][ y ], and G[ c ][ x ][ y ], where c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples – 1 for c = 0 and x = 0.. PicWidthInSamplesC − 1, and y = 0.. PicHeightInSamplesC – 1 for c = 1..2.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).