| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0108-v2 |
| --- | --- |

| Title: | AhG3/AhG12 Modification of JVET CTC for environmental considerations. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Franck Galpin Miloš Radosavljević Edouard Francois | Email: | first.last @ interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In this contribution, new CTCs are proposed, aiming at reducing the environmental footprint of some of the tests conducted by the JVET contributors. As the overall complexity of the reference software steadily increases, required total computational power increases accordingly. This computational power is directly impacting the environmental footprint. In this contribution, a specific focus on the CO2e (Carbon Dioxide Equivalent) footprint is made, and an attempt to estimate this footprint for current reference software on the CTC is presented.

# Current process footprint

The estimation of the exact footprint of each computation of a full CTC is a complex task to carry. Many factors are involved and are different for each setup:

- Core efficiency (usually depending on the grid generation)

- Run policy (build type, SIMD level, turbo level etc.)

- Data center efficiency: cooling efficiency and policy, datacenter location etc.

- Life cycle methodology: rules greatly vary from one methodology to another (e.g., 14064-1:2018 [2])

- Grid replacement policy (e.g., every 4 years)

- Energy generation footprint, measured in CO2e/kWh

- Methodology for CO2e emission computation (e.g., CO2e at 100 years etc.)

In the following, it is tried to give rough estimates and ranges of current exploratory reference software (ECM) when running a full CTC set.

## Current CTC

The current CTC used to estimate the CO2e footprint are the one described in JVET-X2017. It requires to encode for 3 configurations (AI/RA/LDB) the common sequences set. The tests are run including the optional TGM classes but without other optional classes (H1, H2 etc).

Experiment on our grid shows a total computation time for the CTC between ~886 days and ~996 days depending on the build options, turbo settings etc. For the remaining, we use an estimate of 22000 hours of computation.

## Using vendor estimate

A first rough estimate of the CO2e footprint is given by the total electricity consumption (TEC). However, the TEC greatly varies between datacenter, depending on the datacenter location and efficiency, cooling policy adopted etc.

### Direct energy consumption for 1 full run of CTC

In [1], an example is given by the vendor for a typical grid unit. We choose this unit because Life Cycle figures are also available, however specifications are likely below a typical grid unit required for JVET CTC (for example amount of memory). We assume that the figures provided by the vendor correspond to the default configuration. Assuming an ideal use of this unit, providing 12 cores for computation, and using just the direct consumption for the CTC, we find a total consumption C for the whole CTC of:

C = 1760kWh/y / (12 cores * 365d * 24h) * 22000h ~ 370 kWh

This estimate is a lower bound of direct energy consumption. Other computation given another unit can be done using data in [3].

### CO2 emission for 1 full run of CTC

In [1], an attempt to estimate the full life cycle CO2e footprint is given. Assuming an ideal use of the unit during the life cycle (typically 4 years), we find the CO2e footprint per hour of computation:

F = 8640 kgCO2e / (12 cores * 365d * 24h * 4y) ~ 20gCO2e/hour

For a full CTC test, the CO2e footprint F_ctc corresponds to:

F_ctc = 22000h/CTC * 20 gCO2e/h ~ 450 kgCO2e/CTC

### Impact of the datacenter local on the Energy intensity

As a big part of the CO2 emission is given by the energy used during computation, the CO2 emission figure estimated above actually greatly depends on the energy intensity at the location of the datacenter.

As an example, taking 2 locations in Canada [4], we found on the same day in Canada an energy intensity of 20g/kWh in Ontario, while Alberta was having an energy intensity of 550g/kWh. Please note that these figures don’t necessarily include the full life cycle of energy production as the methodology also varies per country. Energy intensity can even be greater than 1000g/kWh at some locations, depending on the period.

## Real life estimate

As above figures are summing up many different factors without giving any details, we also present the result of a French study in [5] including more details.

The study has been conducted on a full datacenter, including all factors which gives a more realistic figure. For example, compared to the previous estimate, it takes into account:

- the network infrastructure

- the storage infrastructure

- the maintenance (including footprint of workers in the datacenter during working hours),

- the average charge of the datacenter

- other hardware used during the datacenter life

- the datacenter building (amortized over many years)

etc.

In this study, assuming an energy intensity of ~100 gCO2e/kWh, the study finds a footprint of an hour of computation of ~5.8 gCO2e/h for a life cycle of 4 years. The study also reports a footprint of ~20 gCO2e/h when using an energy intensity of ~700 gCO2e/kWh.

The study also shows that the direct electricity consumption for computation only represents ~40% of the total CO2e emissions.

## Estimate per contribution

Using a very rough range estimate for the emission per hour of computation of [2.5-25] gCO2e/h (including factors as energy intensity, life cycle policy, efficiency etc.), the following estimate per contribution can be computed:

F_ctc = 22000h/CTC * [2.5-25] gCO2e/h ~ [55-550] kgCO2e/CTC

Depending on the JVET contribution type, it can be assumed that the number of full runs of CTC per contribution is roughly in the range [1-25]. It gives a rough range of [55-13000] kgCO2e/contribution.

As a reference, the average CO2e footprint per person worldwide is ~4000 kgCO2e/person (with great variation per country as reported in [6]), or a roundtrip flight between Europe to and US West Coast or East Asia is ~3000 kgCO2e/person.

## Estimate per year

Depending on the JVET period, we estimate a rough number of [500-4000] contributions.

Some contributions 	are purely informative, while some contributions (like the ones in AhG13) runs many CTC.

# Proposed new process

Assuming a conversative lower bound in the range of CO2e footprint for a given year a JVET contribution, it may be justified to improve the global footprint of the JVET group.

In order to keep a reliable evaluation of the JVET technical contributions, we propose to split the CTC in two categories: CTC for the pre-adoption stage contributions, CTC for the formal EE stage. The CTC for the pre-adoption stage contributions impacts only contributions not in EE/CE, i.e., before the final adoption in a reference software or standard. Contributions in the final stage still use the current CTC in order to keep the same reference as before, however, contributions in early stages mandatorily use a less energy intensive CTC in order to reduce the CO2e footprint of the group as a whole.

It should be emphasized that such contributions must NOT disclose results on the whole CTC, as it would defeats the purpose of the CO2e footprint reduction.

## Reduced CTC

We proposed to reduce the CTC as depicted in the table below.

|  | Original | Half |
| --- | --- | --- |
| AI | 75 | 37 |
|  | 63 | 32 |
|  | 38 | 19 |
|  | 37 | 18 |
| RA | 600 | 288 |
|  | 500 | 256 |
|  | 300 | 160 |
|  | 294 | 160 |
| LDB | 250 | 125 |
|  | 150 | 75 |
|  | 100 | 50 |

For AI the number of original frames divided by 2 and divided by the temporal subsampling 8 and rounded to the nearest integer.

For LDB, the number of frames divided by 2 and rounded to the nearest integer.

For RA, the number of frames divided by 2, rounded to the nearest integer with a modulo of GOP size 32.

## Impact of reduced CTC

Here we present the results by comparing full CTC and half CTC. Note that the results of half CTC have been extracted from the full results log. While BDrate is accurate, timing might be a bit approximative since they are recomputed using the sum of all frames timing.

### Between 2 ECM – ECM 2.0 vs ECM 3.1

Detailed results are available in attached files.

#### AI

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -1.75% | -4.82% | 102% | 88% |
| Class A2 | 0.00% | -3.85% | -5.60% | 100% | 92% |
| Class B | -0.14% | -4.23% | -3.41% | 104% | 112% |
| Class C | -0.35% | -1.58% | -1.64% | 95% | 74% |
| Class E | -0.37% | -2.03% | -3.62% | 97% | 74% |
| Overall | -0.22% | -2.80% | -3.65% | 100% | 89% |
| Class D | -0.16% | -1.51% | -1.71% | 92% | 88% |
| Class F | -0.33% | -2.64% | -1.84% | 97% | 95% |
| Class TGM | -0.15% | -1.49% | -1.13% | 100% | 90% |

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 2.0 half |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -1.42% | -4.86% | 105% | 88% |
| Class A2 | -0.01% | -3.63% | -5.76% | 100% | 92% |
| Class B | -0.16% | -3.73% | -2.76% | 101% | 112% |
| Class C | -0.32% | -1.57% | -1.65% | 96% | 75% |
| Class E | -0.38% | -2.01% | -3.53% | 97% | 75% |
| Overall | -0.22% | -2.56% | -3.49% | 100% | 89% |
| Class D | -0.18% | -1.57% | -1.87% | 92% | 87% |
| Class F | -0.29% | -2.41% | -1.57% | 97% | 95% |
| Class TGM | -0.17% | -1.67% | -1.16% | 102% | 90% |

|  | Y | U | V |
| --- | --- | --- | --- |
| MAE | 0.04% | 0.29% | 0.30% |
| STDEV ABS | 0.05% | 0.33% | 0.40% |
| MAX | 0.25% | 1.47% | 2.12% |

The table above report the average absolute difference, standard deviation of the absolute difference and maximum difference between the full and the half tests.

In the case of AI configuration, the largest difference is for the class F “SlideShow” (-0.50% vs -0.26%).

#### RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.65% | -0.71% | -2.11% | 105% | 82% |
| Class A2 | -1.70% | -3.79% | -4.82% | 114% | 78% |
| Class B | -1.23% | -3.10% | -2.36% | 111% | 88% |
| Class C | -1.13% | -1.05% | -0.67% | 111% | 85% |
| Class E |  |  |  |  |  |
| Overall | -1.38% | -2.21% | -2.35% | 111% | 84% |
| Class D | -0.88% | -1.65% | -1.48% | 106% | 81% |
| Class F | -0.39% | -1.54% | -1.03% | 112% | 93% |
| Class TGM | -0.28% | -1.19% | -0.97% | 112% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 2.0 half |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.61% | 0.04% | -1.86% | 105% | 85% |
| Class A2 | -1.73% | -3.74% | -4.83% | 116% | 78% |
| Class B | -1.37% | -2.87% | -2.01% | 114% | 88% |
| Class C | -1.03% | -1.15% | -0.52% | 111% | 86% |
| Class E |  |  |  |  |  |
| Overall | -1.40% | -2.00% | -2.15% | 112% | 85% |
| Class D | -0.93% | -1.52% | -1.20% | 107% | 80% |
| Class F | -0.40% | -1.44% | -0.98% | 112% | 93% |
| Class TGM | -0.26% | -1.29% | -1.01% | 113% | 104% |

|  | Y | U | V |
| --- | --- | --- | --- |
| MAE | 0.11% | 0.37% | 0.31% |
| STDEV ABS | 0.15% | 0.42% | 0.36% |
| MAX | 0.77% | 1.98% | 1.60% |

In the case of RA configuration, the largest relative error is for the class B “MarketPlace” (-1.78% vs -2.54%).

#### LDB

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 2.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.17% | -5.17% | -4.97% | 104% | 106% |
| Class C | -0.80% | -3.35% | -2.86% | 100% | 114% |
| Class E | -1.74% | -4.05% | -5.82% | 122% | 109% |
| Overall | -1.60% | -4.28% | -4.48% | 107% | 109% |
| Class D | -0.15% | -3.39% | -2.95% | 89% | 98% |
| Class F | -0.56% | -2.63% | -2.22% | 106% | 105% |
| Class TGM | -0.35% | -1.72% | -1.30% | 105% | 103% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 2.0 half |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.08% | -5.00% | -5.11% | 106% | 106% |
| Class C | -0.65% | -3.06% | -2.87% | 96% | 114% |
| Class E | -1.71% | -3.74% | -5.87% | 118% | 110% |
| Overall | -1.51% | -4.04% | -4.56% | 105% | 109% |
| Class D | -0.09% | -2.87% | -2.96% | 90% | 98% |
| Class F | -0.42% | -1.89% | -1.84% | 103% | 107% |
| Class TGM | -0.30% | -1.62% | -1.40% | 108% | 102% |

|  | Y | U | V |
| --- | --- | --- | --- |
| MAE | 0.14% | 0.48% | 0.35% |
| STDEV ABS | 0.14% | 0.53% | 0.34% |
| MAX | 0.53% | 2.18% | 1.33% |

In the case of LDB, the largest relative error is for the class F “SlideShow” (-1.98% vs -0.45%).

In the three configurations, we observed that the global trends and average of the gains and complexity are not significatively changed.

### For a single tool

We also present the impact of the reduced CTC on a single tool (here the EE1 test 1.1a on UBT). Detailed results are available in attached files.

#### AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | 0.00% | -0.08% | 183% | 96% |
| Class A2 | -0.39% | -0.14% | -0.08% | 180% | 94% |
| Class B | -0.42% | -0.17% | -0.21% | 182% | 92% |
| Class C | -0.18% | -0.10% | -0.10% | 166% | 96% |
| Class E | -0.28% | -0.14% | -0.13% | 170% | 91% |
| Overall | -0.33% | -0.12% | -0.13% | 176% | 94% |
| Class D | -0.08% | 0.09% | -0.05% | 169% | 109% |

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 half |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.04% | -0.13% | 169% | 95% |
| Class A2 | -0.38% | -0.21% | -0.12% | 181% | 94% |
| Class B | -0.46% | -0.20% | -0.26% | 184% | 92% |
| Class C | -0.18% | -0.17% | 0.02% | 166% | 96% |
| Class E | -0.27% | -0.21% | -0.11% | 177% | 91% |
| Overall | -0.33% | -0.17% | -0.13% | 176% | 94% |
| Class D | -0.07% | 0.11% | 0.00% | 168% | 110% |

|  | Y | U | V |
| --- | --- | --- | --- |
| MAE | 0.03% | 0.10% | 0.11% |
| STDEV ABS | 0.03% | 0.07% | 0.08% |
| MAX | 0.14% | 0.22% | 0.33% |

The table above report the average absolute difference, standard deviation of the absolute difference and maximum difference between the full and the half tests.

In the case of AI configuration, the largest difference is for the class A1 “FoodMarket4” (-0.50% vs -0.26%).

#### RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.86% | -0.99% | -1.23% | 124% | 96% |
| Class A2 | -0.72% | -0.63% | -0.79% | 131% | 92% |
| Class B | -0.71% | -0.95% | -0.91% | 131% | 93% |
| Class C | -0.48% | -0.76% | -0.64% | 144% | 93% |
| Class E |  |  |  |  |  |
| Overall | -0.68% | -0.84% | -0.88% | 133% | 93% |
| Class D | -0.36% | -0.34% | -0.50% | 154% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 half |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.81% | -1.02% | -1.29% | 123% | 96% |
| Class A2 | -0.68% | -0.66% | -0.79% | 132% | 92% |
| Class B | -0.76% | -0.93% | -0.86% | 130% | 93% |
| Class C | -0.45% | -0.58% | -0.70% | 145% | 93% |
| Class E |  |  |  |  |  |
| Overall | -0.67% | -0.80% | -0.89% | 133% | 93% |
| Class D | -0.35% | -0.59% | -0.68% | 153% | 98% |

|  | Y | U | V |
| --- | --- | --- | --- |
| MAE | 0.05% | 0.20% | 0.14% |
| STDEV ABS | 0.05% | 0.25% | 0.14% |
| MAX | 0.15% | 1.07% | 0.46% |

In the case of RA configuration, the largest relative error is for the class B “MarketPlace” (-0.75% vs -0.90%).

# Proposals

Adopt the new process for current CTC:

- No changes for EE/CE contributions, i.e., contributions in the last stage before potential adoption.

- Adopt reduced CTC for any first contributions, i.e., contributions not in the last stage before potential adoption. Emphasis on the rule that such contribution MUST NOT embed full CTC results.

# References

[1] https://i.dell.com/sites/csdocuments/CorpComm_Docs/en/carbon-footprint-poweredge-r740.pdf

[2] https://www.iso.org/standard/66453.html

[3] https://corporate.delltechnologies.com/en-us/social-impact/advancing-sustainability/sustainable-products-and-services/product-carbon-footprints.htm#tab0=3

[4] https://app.electricitymap.org/map

[5] https://gricad.univ-grenoble-alpes.fr/documents/GES-h-coeur-GRICAD-2020.pdf

[6] https://data.worldbank.org/indicator/EN.ATM.CO2E.PC

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.

Acknowledgment

This work has been partly achieved in the context of the 3EMS project funded by the “Region Bretagne”.