| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0109-v2 |
| --- | --- |

| Title: | AhG12: Neural Network-based intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thierry Dumas Franck Galpin, Philippe Bordes, Ferdinand Mom, Edouard François | Email: | first.last@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes a new intra prediction mode based on neural network algorithm in ECM. It is based on the algorithm already tested in EE1 and reported in JVET-X0118 but contains algorithmic improvements for further complexity reduction. Following results are reported:

- RA:  -0.71%, -0.16% and -0.23% BD-rate PSNR Y, U and V gains, respectively for a complexity of 104% and 111% at encoder and decoder,

- AI:   -1.77%, -1.33% and -1.13% BD-rate PSNR Y, U and V gains, respectively, for a complexity of 136% and 330% at encoder and decoder,

It is proposed to include this tool in the next ECM version.

# Introduction

A single neural network based intra prediction has been studied in AhG11 for several meeting cycles. The latest related contribution JVET-X0118 proposed a reduced complexity version of the algorithm on top of VTM-11-NNVC. The same algorithm has been implemented and evaluated on top of ECM-3.1, as reported in EE1 3.1.2 (JVET-Y0082). This contribution proposes to consider inclusion of this tool in the next ECM version.

# Description

The neural network based intra prediction (NNIP) algorithm and models are the same as the ones described in EE1 3.1.2:

- SADL for inference

- 16-bit integer for model weights and intermediate results, using 32-bit integer for internal computation

- 7 models depending on the block size

- sparse weights enforced at training time for complexity reduction.

Compared to EE1 3.1.2, the following changes were made:

- each model has been further simplified, resulting in sparser matrices.

- inference algorithm in SADL has been improved: a simple sparse matrix storage (compressed row method) and sparse matrix multiplication algorithm have been implemented, exploiting the matrices sparsity.

Table 1 shows the parameters count and storage memory needed for each model. Memory usage reported is informative since it depends on the implementation for sparse matrix storage. Table 1 shows the total memory storage of the actual tested ECM implementation.

Table 1. Parameters count and storage memory needed for each model.

| Model | Parameter’s count | Nonzero parameters count | Memory footprint |
| --- | --- | --- | --- |
| 4x4 4x8 4x16 4x32 8x8 8x16 16x16 | 1679k 1737k 1854k 2088k 2030k 2263k 4054k | 99k 111k 134k 181k 143k 190k 350k | 387 kB 433 kB 524 kB 706 kB 558 kB 741 kB 1369 kB |

The complexity measured in MAC/pixel for the different neural network models is reported in Table 2. The bottom part of the table shows the complexity for blocks predicted with down-sampled reference samples.

Table 2. Complexity measured in MAC/pixel for the different neural network models.

| Model | Complexity (MAC/pixel) |
| --- | --- |
| 4x4 4x8 4x16 4x32 8x8 8x16 16x16 | 6.2k 3.5k 2.1k 1.4k 2.2k 1.5k 1.4k |
| 8x32 | 0.74k |
| 16x32 | 0.68k |
| 32x32 | 0.34k |
| 64x64 | 0.08k |
| 128x128 | 0.02k |

# Results

The proposed algorithm has been tested on top of ECM-3.1 using AhG12 CTC.

## AI configuration

Table 3. BD-rate results for AI configuration.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.02% | -1.36% | -1.26% | 138% | 279% |
| Class A2 | -1.39% | -0.94% | -1.02% | 139% | 323% |
| Class B | -1.57% | -1.07% | -1.01% | 136% | 330% |
| Class C | -1.62% | -1.35% | -1.17% | 134% | 371% |
| Class E | -2.44% | -2.09% | -1.26% | 137% | 344% |
| Overall | -1.77% | -1.33% | -1.13% | 136% | 330% |
| Class D | -1.61% | -1.20% | -1.43% | 131% | 399% |
| Class F | -1.05% | -0.61% | -0.63% | 122% | 205% |

## RA configuration

Table 4. BD-rate results for RA configuration.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM 3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.92% | -0.28% | -0.49% | 103% | 109% |
| Class A2 | -0.53% | 0.07% | -0.25% | 107% | 107% |
| Class B | -0.70% | 0.01% | -0.08% | 102% | 108% |
| Class C | -0.70% | -0.45% | -0.19% | 105% | 120% |
| Class E |  |  |  |  |  |
| Overall | -0.71% | -0.16% | -0.23% | 104% | 111% |
| Class D | -0.67% | -0.27% | -0.70% | 110% | 133% |
| Class F | -0.38% | -0.17% | -0.37% | 108% | 115% |

# Conclusion

A neural network-based new intra prediction mode with reduced complexity is proposed. The tool brings 0.71% PSNR-Y BD-rate gains in RA configuration respectively with encoding and decoding runtime variations 104% and 111% respectively.

In AI configuration, 1.77% PSNR-Y BD-rate gains are achieved with encoding and decoding runtime variations 136% and 330% respectively.

It is proposed to adopt the tool in ECM reference software.

Thank you to ByteDance for crosschecking the contribution (JVET-Y0211).

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).