| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0110-v3 |
| --- | --- |

| Title: | AHG11: Small Ad-hoc Deep-Learning Library update |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Franck Galpin Ferdinand Mom Thierry Dumas Philippe Bordes Pavel Nikitin Edouard François | Tel: Email: | franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents an update on the SADL library already described in JVET-W0181.

# Changes

- Change the input format of the converter to ONNX [1] for more flexible and universal support

- Added counters to assess accurately MAC/pix for a given model,

- Minor fixes

The table below summarize the framework characteristics.

| Language | Pure C++, header only. |
| --- | --- |
| Footprint | ~3400 LOC, library ~200kB, no dependency |
| Optimization | Some SIMD at hot spots (best effort) |
| Compatibility | Onnx to sadl converter |
| Layer Supports | constants, add, maxPool, matMul, reshape, ReLU,conv2D, mul, concat, max, leakyReLU |
| Type support | float, int32, int16, int8 |
| Quantization | Support adaptive quantizer per layer |
| License | BSD 3-Clause |

# Usage

## Model Conversion

As there are many different formats in each available framework with various constraints, the preferred method for conversion is to first convert to the ONNX format (currently tested up to opset 13) and then convert the onnx model to a sadl model.

import tensorflow as tf

import tf2onnx

import onnx

…

model_onnx, _ = tf2onnx.convert.from_keras(model,inputs, opset=13)

onnx.save(model_onnx,”mymodel.onnx”)

From Tensorflow

import torch

import onnx

…

torch.onnx.export(model,inputs, “mymodel.onnx”)

From PyTorch

### Convert to SADL

converter/main.py –input_onnx mymodel.onnx –output model.sadl

- converter/main.py –input_onnx mymodel.onnx –output model.sadl

If the conversion to onnx is not possible, it is likely the model contains “exotic” layers (hook, custom layer, scripted layers etc.) which need to be explicitly supported by adding code to the library.

## Counters

Building your binary with the following options:

#define DEBUG_VALUES            1 // show values

/#define DEBUG_MODEL             1 // show pb with model

/#define DEBUG_COUNTERS      1 // print overflow etc.

[INFO] 108824 MACs

[INFO] 99096 MACs non 0

[INFO] MAC/nb output for output 0, 16 elts

[INFO] 6801.5 MAC/elt

[INFO] 6193.5 MACs non 0/elt

Allows to output the number of MAC and MAC/pix:

# Proposal

Using a common format for conversion would make the maintenance of the library easier and the conversion easier for proponents. It is proposed to update the current version with the new version.

# Reference

[1] ONNX https://onnx.ai/

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.