| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0115-v1 |
| --- | --- |

| Title: | AHG9: On post-filter SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Maria Santamaria, Francesco Cricri, Emre B. Aksu, Hamed. R. Tavakoli Nokia Technologies Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes an NNR post-filter SEI message into a working draft of a VSEI amendment. The NNR post-filter SEI message may carry an ISO/IEC 15938-17 bitstream that specifies a neural-network-based post-filter. The contribution is a follow-up of JVET-X0112.

# Introduction

## Updates relative to JVET-X0112 [1]

JVET-X0112 proposed two SEI messages:

- An NNR post-filter SEI message for the carriage of an MPEG Neural Network Representation (NNR, ISO/IEC 15938-17) bitstream that specifies a neural-network-based post-filter.

- A post-filter purpose SEI message that currently specifies quality enhancement and super resolution as purposes and allows other purposes to be specified in the future.

The following bullet list copies the comments on JVET-X0112 from the JVET meeting minutes and summarizes how this contribution responds to the comments.

- Is it really necessary to have two separate messages? The PfP message does not contain much info, and likely both would be always updated together. A single SEI message might be more appropriate.

Response: The post-filter purpose (PfP) SEI message is no longer proposed in this contribution. The purpose indicator is included in the NNR post-filter SEI message.

- The purpose of PfP is not fully clear: Where would the actual method e.g. of super resolution be defined, and is it at all necessary that the encoder describes it? The resolution after upsampling might finally be display specific, so is it necessary to signal it at all? (it is answered that e.g. the NNR post filter could be used for that purpose)

Response: In this contribution, only a single purpose value (visual quality improvement) is proposed, whereas the super-resolution purpose is no longer proposed in this contribution. A new value for the purpose indicator for super-resolution can be specified in the future. Conditioned by this value, syntax elements related to super resolution can also be added in the future, if needed.

- Is the NNR specification complete e.g. in describing input/output ordering w.r.t. the video decoder output? Could it e.g. describe a 4:2:0 input and 4:4:4 output?

Response: An input/output ordering indicator is included in the NNR post-filter SEI message proposed in this contribution. A single value of the indicator is specified, and its use requires the 4:2:0 chroma format. The input and output ordering are specified with pseudo code in a manner that suits the post-filter tested in JVET-Y0059 [8]. Other values of the input/output ordering indicator can be specified in the future.

- How would block-wise processing, block-overlapping, and boundary processing be managed?

Response: The block-wise processing, block overlapping, and boundary processing are specified together with input and output ordering with pseudo code. Sample locations that would be outside the boundaries of the cropped decoded output picture are clipped to be within the boundaries.

- What could be the maximum amount of payload data in NNR? It is also asked if referring to a relative complex standard is the purpose of a SEI message? For the currently foreseen purposes, it might not be updated frequently. There could also be other systems-related mechanisms.

Response: SEI messages do not have a size limit. In the experiments by the proponent, the NNR-coded data has been up to 2134 bytes.

- What does URI mean, and what does it refer to?

Response: A cross-reference to URI has been added (IETF Internet Standard 66).

- Extensibility aspect might require more consideration. In the past, extensibility was not often used, even if it had been foreseen in an SEI message.

Response: This contribution suggests that new values of the purpose indicator and an input/output ordering indicator can be specified in the future.

## Related contributions

JVET-T0076 [2] proposed enabling the usage of ALF as a post-filter and presented additional sharpening and bokeh effect as use cases. The contribution pointed out that ALF post-filtering enables selective region-wise sharpening or blurring.

JVET-T0092 [3] proposed neural network (NN) based super resolution post-filtering and presented a concept to convey the NN parameters in an SEI message. MPEG NNR, ONNX (Open Neural Network eXchange), and NNEF (Neural Network Exchange Format) were mentioned as potential formats for the NN parameters in the SEI message.

JVET-U0091 [4] presented syntax and semantics for a neural-network-based post-filtering SEI message, which included the following aspects:

- Purpose for the post-filtering

| nn_postfilter_type_idc | Post filtering type |
| --- | --- |
| 0 | Visual quality improvement with single input picture |
| 1 | Visual quality improvement with multiple input pictures |
| 2 | Super resolution with single input picture |
| 3 | Super resolution with multiple input pictures |
| 4..15 | Reserved |

- Neural network topology and parameter storage formats, e.g. NNEF or ONNX

- Either a URI identifying the neural network topology or a coded representation of the topology

- Either a URI identifying the neural network parameter values or a coded representation of the parameter values

JVET-V0058 [5] proposed a post-filter nesting SEI message, which contains inter-related SEI messages jointly specifying a post-filtering operation. The contribution envisioned that the nested SEI messages could include the following:

- An SEI message specifying the purpose for filtering, such as quality improvement or super-resolution, and parameters specific to the purpose, such as an applicable scaling ratio range for a super-resolution filter.

- An SEI message specifying the filter itself. Such an SEI message could for example contain or reference (through a URI) an MPEG Neural Network Representation (NNR, ISO/IEC 15938-17) bitstream.

JVET-V0058 was discussed in a joint meeting during which the following guidance was noted (quote from the meeting minutes):

It was suggested to continue with the intent to not significantly extend existing SEI messages (at least if there was not a clear plan to do so that was established in advance, and that the existence of a reserved value is not sufficient to indicate such a plan). A NOTE in the text to describe future plans could be an approach to consider for new SEI messages.

JVET-W0063 [6] proposed a NN-based super-resolution post-filter. Two sets of results were provided, where test 1 used constant neural network parameters and test 2 used content-adaptive neural network parameters per each sequence and QP that were coded with the NCTM of MPEG NNR. Test 1 was shown to provide an average BD-rate reduction over the use of VVC RPR for upsampling. Test 2 was shown to provide a consistent BD-rate reduction compared to test 1. An SEI message or systems layer signaling was proposed to convey the network parameters to the decoder.

JVET-X0111-v1 [7] proposes a content-adaptive NN-based quality enhancement post-filter where the NN parameters are coded with MPEG NNR. The results are compared with the same NN parameters coded with Lempel-Ziv-Markov chain algorithm 2 (LZMA2). A consistent BD-rate reduction with the MPEG NNR coding compared to LZMA2 coding is reported in all tested classes (B, C, D, F).

JVET-Y0059 [8] applies the content-adaptation technique proposed in JVET-X0111-v1 [7] to the in-loop filter proposed in JVET-W0131 [9] when used as a post-filter. The contribution shows that content-adaptation provides consistent coding gains also for a very different neural network architecture than the one in JVET-X0111-v1 [7].

## Observations

We have the following observations based on the related contributions:

- Post-filtering can be performed for multiple purposes, including but not limited to those mentioned in JVET-U0091 and JVET-T0076.

- Content-adaptive NN-based post-processing for quality enhancement and super-resolution has been shown to provide a BD-rate reduction compared to pre-trained NN-based post-processing with the same neural network topology.

- Coding neural network parameters with MPEG NNR has been shown to be a viable approach for content-adaptive NN-based post-processing for quality enhancement and super-resolution.

# Proposal

## Overview

This contribution proposes an NNR post-filter SEI message into a working draft of a VSEI amendment. The NNR post-filter may carry an ISO/IEC 15938-17 bitstream that specifies a neural-network-based post-filter. The contribution is a follow-up of JVET-X0112.

## NNR post-filter SEI message

| nnr_post_filter( payloadSize ) { | Descriptor |
| --- | --- |
| nnrpf_id | u(21) |
| nnrpf_mode_idc | u(2) |
| if( nnrpf_mode_idc > 0 ) { |  |
| nnrpf_persistence_flag | u(1) |
| if( nnrpf_mode_idc = = 1 ¦ ¦ nnrpf_mode_idc = = 2 ) { |  |
| nnrpf_purpose | ue(v) |
| nnrpf_io_idc | ue(v) |
| if( nnr_io_idc = = 0 ) { |  |
| nnrpf_patch_size_minus1 | ue(v) |
| nnrpf_overlap | ue(v) |
| } |  |
| while( !byte_aligned( ) ) |  |
| nnrpf_reserved_zero_bit | u(1) |
| } |  |
| if( nnrpf_mode_idc = = 1 ) { |  |
| i = 0 |  |
| do |  |
| nnrpf_uri[ i ] | b(8) |
| while( nnrpf_uri[ i++ ] != 0 ) |  |
| } |  |
| if( nnrpf_mode_idc = = 2 ¦ ¦ nnrpf_mode_idc = = 3 ) { |  |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnrpf_payload_byte[ i ] | b(8) |
| } |  |
| } |  |
| } |  |

This SEI message either specifies a neural network that may be used as a post-processing filter or cancels the persistence of the previous post-processing filter with the same identifier value (i.e., the same value of nnrpf_id).

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	Luma sample array CroppedYPic[ y ][ x ] and chroma sample arrays CroppedCbPic[ y ][ x ] and CroppedCrPic[ y ][ x ], when present, of the cropped decoded output picture for vertical coordinates y and horizontal coordinates x, where the top-left corner of the sample array has coordinates y equal to 0 and x equal to 0.

When this SEI message specifies a neural network that may be used as a post-processing filter, the semantics specify the derivation of the luma sample array FilteredYPic[ y ][ x ] and chroma sample arrays FilteredCbPic[ y ][ x ] and FilteredCrPic[ y ][ x ], when present, that contain the output of the post-processing filter.

nnrpf_id contains an identifying number that may be used to identify a post-processing filter.

Values of nnrpf_id from 0 to 220 − 1, inclusive, may be used as determined by the application. Values of nnrpf_id from 220 to 221 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC.

nnrpf_mode_idc equal to 0 indicates that no post-processing filter associated with the nnrpf_id value is specified in this SEI message and that the persistence of the previous post-processing filter with the same value of nnrpf_id is cancelled. nnrpf_mode_idc equal to 1 specifies that the post-processing filter associated with the nnrpf_id value is a neural network identified by a given Uniform Resource Identifier (URI), with syntax and semantics as specified in IETF Internet Standard 66. nnrpf_mode_idc equal to 2 specifies that the post-processing filter associated with the nnrpf_id value is a neural network represented by the ISO/IEC 15938-17 bitstream contained in this SEI message. nnrpf_mode_idc equal to 3 specifies that the post-processing filter is the neural network identified by the previous NNR post-filter SEI message, in decoding order, with nnrpf_mode_idc equal to 1 or 2 and updated by the ISO/IEC 15938-17 bitstream contained in this SEI message.

nnrpf_persistence_flag specifies the persistence of this SEI message for the current layer.

nnrpf_persistence_flag equal to 0 specifies that this SEI message applies to the current decoded picture only.

nnrpf_persistence_flag equal to 1 specifies that this SEI message applies to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	This SEI message has nnrpf_mode_idc equal to 1 or 2 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and nnrpf_mode_idc value equal to 0, 1, or 2 is output.

–	This SEI message has nnrpf_mode_idc equal to 3 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnrpf_id value as that in this SEI message and any nnrpf_mode_idc value is output.

nnrpf_purpose indicates the purpose of post-processing filter as specified in Table XYZ. The value of nnrpf_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of nnrpf_purpose that do not appear in Table XYZ are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnrpf_purpose.

Table XYZ – Definition of nnrpf_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Visual quality improvement |

NOTE – When a reserved value of nnrpf_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnrpf_purpose being equal to that value.

nnrpf_io_idc indicates the method of forming the input to the post-processing filter from the sample arrays of a cropped decoded output picture and the format of the output from the post-processing filter. The semantics of nnrpf_io_idc equal to 0 are specified below. When the chroma format of the cropped decoded output picture is not 4:2:0, nnrpf_io_idc shall not be equal to 0. The value of nnrpf_io_idc shall be in the range of 0 to 31, inclusive. Values of nnrpf_io_idc greater than 0 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnrpf_io_idc.

nnrpf_patch_size_minus1 + 1 specifies the horizontal and vertical sample counts of each of the six output channels of the post-processing filter. The value of nnrpf_patch_size_minus1 shall be in the range of 0 to 32766, inclusive.

nnrpf_overlap * 2 + nnrpf_patch_size_minus1 + 1 specifies the horizontal and vertical sample counts of each of the six input tensors of the post-processing filter. The value of nnrpf_overlap shall be in the range of 0 to 16383, inclusive.

Let patchSize be equal to nnrpf_patch_size_minus1 + 1 and tensorSize be equal to patchSize + nnrpf_overlap * 2.

When nnrpf_io_idc is equal to 0, the input tensors inputTensor[ idxInp ][ yInp ][ xInp ] are derived and filtered with the post-processing filter postProcessingFilter( ) to generate outputTensor[ idxOut ][ yOut ][ xOut ], where idxInp is a channel index in the range of 0 to 9, inclusive, yInp and xInp are in the range of 0 to tensorSize − 1, inclusive, idxOut is a channel index in the range of 0 to 5, and yOut and xOut are in the range of 0 to patchSize − 1, inclusive:

for( cTop = 0; cTop < PicHeightInLumaSamples; cTop += patchSize * 2 ) {
	for( cLeft = 0; cLeft < PicWidthInLumaSamples; cLeft += patchSize * 2 ) {
		for ( yP = –nnrpf_overlap; yP < patchSize + nnprf_overlap; yP++) {
			for ( xP = –nnrpf_overlap; xP < patchSize + nnprf_overlap; xP++ ) {
				yTopLeft = Clip3( 0, PicHeightInLumaSamples – 1, cTop + yP * 2 )
				xTopLeft = Clip3( 0, PicWidthtInLumaSamples – 1, cLeft + xP * 2 )
				yBottomRight = Clip3( 0, PicHeightInLumaSamples – 1,  yTopLeft + 1 )
				xBottomRight = Clip3( 0, PicWidthtInLumaSamples – 1, xTopLeft + 1 )
				inputTensor[ 0 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] = CroppedYPic[ yTopLeft ][ xTopLeft ]
				inputTensor[ 1 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] = CroppedYPic[ yTopLeft ][ xBottomRight ]
				inputTensor[ 2 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] = CroppedYPic[ yBottomRight ][ xTopLeft ]
				inputTensor[ 3 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] = CroppedYPic[ yBottomRight ][ xBottomRight ]
				y = Clip3( 0, PicHeightInLumaSamples / 2 – 1, cTop / 2 )
				x = Clip3( 0, PicWidthtInLumaSamples / 2 – 1, cLeft / 2 )
				inputTensor[ 4 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] = CroppedCbPic[ y ][ x ]
				inputTensor[ 5 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] = CroppedCrPic[ y ][ x ]
				inputTensor[ 6 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] = 2(SliceQPY – 42)/6	
				inputTensor[ 7 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] =  Clip3( 0, ( 1 << BitDepthY ) – 1, 
					bS[ yTopLeft ][ xTopLeft ] << ( BitDepthY – 1 ) )
				inputTensor[ 8 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] =  Clip3( 0, ( 1 << BitDepthC ) – 1, 
					bS[ y ][ x ] << ( BitDepthC – 1 ) ) 
				inputTensor[ 9 ][ yP + nnrpf_overlap ][ xP + nnrpf_overlap ] =  Clip3( 0, ( 1 << BitDepthC ) – 1, 
					bS[ y ][ x ] << ( BitDepthC – 1 ) ) 
			}
		}
		
		outputTensor = postProcessingFilter( inputTensor )

		for( yP = 0, ySrc = cTop / 2 + yP; yP < patchSize; yP++, ySrc++ )
			for( xP = 0, xSrc = cLeft / 2 + xP; xP < patchSize; xP++, xSrc++ ) {
				FilteredYPic[ ySrc * 2 ][ xSrc * 2 ] = outputTensor[ 0 ][ yP ][ xP ]
				FilteredYPic[ ySrc * 2 ][ xSrc * 2 + 1 ] = outputTensor [ 1 ][ yP ][ xP ]
				FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 ] = outputTensor[ 2 ][ yP ][ xP ]
				FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 + 1] = outputTensor[ 3 ][ yP ][ xP ]
				FilteredCbPic[ ySrc ][ xSrc ] = outputTensor[ 4 ][ yP ][ xP ]
				FilteredCrPic[ ySrc ][ xSrc ] = outputTensor[ 5 ][ yP ][ xP ]
			}
		}
	}
}

The neural network that may be used as a post-processing filter for a cropped decoded output picture is the filter that i) is identified by the NNR post-filter SEI message that has a particular nnrpf_id value, has nnrpf_mode_idc equal to 1 or 2, and pertains to the picture and ii) is updated by the NNR post-filter SEI message, if any, that has the same nnrpf_id value, has nnrpf_mode_idc equal to 3, and pertains to the picture.

nnrpf_reserved_zero_bit shall be equal to 0.

nnrpf_uri[ i ] contains the i-th byte of a NULL-terminated UTF-8 character string, as specified in IETF Internet Standard 63. The UTF-8 character string contains a URI, with syntax and semantics as specified in IETF Internet Standard 66, identifying the neural network used as the post-processing filter.

nnrpf_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnrpf_payload_byte[ i ] for all present values of i shall be a complete bitstream that conforms to ISO/IEC 15938-17.

# References

- Miska M. Hannuksela, Emre B. Aksu, Francesco Cricri, Hamed R. Tavakoli, and Maria Santamaria, "AHG9: On post-filter SEI", JVET-X0112, Oct. 2021.

- Han Boon Teo, Hai Wei Sun, Chong Soon Lim, "AHG9: ALF control SEI message", JVET-T0076, Oct. 2020.

- Takeshi Chujoh, Eiich Sasaki, and Tomohiro Ikai, "AHG9/AHG11: Neural network based super resolution SEI", JVET-T0092, Oct. 2020.

- Byeongdoo Choi, Zeqiang Li, Wei Wang, Wei Jiang, Xiaozhong Xu, Stephan Wenger, Shan Liu, "AHG9/AHG11: SEI message for carriage of neural network information for post filtering", JVET-U0091, Jan. 2021.

- Miska M. Hannuksela, Emre B. Aksu, Francesco Cricri, and Hamed R. Tavakoli, "AHG9: On post-filter SEI", JVET-V0058, Apr. 2021.

- Takeshi Chujoh, Yukinobu Yasugi, and Tomohiro Ikai, "EE1.2.2: NN-based super resolution (JVET-V0073)", JVET-W0063, Jul. 2021.

- Maria Santamaria, Jani Lainema, Francesco Cricri, Ramin G. Youvalari, Honglei Zhang, Alireza Zare, Goutham Rangu, Hamed R. Tavakoli, Homayun Afrabandpey, Miska Hannuksela, "AHG11: MPEG NNR compressed bias update for the CNN based post-filter of EE1-1.1", JVET-X0111, Oct. 2021.

- Maria Santamaria, Jani Lainema, Francesco Cricri, Ramin G. Youvalari, Honglei Zhang, Goutham Rangu, Hamed R. Tavakoli, Homayun Afrabandpey, Miska Hannuksela, "AHG11: Content-adaptive post-processing filter", JVET-Y0059, Jan. 2021.

- Hongtao Wang, Jianle Chen, Kevin Reuze, Anand Meher Kotra, Marta Karczewicz, “EE1-related: Neural Network-based in-loop filter with constrained computational complexity”, JVET-W0131, Jul. 2021.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).