| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0124-v1 |
| --- | --- |

| Title: | EE2-related: Intra Block Copy with An Extended Reference Area |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu 8910 University Center Ln San Diego, CA 92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | ByteDance |  |  |

_____________________________

# Abstract

This contribution presents IBC with an extended reference area. By extending the reference area to cover more CTUs that have been used by other coding modes in the current and above CTU rows, the average BD-rate(Y) numbers are –2.94% for classF and –5.88% for classTGM, with EncT change of 104% for classF and 99% for classTGM; DecT change of 95% for classF and 87% for classTGM in All Intra configurations.

# Introduction

Intra coding in ECM-3 includes several coding modes including TIMD, DIMD, Intra TMP, that need to access more memory than VVC. For example, Intra TMP mode needs to access the left, upper-left and above CTUs. Considering this change, it is straightforward to allow IBC to access more memory to gain more.

This contribution proposes an IBC reference area design that does not increase the current memory area required by ECM-3 and tests the performance.

# IBC with an extended reference area

…

…

Figure 1. Illustration of the extended reference area.

Figure 1 illustrates the design. In the figure, the blue square denotes the current CTU and the green ones denote CTUs that may be used by IBC reference. Specifically, assume that W denotes the maximum horizontal CTU index and the current CTU index is (m, n), for coding units in the current CTU, CTUs with index (0, n)…(m, n) and (m–1, n)…(W, n) defines the reference area that can be used by IBC.

One reason to have such a design is that in the current ECM, the left, above and upper-left CTUs are being used and thus need to be saved. To achieve this, all CTUs to the right of the above CTU in the above CTU row (for CTUs to be coded in the current CTU row) and all CTUs to the left of the current CTU in the current CTU row (for CTUs to be coded in the next CTU row) must be kept. It means that such a design does not increase the buffer size required by the current ECM.

# Experimental results

The proposed design has been implemented on ECM-3.1. The per-sample block vector search (or called local search) range is limited to [–(C << 1), 0] horizontally and [–C, C >> 2] vertically to adapt to the referecnce area extension, where C denotes the CTU size. Table I shows the summarized results. Detailed results can be found in the accompanying excel file.

Table I. Summarized coding performance compared with the ECM-3.1 anchor.

It is noted that the decoding runtime decrease in All Intra configuration is as expected because 1) bit-rate decreases and 2) more blocks use IBC, which can be comparatively simpler than other coding modes.

# Patent rights declaration(s)

ByteDance may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).