| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0125 |
| --- | --- |

| Title: | AHG12: Enhanced bi-directional motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Che-Wei Kuo, Ning Yan, Wei Chen, Xiaoyu Xiu, Xianglin Wang | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current ECM, the bi-directional motion compensated (MC) predictor is generated by weighted average of two uni-directional MC predictors. In the proposed scheme, the uni-directional MC predictor is regarded as out-of-boundary (OOB) when the location of the reference interpolation sample is located outside the reference picture beyond half sample. For each predictor in a bi-directional MC block, when one uni-directional MC predictor is OOB and the other one is non-OOB, this bi-directional predictor is changed into the non-OOB uni-directional MC predictor because the OOB predictor is less effective.

Compared to the ECM-3.1 anchors, simulation results reportedly show that the proposed enhancements provide average {Y, U, V}BD-rate changes of {-0.22%, -0.15%, -0.15%} and {-0.04%, -0.01%, -0.06%} for RA and LDB configurations, respectively. The corresponding enc/dec run-time are 102%/103% for RA and 101%/100% for LDB. It is noted that SIMD is not used in the current implementation, but the run time could be further optimized after SIMD implementation is applied to the proposed scheme.

It is also noted that, for class D, simulation results reportedly show that the proposed enhancements provide average {Y, U, V}BD-rate changes of {-0.81%, -0.41%, -0.38%} and {-0.15%, -0.19%, -0.59%} for RA and LDB configurations, respectively. The corresponding enc/dec run-time are 102%/104% for RA and 103%/104% for LDB.

# Introduction

In ECM-3.1 [1], due to the reference samples padding of the reference picture, it is valid for an inter CU to have a reference block located outside the reference picture partially or totally as illustrated in Figure 1. In the Figure 1, bi-directional motion compensation is performed to generate the inter prediction block of the current block. In this example, list0 reference block is partially out-of-boundary (OOB) while list 1 reference block is fully inside the reference picture. However, the OOB part of a motion compensated blocks usually provides less prediction efficiency because the OOB part is simply repetitive samples derived from the boundary samples within the reference picture. Yet, in the ECM, the less efficiency of the OOB part is not considered for the inter prediction.

Figure 1: Bi-directional MC block in current ECM

# Proposal

Since the OOB reference samples are generated through the padding process, the motion compensation (MC) predictors generated using the OOB reference samples are less effective. In this contribution, when combining more than one prediction blocks generated by the motion compensation process, the OOB predictors are discarded and only the non-OOB predictors are used to generate the final predictor.

To be specific, the positions of the predictors within the current block are denoted as Pos_xi,j and Pos_yi,j, the MV of the current block are denoted as Mv_xi,jLx and Mv_yi,jLx (x could be 0 or 1 for list 0 and list1, respectively). PosLeftBdry, PosRightBdry, PosTopBdry, PosBottomBdry are the positions of four boundaries of a picture. Since 1/16-pel MV is used in the ECM, all variables are denoted in the unit of the 1/16 sample and thus the value of half_pixel is set equal to 8.

The predictor Pi,jLx is regarded as OOB when at least one of the following conditions holds

(Pos_xi,j+Mv_xi,jLx)>(PosRightBdry+half_pixel),

(Pos_xi,j+Mv_xi,jLx)<(PosLeftBdry- half_pixel),

(Pos_yi,j+Mv_yi,jLx)>(PosBottomBdry+ half_pixel),

or  (Pos_yi,j+Mv_yi,jLx)<(PosTopBdry- half_pixel)

otherwise, when none of the above conditions holds, the predictor Pi,jLx is regarded as non-OOB.

Figure 2: Proposed Bi-directional MC

As illustrated in Figure 2, after the OOB condition is determined for each predictor, the proposed scheme is applied to the bi-directional MC blocks to generate the final predictors where the following procedures are deployed. Please be noted that the same checking mechanism is also applied when BCW is enabled.

If Pi,jL0 is OOB and Pi,jL1 is non-OOB

Pi,jfinal=Pi,jL1

else if Pi,jL0 is non-OOB and Pi,jL1 is OOB

Pi,jfinal=Pi,jL0

else

Pi,jfinal=(Pi,jL0+Pi,jL1+1)>>1

# Simulation results

The proposed methods were implemented on top of the reference software ECM-3.1 [2] and tested using the common test conditions as defined in [3]. It is noted that SIMD is not used in the current implementation, but the run time could be further optimized after SIMD implementation is applied to the proposed scheme.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.18% | -0.23% | 102% | 103% |
| Class A2 | -0.17% | 0.00% | -0.12% | 102% | 103% |
| Class B | -0.23% | -0.16% | -0.08% | 102% | 104% |
| Class C | -0.31% | -0.24% | -0.20% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.15% | -0.15% | 102% | 103% |
| Class D | -0.81% | -0.41% | -0.38% | 102% | 104% |
| Class F | -0.02% | -0.06% | -0.04% | 102% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.46% | 0.04% | 100% | 100% |
| Class C | -0.07% | 0.01% | -0.11% | 101% | 101% |
| Class E | -0.04% | 0.73% | -0.14% | 102% | 98% |
| Overall | -0.04% | -0.01% | -0.06% | 101% | 100% |
| Class D | -0.15% | -0.19% | -0.59% | 103% | 104% |
| Class F | -0.01% | 0.44% | 0.27% | 101% | 102% |

# References

M. Coban, F. Le Léannec, J. Ström, “Algorithm description of Enhanced Compression Model 3 (ECM3)”, JVET-X2025, October, 2021, Teleconference.

ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, October, 2021.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).