| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0126-v1 |
| --- | --- |

| Title: | AHG10: VTM encoder configurations for tests targeting improved coding performance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy, Marta Karczewicz, (Qualcomm), Kenneth Andersson, Rickard Sjöberg, Lukasz Litwic (Ericsson), P. Nikitin, G. Martin-Cocher (InterDigital), Adam Wieckowski, Jens Brandenburg, Benjamin Bross (HHI) | Email: | dmytror@qti.qualcomm.com, martak@qti.qualcomm.com, kenneth.r.andersson@ericsson.com rickard.sjöberg@ericsson.com lukasz.litwic@ericsson.com first.last @ interdigital.com first.last@hhi.fraunhofer.de |
| Source: | Ericsson, HHI, InterDigital, Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, a set of encoder-only modifications and settings targeting improved compression efficiency of VTM reference software is proposed. Proposed method is implemented as minor encoder software change on top VTM-14.2 and encoder configuration files for RA and LD. It is reported that under the CTC, proposed method provides the following bd-rate gain vs. VTM-14.2 anchor:

- RA: -2.66%/-6.24%/-6.13% with 168%EncT/96%DecT

It is proposed to adopt SW change to the VTM reference software and include configuration files for tests targeting improved coding performance, for example for benchmarking purposes. No changes to VTM CTC are requested.

# Introduction

Observing an ongoing VVC adoption by the industry, proponents of the current contribution identified a need for VTM encoder configuration providing higher coding efficiency comparing to the defined in JVET CTC [2]. Assuming, that such configuration would not be used for research or development of new algorithms, but rather for benchmarking purposes, less constrained ratios for achieved coding gain vs. run time increase can be tolerated.

# Proposed Method

The current contribution proposes the following minor SW changes:

#define FAST_ADAPT_COST_PRED 1 // changes RDO cost factor for partitioning 	                                  0 VTM-14.2

1 modified for more coding efficiency

2 modified for lower encoding time

#define FAST_NO_TT_FROM_BT   0 // fast decision for TT from BT

1 on (VTM-14.2)

2 off (more coding efficiency but slower)

#define FAST_RPR_CHECK       1 // fix to reduce runtime (bit exact)

A patch that contains these changes is provided with this contribution. In case the changes are agreed to be integrated into VTM, the first two macros should be replaced by a config file parameter with a default setting that does not change the CTC.

Proposed encoder configuration change combines the following encoder and normative tools adaptations:

Configuration files changes for RA:

MaxMTTHierarchyDepthISliceL  : 4

MaxMTTHierarchyDepthISliceC  : 4

MTS 				: 3

EncDbOpt 			: 1

FEN                         : 0

FDM                        : 0

TemporalFilterStrengthFrame4  : 0.3

DeblockingFilterBetaOffset_div2     : -2

DeblockingFilterCbBetaOffset_div2   : -2

DeblockingFilterCrBetaOffset_div2   : -2

IntraQPOffset          : -4

Frame1 : B 32  -1 0.0    0.0   0 0 1.0 0   0  0 0  0 0  0 2  5 32 64 48 40 36  1  2  32 48 
Frame2 : B 16 0  -4.9309   0.2265  0 0 1.0 0   0  0 0  0 0  1 3  5 16 32 48 24 20  1  1  -16 
Frame3 : B  8 1  -4.5000   0.1900  0 0 1.0 0   0  0 0  0 0  2 4  5 8 24 16 40 12 2  2  -8 -24 
Frame4 : B  4 3  -5.4095   0.2571  0 0 1.0 0   0  0 0  0 0  3 3  3 4 20 8 3  3  -4 -12 -28 
Frame5 : B  2 5  -4.4895   0.1947  0 0 1.0 0 0  0 0  0 0  4 3  3 2 18 6 4  4  -2 -6 -14 -30 
Frame6 : B  1 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 1  1 1 2  5  -1 -3 -7 -15 -31 
Frame7 : B  3 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  2 1 3  2  4  -1 -5 -13 -29 
Frame8 : B  6 5  -4.4895 0.1947  0 0 1.0 0 0  0 0  0 0  4 3  3 2 6 4  3  3  -2 -10 -26 
Frame9 : B  5 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  2 1 5  2  4  -1 -3 -11 -27 
Frame10  : B  7 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  3 1 3 7  2  3  -1 -9 -25 
Frame11  : B 12 3  -5.4095 0.2571  0 0 1.0 0 0  0 0  0 0  3 3  4 4 12 8 6 2  2  -4 -20 
Frame12  : B 10 5  -4.4895 0.1947  0 0 1.0 0 0  0 0  0 0  4 4  4 2 6 10 4 3  3  -2 -6 -22 
Frame13  : B  9 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  3 1 5  9 2  4  -1 -3 -7 -23 
Frame14  : B 11 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  3 1 3 11 2  3  -1 -5 -21 
Frame15  : B 14 5  -4.4895 0.1947  0 0 1.0 0 0  0 0  0 0  4 4  4 2 4 6 14 2  2  -2 -18 
Frame16  : B 13 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  3 1 5 13 2  3  -1 -3 -19 
Frame17  : B 15 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  4 1 3 7 15 2  2  -1 -17 
Frame18  : B 24 1  -4.5000 0.1900  0 0 1.0 0 0  0 0  0 0  2 3  3 8 24 16  1  1  -8
Frame19  : B 20 3  -5.4095 0.2571  0 0 1.0 0 0  0 0  0 0  3 3  3 4 12 20  2  2  -4 -12 
Frame20  : B 18 5  -4.4895 0.1947  0 0 1.0 0 0  0 0  0 0  4 3  3 2 10 18  3  3  -2 -6 -14 
Frame21  : B 17 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  3 1 9 17 2  4  -1 -3 -7 -15 
Frame22  : B 19 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  3 1 3 19 2  3  -1 -5 -13 
Frame23  : B 22 5  -4.4895 0.1947  0 0 1.0 0 0  0 0  0 0  4 3  3 2 6 22 3  3  -2 -10 4 
Frame24  : B 21 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  3 1 5 21 2  3  -1 -3 -11 
Frame25  : B 23 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  4 1 3 7 23 2  2  -1 -9 
Frame26  : B 28 3  -5.4095 0.2571  0 0 1.0 0 0  0 0  0 0  3 4  4 4 8 12 28 1  1  -4 
Frame27  : B 26 5  -4.4895 0.1947  0 0 1.0 0 0  0 0  0 0  4 4  4 2 6 10 26 2  2  -2 -6 
Frame28  : B 25 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  4 1 5 9 25 2  3  -1 -3 -7 
Frame29  : B 27 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  4 1 3 11 27 2  2  -1 -5 
Frame30  : B 30 5  -4.4895 0.1947  0 0 1.0 0 0  0 0  0 0  4 4  4 2 6 14 30 1  1  -2 
Frame31  : B 29 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  4 1 5 13 29 2  2  -1 -3 
Frame32  : B 31 6  -5.4429 0.2429  0 0 1.0 0 0  0 0  0 0  5 2  5 1 3 7 15 31 1  1  -1

Configuration files changes for LD:

MaxMTTHierarchyDepthISliceL  : 4

MaxMTTHierarchyDepthISliceC  : 4

MTS 				: 3

EncDbOpt 			: 1

FEN  : 0

FDM : 0

DeblockingFilterBetaOffset_div2 : -2

DeblockingFilterCbBetaOffset_div2 : -2

DeblockingFilterCrBetaOffset_div2 : -2

Examples of modified configuration files are provided along this contribution.

# Test Results

Proposed changes have been tested on top of VTM-14.2 software [1] under JVET CTC [2].

Table 1 summarizes the performance of the proposed encoder-only modification and settings, comparing the VTM-14.2 anchor.

Table 1- Test results of the proposed changes

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test Over VTM-14.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.53% | -3.62% | -4.57% | 155% | 99% |
| Class A2 | -2.80% | -5.48% | -4.54% | 170% | 100% |
| Class B | -3.01% | -7.64% | -7.59% | 169% | 93% |
| Class C | -2.94% | -7.02% | -6.69% | 175% | 95% |
| Class E |  |  |  |  |  |
| Overall | -2.66% | -6.24% | -6.13% | 168% | 96% |
| Class D | -2.18% | -7.34% | -6.90% | 171% | 93% |
| Class F |  |  |  |  |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-14.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -1.66% | -1.18% | -0.98% | 161% | 98% |
| Class E | -3.10% | -3.54% | -3.02% | 133% | 101% |
| Overall |  |  |  |  |  |
| Class D | -1.23% | -1.05% | -0.98% | 163% | 101% |
| Class F | -1.66% | -1.27% | -1.46% | 142% | 90% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-14.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusion

This contribution proposes an encoder only software change and encoder configuration changes for VTM to achieve a higher coding efficiency operational point. Comparing to VTM-14.2, BD-rate coding gain of  - 2.66% / -6.24%/ -6.13% for Y/U/V components in RA configuration is achieved with 168% EncT / 96% DecT run time change.

# Reference

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-14.2

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.

HHI does not have any current or pending patent rights relating to the technology described in this contribution.

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).