| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0127-v1 |
| --- | --- |

| Title: | AHG5: Editors update on VVC conformance testing |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Tomonori Hashimoto Hong-Jheng Jhu Iole Moccagatta Mohammed Sarwer Yue Yu | Email: | dmytror@qti.qualcomm.com tomonori.hashimoto@sharp.co.jp jhuhong-jheng@kwai.com iole.moccagatta@intel.com m.sarwer@alibaba-inc.com yue.yu@oppo.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is draft 2 of the conformance testing specification for VVC operation range extensions, for testing the new coding tool features being added in the second edition of VVC (Rec. ITU-T H.266 | ISO/IEC 23090-3).

Due to the size of the electronic attachments, the associated conformance test set data files are provided separately, at the following location:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2/under_test/VTM-14.0/

(user id: avguest, password: Avguest201007)

For ftp access, it is suggested to use FileZilla, for which the site manager feature should be used and ftp encryption should be set to “Require implicit ftp over TLS” (see Annex A of this document).

Alternatively, the same files are available by http at:

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2/under_test/VTM-14.0/

Annex A of this document contains JVET guidelines for contributing to the conformance bitstream test set. It is not expected to be published as part of the text approved by the parent bodies.

Conformance bitstreams preparation (editors’ notes – not part of the specification text)

This section is to be removed after work on the conformance bitstream production and verification is completed.

reference software decoder: The software decoder provided in Rec. ITU-T H.266.2 | ISO/IEC 23090-16; the current implementation is VTM-14.0.

reference software encoder: The software encoder provided in Rec. ITU-T H.266.2 | ISO/IEC 23090-16; the current implementation is VTM-14.0.

See also Annex A of this document.

Draft changes to the specification text

# Clause 3

# In clause 3 (Definitions), add the following abreviation terms:

3.6	VVCv1 tools: Coding tools of Versatile Video Coding (VVC), specified in in ISO/IEC 23090-3.

3.7	VVCv2 tools: Coding tools of the VVC operation range extensions, specified in in ISO/IEC DIS 23090-3 (2nd edition).

# Clause 4

# In clause 4 (Abreviations and acronyms), add the following abreviation terms:

EPP: Extended precision processing

ERRC: Extended regular residual coding

ETSRC: Extended transform skip residual coding

PRRC: Persistent regular residual coding

RA: Random Access

RLSCP: Reversed last significant coefficient position

RRC: Regular residual coding

WPP: Wavefront parallel processing

# 6.6

# In subclause 6.6, add subclauses as follows:

    - Test bitstreams – Main 12 profile

      - VVCv1 tools set

        - Test bitstream 12b420vvc1_A_Alibaba

Specification: The bitstream is 12-bit 4:2:0 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:2:0 content.

    - Test bitstreams – Main 12 Intra profile

      - VVCv1 tools set

        - Test bitstream 12b420Ivvc1_A_Interdigital[bitstream is currently absent]

Specification: The bitstream is 12-bit 4:2:0, in all intra mode, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:2:0 content.

    - Test bitstreams – Main 12 Still Picture profile

      - VVCv1 tools set

        - Test bitstream 12b420SPvvc1_A_KDDI

Specification: The bitstream contains a single picture in 12-bit 4:2:0 chroma format and exercises MinCr restriction for level 2.0, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Still picture.

Purpose: Check that the decoder can properly decode bitstreams conforming to the still picture profiles.

    - Test bitstreams – Main 12 4:4:4 profile

      - VVCv1 tools set

        - Test bitstream 12b444vvc1_A_Sony

Specification: The bitstream is 12-bit 4:4:4 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 2.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:4:4 content.

        - Test bitstream 12b444vvc1_B_Sony

Specification: The bitstream is 12-bit 4:4:4 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 4.1, Main Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:4:4 content.

        - Test bitstream 12b444vvc1_C_Sony

Specification: The bitstream is 12-bit 4:4:4 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 5.1, Main Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:4:4 content.

        - Test bitstream 12b444vvc1_D_Sony

Specification: The bitstream is 12-bit 4:4:4 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 4.1, High Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:4:4 content.

        - Test bitstream 12b444vvc1_E_Sony

Specification: The bitstream is 12-bit 4:4:4 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 5.1, High Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:4:4 content.

      - Extended Precision Processing (EPP)

        - Test bitstream 12b444epp_A_Sharp[bitstream is currently absent]

Specification: The bitstream is 12-bit 4:4:4 video data being coded with various random access picture types and EPP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with EPP enabled.

      - Extended RRC (ERRC)

        - Test bitstream 12b444errc_A_Qualcomm

Specification: 12 bits video data with 4:4:4 chroma format is being coded with random access picture types, testing of the ERRC functionality, PRRC is enabled, Level 4.1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

        - Test bitstream 12b444errc_B_Qualcomm

Specification: 12 bits video data with 4:4:4 chroma format is being coded with random access picture types, testing of the ERRC functionality, PRRC is disabled, Level 4.1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

        - Test bitstream 12b444errc_C_Qualcomm

Specification: 12 bits video data with 4:4:4 chroma format is being coded with random access picture types, testing of the ERRC functionality, PRRC is enabled, Level 5.1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

      - Persistent RRC (PRRC)

        - Test bitstream 12b444prrc_A_Qualcomm

Specification: 12 bits video data with 4:4:4 chroma format is being coded with random access picture types, testing of the PRRC functionality, ERRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with PRRC enabled.

      - Extended TSRC (ETSRC)

        - Test bitstream 12b444etsrc_A_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded with various random access picture types and ETSRC enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

      - Reversed Last Significant Coefficient Position (RLSCP)

        - Test bitstream 12b444rlscp_A_OPPO

Specification: The bitstream is 12-bit 4:4:4 video data being coded with various random access picture types and RLSCP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with RLSCP enabled.

      - Wavefront Parallel Process (WPP)

        - Test bitstream 12b444wpp_A_OPPO[bitstream is currently absent]

Specification: The bitstream is 12-bit 4:4:4 video data being coded with various random access picture types and sps_entropy_coding_sync_enabled_flag equal to 1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode bitstreams with wavefront enabled.

    - Test bitstreams – Main 12 4:4:4 Intra profile

      - VVCv1 tools set

        - Test bitstream 12b444Ivvc1_A_Alibaba

Specification: The bitstream is 12-bit 4:4:4, in all intra mode, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:4:4 content.

      - Extended Precision Processing (EPP)

        - Test bitstream 12b444Iepp_A_Sharp[bitstream is currently absent]

Specification: The bitstream is 12-bit 4:4:4, in all intra mode and EPP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with EPP enabled.

      - Extended RRC (ERRC)

        - Test bitstream 12b444Ierrc_A_Qualcomm

Specification: 12 bits video data with 4:4:4 chroma format is being coded in all intra configuration, testing of the ERRC functionality, PRRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

        - Test bitstream 12b444Ierrc_B_Qualcomm

Specification: 12 bits video data with 4:4:4 chroma format is being coded in all intra configuration, testing of the ERRC functionality, PRRC is disabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

      - Persistent RRC (PRRC)

        - Test bitstream 12b444Iprrc_A_Qualcomm

Specification: 12 bits video data with 4:4:4 chroma format is being coded in all intra configuration, testing of the PRRC functionality, ERRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with PRRC enabled.

      - Extended TSRC (ETSRC)

        - Test bitstream 12b444Ietsrc_A_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded in all intra configuration and ETSRC enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

      - Reversed Last Significant Coefficient Position (RLSCP)

        - Test bitstream 12b444Irlscp_A_OPPO

Specification: The bitstream is 12-bit 4:4:4, in all intra mode and RLSCP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with RLSCP enabled.

      - Wavefront Parallel Process (WPP)

        - Test bitstream 12b444Iwpp_A_OPPO[bitstream is currently absent]

Specification: The bitstream is 12-bit 4:4:4, in all intra mode and sps_entropy_coding_sync_enabled_flag equal to 1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode bitstreams with wavefront  enabled.

    - Test bitstreams – Main 12 4:4:4 Still Picture profile

      - VVCv1 tools set

        - Test bitstream 12b444SPvvc1_A_Alibaba

Specification: The bitstream contains a single picture in 12-bit 4:4:4 chroma format, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Still picture.

Purpose: Check that the decoder can properly decode bitstreams conforming to the still picture profiles.

    - Extended Precision Processing (EPP)

      - Test bitstream 12b444SPepp_A_Sharp[bitstream is currently absent]

Specification: The bitstream contains a single picture in 12-bit 4:4:4 chroma format and EPP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with EPP enabled.

      - Extended RRC (ERRC)

        - Test bitstream 12b444SPerrc_A_Qualcomm

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, testing of the ERRC functionality, PRRC is enabled, Level 5.1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

      - Persistent RRC (PRRC)

        - Test bitstream 12b444SPprrc_A_Qualcomm

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, testing of the PRRC functionality, ERRC is enabled, Level 5.1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with PRRC enabled.

      - Extended TSRC (ETSRC)

        - Test bitstream 12b444SPetsrc_A_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 12b444SPetsrc_B_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 12b444SPetsrc_C_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 3.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 12b444SPetsrc_D_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 4.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 12b444SPetsrc_E_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, ETSRC is enabled and its Rice parameter is set to 5.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 12b444SPetsrc_F_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, ETSRC is enabled and its Rice parameter is set to 6.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 12b444SPetsrc_G_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, ETSRC is enabled and its Rice parameter is set to 7.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 12b444SPetsrc_H_Kwai

Specification: 12 bits video data with 4:4:4 chroma format is being coded in still image profile configuration, ETSRC is enabled and its Rice parameter is set to 8.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

      - Reversed Last Significant Coefficient Position (RLSCP)

        - Test bitstream 12b444SPrlscp_A_OPPO

Specification: The bitstream contains a single picture in 12-bit 4:4:4 chroma format and RLSCP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with RLSCP enabled.

    - Wavefront Parallel Process (WPP)

        - Test bitstream 12b444SPwpp_A_OPPO[bitstream is currently absent]

Specification: The bitstream contains a single picture in 12-bit 4:4:4 chroma format and sps_entropy_coding_sync_enabled_flag equal to 1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode bitstreams with wavefront  enabled.

    - Test bitstreams – Main 16 4:4:4 profile

      - VVCv1 tools set

        - Test bitstream 16b444vvc1_A_Alibaba

Specification: The bitstream is 16-bit 4:4:4 video data being coded with various random access picture types, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 16-bit 4:4:4 content.

      - Extended Precision Processing (EPP)

        - Test bitstream 16b444epp_A_Sharp[bitstream is currently absent]

Specification: 16 bits video data with 4:4:4 chroma format is being coded with various random access picture types and EPP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with EPP enabled.

      - Extended RRC (ERRC)

        - Test bitstream 16b444errc_A_Qualcomm

Specification: 16 bits RGB video data with is being coded with random access picture types, testing of the ERRC functionality, PRRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

        - Test bitstream 16b444errc_B_Qualcomm

Specification: 16 bits RGB video data with is being coded with random access picture types, testing of the ERRC functionality, PRRC is disabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

        - Test bitstream 16b444errc_C_Qualcomm

Specification: 16 bits RGB video data with is being coded with random access picture types, testing of the ERRC functionality, PRRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

      - Persistent RRC (PRRC)

        - Test bitstream 16b444prrc_A_Qualcomm

Specification: 16 bits RGB video data with is being coded with random access picture types, testing of the PRRC functionality, ERRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with PRRC enabled.

      - Extended TSRC (ETSRC)

        - Test bitstream 16b444etsrc_A_Kwai

Specification: 16 bits RGB video data is being coded with various random access picture types and ETSRC enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

      - Reversed Last Significant Coefficient Position (RLSCP)

        - Test bitstream 16b444rlscp_A_OPPO

Specification: 16 bits video data with 4:4:4 chroma format is being coded with various random access picture types and RLSCP enabled.

Functional stage: Reconstruction process

Purpose: Check that the decoder can properly decode slices with RLSCP enabled.

      - Wavefront Parallel Process (WPP)

        - Test bitstream 16b444wpp_A_OPPO[bitstream is currently absent]

Specification: 16 bits video data with 4:4:4 chroma format is being coded with various random access picture types and sps_entropy_coding_sync_enabled_flag equal to 1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode bitstreams with wavefront wavefront enabled.

#### Chroma format and bit depth

        - Test bitstream 10b444P16_A_Sony

Specification: A random access sequence of 10-bit 4:4:4 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 2.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 10-bit 4:4:4 content.

        - Test bitstream 10b444P16_B_Sony

Specification: A random access sequence of 10-bit 4:4:4 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 4.1, Main Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 10-bit 4:4:4 content.

        - Test bitstream 10b444P16_C_Sony

Specification: A random access sequence of 10-bit 4:4:4 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 5.1, Main Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 10-bit 4:4:4 content.

        - Test bitstream 10b444P16_D_Sony

Specification: A random access sequence of 10-bit 4:4:4 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 4.1, High Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 10-bit 4:4:4 content.

        - Test bitstream 10b444P16_E_Sony

Specification: A random access sequence of 10-bit 4:4:4 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 5.1, High Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 10-bit 4:4:4 content.

        - Test bitstream 12b400P16_A_Sony

Specification: A random access sequence of 12-bit 4:0:0 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 2.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:0:0 content.

        - Test bitstream 12b400P16_B_Sony

Specification: A random access sequence of 12-bit 4:0:0 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 4.1, Main Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:0:0 content.

        - Test bitstream 12b400P16_C_Sony

Specification: A random access sequence of 12-bit 4:0:0 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 5.1, Main Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:0:0 content.

        - Test bitstream 12b400P16_D_Sony

Specification: A random access sequence of 12-bit 4:0:0 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 4.1, High Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:0:0 content.

        - Test bitstream 12b400P16_E_Sony

Specification: A random access sequence of 12-bit 4:0:0 data encoded in the Main 16 4:4:4 profile, VVCv1 tools set is enabled and no VVCv2 tools are enabled, Level 5.1, High Tier.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 12-bit 4:0:0 content.

    - Test bitstreams – Main 16 4:4:4 Intra profile

      - VVCv1 tools set

        - Test bitstream 16b444Ivvc1_A_Alibaba

Specification: The bitstream is 16-bit 4:4:4, in all intra mode, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Decoder.

Purpose: Check that the decoder can properly decode 16-bit 4:4:4 content.

      - Extended Precision Processing (EPP)

        - Test bitstream 16b444Iepp_A_Sharp[bitstream is currently absent]

Specification: The bitstream is 16-bit 4:4:4, in all intra mode and EPP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with EPP enabled.

      - Extended RRC (ERRC)

        - Test bitstream 16b444Ierrc_A_Qualcomm

Specification: 16 bits RGB video data with is being coded with intra picture types, testing of the ERRC functionality, PRRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

      - Persistent RRC (PRRC)

        - Test bitstream 16b444Iprrc_A_Qualcomm

Specification: 16 bits RGB video data with is being coded with intra picture types testing of the PRRC functionality, ERRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with PRRC enabled.

      - Extended TSRC (ETSRC)

        - Test bitstream 16b444Ietsrc_A_Kwai

Specification: 16 bits RGB video data is being coded in all intra configuration and ETSRC enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

      - Reversed Last Significant Coefficient Position (RLSCP)

        - Test bitstream 16b444Irlscp_A_OPPO

Specification: The bitstream is 16-bit 4:4:4, in all intra mode and RLSCP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with RLSCP enabled.

      - Wavefront Parallel Process (WPP)

        - Test bitstream 16b444Iwpp_A_OPPO[bitstream is currently absent]

Specification: The bitstream is 16-bit 4:4:4, in all intra mode and sps_entropy_coding_sync_enabled_flag equal to 1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode bitstreams with wavefront  enabled.

    - Test bitstreams – Main 16 4:4:4 Still Picture profile

      - VVCv1 tools set

        - Test bitstream 16b444SPvvc1_A_Alibaba

Specification: The bitstream contains a single picture in 16-bit 4:4:4 chroma format, VVCv1 tools set is enabled and no VVCv2 tools are enabled.

Functional stage: Still picture.

Purpose: Check that the decoder can properly decode bitstreams conforming to the still picture profiles.

      - Extended Precision Processing (EPP)

        - Test bitstream 16b444SPepp_A_Sharp[bitstream is currently absent]

Specification: The bitstream contains a single picture in 16-bit 4:4:4 chroma format and EPP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with EPP enabled.

      - Extended RRC (ERRC)

        - Test bitstream 16b444SPerrc_A_Qualcomm

Specification: 16 bits RGB video data with is being coded with still picture profile configuration, testing of the ERRC functionality, PRRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ERRC enabled.

      - Persistent RRC (PRRC)

        - Test bitstream 16b444SPprrc_A_Qualcomm

Specification: 16 bits RGB video data with is being coded with still picture profile configuration, testing of the PRRC functionality, ERRC is enabled, Level 6.2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with PRRC enabled.

      - Extended TSRC (ETSRC)

        - Test bitstream 16b444SPetsrc_A_Kwai

Specification: 16 bits RGB video data is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 16b444SPetsrc_B_Kwai

Specification: 16 bits RGB video data is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 2.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 16b444SPetsrc_C_Kwai

Specification: 16 bits RGB video data is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 3.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 16b444SPetsrc_D_Kwai

Specification: 16 bits RGB video data is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 4.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 16b444SPetsrc_E_Kwai

Specification: 16 bits RGB video data is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 5.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 16b444SPetsrc_F_Kwai

Specification: 16 bits RGB video data is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 6.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 16b444SPetsrc_G_Kwai

Specification: 16 bits RGB video data is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 7.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

        - Test bitstream 16b444SPetsrc_H_Kwai

Specification: 16 bits RGB video data is being coded in still image profile configuration, ETSRC is enabled, and its Rice parameter is set to 8.

Functional stage: Reconstruction process

Purpose: Check that the decoder can properly decode slices with ETSRC enabled.

      - Reversed Last Significant Coefficient Position (RLSCP)

        - Test bitstream 16b444SPrlscp_A_OPPO

Specification: The bitstream contains a single picture in 16-bit 4:4:4 chroma format and RLSCP enabled.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode slices with RLSCP enabled.

    - Wavefront Parallel Process (WPP)

        - Test bitstream 16b444SPwpp_A_OPPO[bitstream is currently absent]

Specification: The bitstream contains a single picture in 16-bit 4:4:4 chroma format and sps_entropy_coding_sync_enabled_flag equal to 1.

Functional stage: Reconstruction process.

Purpose: Check that the decoder can properly decode bitstreams with wavefront enabled.

# 6.7

# In subclause 6.7 (Conformance test suites for Rec. ITU-T H.266 | ISO/IEC 23090-3), add subclauses as follows:

    - Bitstreams for Main 12 profile

Table 11: Bitstreams for Main 10 profile

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 12 | VVC1 tools | VVC1 tools | 12b420vvc1 | 12-bit 4:2:0, VVCv1 tools set |

    - Bitstreams for Main 12 Intra profile

Table 12: Bitstreams for Main 12 Intra profile

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 12 Intra | VVC1 tools | VVC1 tools | 12b420Ivvc1 | 12-bit 4:2:0, VVCv1 tools set |

    - Bitstreams for Main 12 Still Picture profile

Table 13: Bitstreams for Main 12 Still Picture profile

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 12 Still Picture | VVC1 tools | VVC1 tools | 12b420SPvvc1 | 12-bit 4:2:0, VVCv1 tools set |

    - Bitstreams for Main 12 4:4:4 profile

Table 14: Bitstreams for Main 12 4:4:4 profile

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 12 4:4:4 | VVC1 tools | VVC1 tools | 12b444vvc1 | 12-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 12b444errc | 12-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 12b444prrc | 12-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 12b444etsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 12b444epp | 12-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSCP | RLSCP | 12b444rlscp | 12-bit 4:4:4, VVCv2 set, RLSCP test |
|  | WPP | WPP | 12b444wpp | 12-bit 4:4:4, VVCv2 set, WPP test |
| Main 12 4:4:4 | Chroma format and bit depth | 10b 4:4:4 | 10b444P12 | 10-bit 4:4:4, VVCv1 tools set, RA |
|  |  | 12b 4:0:0 | 12b400P12 | 12-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:0 | 12b420P12 | 12-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:2 | 12b422P12 | 12-bit 4:2:2, VVCv2 tools set, RA |

    - Bitstreams for Main 12 4:4:4 Intra profile

Table 15: Bitstreams for Main 12 4:4:4 Intra profile

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 12 4:4:4 Intra | VVC1 tools | VVC1 tools | 12b444Ivvc1 | 12-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 12b444Ierrc | 12-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 12b444Iprrc | 12-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 12b444Ietsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 12b444Iepp | 12-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSCP | RLSCP | 12b444Irlscp | 12-bit 4:4:4, VVCv2 set, RLSCP test |
|  | WPP | WPP | 12b444Iwpp | 12-bit 4:4:4, VVCv2 set, WPP test |

    - Bitstreams for Main 12 4:4:4 Still Picture profile

Table 16: Bitstreams for Main 12 4:4:4 Still Picture profile

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 12 4:4:4 Still Picture | VVC1 tools | VVC1 tools | 12b444SPvvc1 | 12-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 12b444SPerrc | 12-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 12b444SPprrc | 12-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 12b444SPetsrc | 12-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 12b444SPepp | 12-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSCP | RLSCP | 12b444SPrlscp | 12-bit 4:4:4, VVCv2 set, RLSCP test |
|  | WPP | WPP | 12b444SPwpp | 12-bit 4:4:4, VVCv2 set, WPP test |

    - Bitstreams for Main 16 4:4:4 profile

Table 17: Bitstreams for Main 16 4:4:4 profile

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 16 4:4:4 | VVC1 tools | VVC1 tools | 16b444vvc1 | 16-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 16b444errc | 16-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 16b444prrc | 16-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 16b444etsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 16b444epp | 16-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSCP | RLSCP | 16b444rlscp | 16-bit 4:4:4, VVCv2 set, RLSCP test |
|  | WPP | WPP | 16b444wpp | 16-bit 4:4:4, VVCv2 set, WPP test |
|  | Chroma format and bit depth | 10b 4:4:4 | 10b444P16 | 10-bit 4:4:4, VVCv1 tools set, RA |
|  |  | 12b 4:0:0 | 12b400P16 | 12-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:0 | 12b420P16 | 12-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 12b 4:2:2 | 12b422P16 | 12-bit 4:2:2, VVCv2 tools set, RA |
|  |  | 12b 4:4:4 | 12b444P16 | 12-bit 4:4:4, VVCv2 tools set, RA |
|  |  | 16b 4:0:0 | 16b400P16 | 16-bit 4:0:0, VVCv2 tools set, RA |
|  |  | 16b 4:2:0 | 16b420P16 | 16-bit 4:2:0, VVCv2 tools set, RA |
|  |  | 16b 4:2:2 | 16b422P16 | 16-bit 4:2:2, VVCv2 tools set, RA |

    - Bitstreams for Main 16 4:4:4 Intra profile

Table 18: Bitstreams for Main 16 4:4:4 Intra profile

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 16 4:4:4 Intra | VVC1 tools | VVC1 tools | 16b444Ivvc1 | 16-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 16b444Ierrc | 16-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 16b444Iprrc | 16-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 16b444Ietsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 16b444Iepp | 16-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSCP | RLSCP | 16b444Irlscp | 16-bit 4:4:4, VVCv2 set, RLSCP test |
|  | WPP | WPP | 16b444Iwpp | 16-bit 4:4:4, VVCv2 set, WPP test |

    - Bitstreams for Main 16 4:4:4 Still Picture profile

Table 19: Bitstreams for Main 16 4:4:4 Still Picture profile

| Profile | Category | Tool description | Feature Name | Bitstream features |
| --- | --- | --- | --- | --- |
| Main 16 4:4:4 Still Picture | VVC1 tools | VVC1 tools | 16b444SPvvc1 | 16-bit 4:4:4, VVCv1 tools set |
|  | ERRC | ERRC | 16b444SPerrc | 16-bit 4:4:4, VVCv2 set, ERRC test |
|  | PRRC | PRRC | 16b444SPprrc | 16-bit 4:4:4, VVCv2 set, PRRC test |
|  | ETSRC | ETSRC | 16b444SPetsrc | 16-bit 4:4:4, VVCv2 set, ETSRC test |
|  | EPP | EPP | 16b444SPepp | 16-bit 4:4:4, VVCv2 set, EPP test |
|  | RLSCP | RLSCP | 16b444SPrlscp | 16-bit 4:4:4, VVCv2 set, RLSCP test |
|  | WPP | WPP | 16b444SPwpp | 16-bit 4:4:4, VVCv2 set, WPP test |

# Annex A to JVET-X2026
Conformance bitstream generation guidelines

# (not part of the draft specification text)

  - General

This annex provides conformance bitstream generation guidelines.

  - Procedure

    - FTP site

An FTP site at ITU-T is used to exchange bitstreams. The site for FTP download of bitstreams is:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2/under_test/VTM-14.0/

(user id: avguest, password: Avguest201007)

FTP encryption should be set to “Require implicit ftp over TLS”. If using FileZilla, set user id and FTP encryption by configuring site manager as follows:

The site is also accessible by HTTP (without a password) at:

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2/under_test/VTM-14.0/

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation will be available at this directory. It includes the list of bitstreams, configuration features and settings, and status of verification.

The FTP site for uploading bitstream files is as follows:

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, password: Avguest201007)

Once a bitstream is uploaded to the dropbox, please send email to jvet-conformance@lists.rwth-aachen.de  to notify the coordinators of its availability.

    - Bitstreams and associated files

Volunteers should upload the bitstream and associated files in a zip archive. All files inside the zip archive should have the same base name. Only the extension is changed in the following way:

*.bit – bitstream as described in section A.2.3 (mandatory)

*.txt – description (mandatory)

*.yuv.md5 – MD5  check sum of the complete decoded yuv file (mandatory)

*.md5 – MD5sum of the bitstream file (mandatory)

*.opl – output picture log as described in section A.2.4 (mandatory)

*.cfg – config file used to generate bitstream with VTM encoder SW (optional, not applicable if VTM encoder release version not used)

All bitstreams are required to use the decoded picture hash SEI message.

All files in the zip archive should be in the top level, without a subfolder.

The .md5 file should contain only the MD5sum value and no additional characters.

The following naming convention is to be used.

Feature_BistreamID_Source_Version

- Feature is summarized in Table 1

- BitstreamID is a single capital letter, to distinguish among multiple bitstreams provided for a particular feature.

- Source is the name of the organization who provided the bitstream.

- Version is the version number of the file. The version number should be 1 for the first version. The version number should be incremented when a bitstream is replaced because of an identified problem or updated for compatibility with a new version of the VTM.

For example, for the feature “feature”, bitstream “A”, from source “organization”, in its version “1”, the archive is named "feature_A_organizationA_1.zip", and the files would be named

feature_A_organization_1.bit

feature_A_organization_1.txt

feature_A_organization_1.yuv.md5

feature_A_organization_1.md5

feature_A_organization_1.opl

feature_A_organization_1.cfg

    - Description of the bitstream (name.txt)

In a short text file which describes the bitstream, the following information is to be provided:

- Bitstream file name

- Explanation of bitstream features

- Profile, tier, and level for the bitstream

- Max picture width and height, and any additional picture width and height values used (i.e., for reference picture resample or scalability)

- Picture rate

- VTM decoder release version number able to correctly decode the bitstream with identical output, or description of VTM decoder incompatibility (preferably including a corresponding error tracking issue number.)

- VTM release version number used to generate the bitstream, if applicable

- Contact name and email of bitstream submitter

    - Output picture log file for each output layer set (name.opl)

Each output picture log file contains one row for each output picture in the bitstream, in output order.

Each row contains the following information, as Comma Separated Values (CSV).

- layer ID

- PicOrderCntVal

- Width of cropped output picture in luma samples

- Height of cropped output picture in chroma samples

- MD5 checksum for the Y component

- MD5 checksum for the U component

- MD5 checksum for the V component

The reference software provides support in the decoder to output an .opl file using the -opl command line argument. For example: DecoderApp -b feature_A_organization_1.bit -opl feature_A_organization_1.opl

    - Recommendations

Test bitstreams should exercise the full range of parameters of modes of the tool tested in the category, including corner cases. Multiple bitstreams should be provided for each feature.

As new releases of the VTM software are made available, it is recommended to provide updated versions of the bitstreams compatible with the latest release.

For testing bitstreams targeting Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture, Main 16 4:4:4, Main 16 4:4:4 Intra and Main 16 4:4:4 Still Picture profiles in VTM software (VTM-14.0 and later), the high bit-depth operation range extension supports should be enabled by defining the following macro in the source code (TypeDef.h):

#define JVET_R0351_HIGH_BIT_DEPTH_ENABLED			1

The examples of the configurations files to enable high-bit depth processing can be found in “cfg/hbd” directory of VTM software.

The test bitstreams shall not violate the profile, tier, and level indicated in the bitstream. The minimum level that the bitstream conforms to should be used.

A VTM config file should be included in the .zip file if an unmodified VTM version is used, and the command line used should be included in the .txt file. If a modified version of the VTM is used, the VTM config file should not be included.

____________