| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0128-v4 |
| --- | --- |

| Title: | Non-EE2: fixing issues for RPR enabling and non-CTC configuration in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang Han Huang Chun-Chi Chen Vadim Seregin Marta Karczewicz Philippe Bordes | Tel: Email: | zhizhang@qti.qualcomm.com hanhuang@qti.qualcomm.com chunchic@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com philippe.bordes@interdigital.com |
| Source: | Qualcomm Incorporated InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes slice level enabling condition check for adaptive DMVR and BM AMVP-merge mode. When the enabling condition is not fulfilled, the flag of adaptive DMVR and/or BM AMVP-merge mode is implicitly determined to be 0. The proposed method solves software issues with non-CTC configuration (e.g., RPR enabling). This contribution also proposes fixes for LIC and TM to enable RPR in ECM-3.1.

The proposed methods were implemented on top of ECM3.1 with merge request 47 (MR47). The simulation is conducted with RPR enabled. The test results confirm the software works properly.

# Introduction

In JVET-V meeting, adaptive decoder side motion vector refinement (adaptive DMVR) [1] and bilateral matching AMVP-merge mode (BM AMVP-merge mode) [2] were adopted into ECM. The enabling condition for adaptive DMVR and BM AMVP-merge mode is that at least one unscaled reference picture is from the past of the current picture and at least one unscaled reference picture is from the future of the current picture. For adaptive DMVR, the enabling condition also requires equal POC distance between reference pictures and the current picture.

# Problem

With non-CTC configuration (e.g., RPR enabling), the current block derives the flag of adaptive DMVR or BM AMVP-merge mode from decoding bitstream regardless of the enabling condition is fulfilled or not. At decoder side, when the parsed flag is true, an undefined case may occur that a block is determined to be coded with adaptive DMVR or BM AMVP-merge mode, but the enabling condition is not fulfilled. For example, the zero-padding candidate in the adaptive DMVR candidate list may not fulfill the enabling condition. In another example, the merge candidates in the BM AMVP-merge mode candidate list may not fulfill the enabling condition.

In the case of LIC for Inter AMVP modes, the CU-level LIC flag would become redundant since ECM-3.1 would skip illuminance compensation process completely during MC stage, when the reference picture size is different from that of the current picture. For TM, the reference template block is interpolated without considering RPR and the search process for RPR case is undefined.

# Proposal

In this contribution, we introduce enabling condition check at slice level for adaptive DMVR and BM AMVP-merge mode. When the enabling condition is not fulfilled, the flag of adaptive DMVR and/or BM AMVP-merge mode is not signaled and is inferred equal to 0.

In this contribution, we also propose to fix the RPR enabling for LIC, TM-MRG and TM-AMVP mode. When the reference picture is scaled, the CU-level LIC flag is not signaled and is inferred equal to 0. When the reference picture is scaled, TM-MRG and TM-AMVP mode are disabled, and MVP index is signaled in AMVP mode to indicate MVP candidate.

# Simulation

We implement the proposed method on top of ECM-3.1 with the merge request 47 (MR47: fix RPR_ENABLE for TMVP when all ref are scaled and with X0049, X0083) [3].

The RPR is enabled in the test with the following configuration parameters:

| Configuration 1: (Each frame is scaled) | Configuration 2: (Resolution switch after every 16 frames) |
| --- | --- |
| ScalingRatioHor : 2.0 | ScalingRatioHor : 2.0 |
| ScalingRatioVer : 2.0 | ScalingRatioVer : 2.0 |
| FractionNumFrames : 1.0 | FractionNumFrames : 1.0 |
| RPR : 1 | RPR : 1 |
| UpscaledOutput : 2 | UpscaledOutput : 1 |
| SwitchPocPeriod : -1 | SwitchPocPeriod : 16 |

The simulation 4.1 and 4.2 reports the PSNR1 values that is output from ECM software. The PSNR1 values are measured on the decoded picture size by calculating MSE for each picture, accumulating MSEs for all pictures and calculating average PSNR from the accumulated MSE [4]. Four QP points {22, 27, 32, 37} are reported in simulation 4.1 and 4.2.

The simulation 4.3 and 4.4 reports the PSNR2 values that is output from ECM software. The PSNR2 values are measured after upsampling of the decoded picture if the decoded picture size is in smaller resolutionon [4]. Five QP points {22, 27, 32, 37, 42} are reported in simulation 4.3 and 4.4.

The simulation 4.5 reports the PSNR2 values that is output from ECM software. In order to match target rates and have a reliable BD-rate measurement, the simulations use the suggested QP selection as in JVET-Y0061 [6]. The simulations compare the performance between ECM-3.1 and the proposed fixes. We only report configuration 1 simulation results, because ECM-3.1 does not work properly with configuration 2.

We report simulation results of random access and low-delay B by using ECM3.1 with common test condition (CTC) [5] as anchor as follows:

## Simulation results (PSNR1) with configuration 1:

|  | Random Access |  |  |  |  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 CTC |  |  |  |  | Over ECM-3.1 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -60.27% | -42.22% | -34.68% | 25% | 28% |  |  |  |  |  |
| Class A2 | -56.36% | -72.64% | -66.13% | 25% | 28% |  |  |  |  |  |
| Class B | -57.66% | -61.74% | -49.38% | 26% | 28% | -56.53% | -57.90% | -43.80% | 28% | 26% |
| Class C | -61.10% | -56.30% | -49.23% | 28% | 29% | -62.86% | -55.52% | -46.97% | 26% | 27% |
| Class E |  |  |  |  |  | -51.43% | -37.90% | -35.36% | 30% | 24% |
| Overall | -58.84% | -58.57% | -49.75% | 26% | 28% | -57.37% | -52.10% | -42.75% | 28% | 26% |
| Class D | -62.49% | -54.73% | -50.66% | 25% | 31% | -62.91% | -54.02% | -46.58% | 24% | 27% |
| Class F | -36.53% | -37.82% | -35.33% | 34% | 28% | -36.95% | -30.20% | -27.80% | 32% | 27% |

## Simulation results (PSNR1) with configuration 2:

|  | Random Access |  |  |  |  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 CTC |  |  |  |  | Over ECM-3.1 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -33.79% | -16.41% | -12.28% | 68% | 64% |  |  |  |  |  |
| Class A2 | -26.64% | -48.70% | -38.52% | 69% | 65% |  |  |  |  |  |
| Class B | -30.31% | -42.36% | -28.62% | 69% | 64% | -31.29% | -40.69% | -27.05% | 83% | 68% |
| Class C | -21.37% | -23.98% | -15.89% | 68% | 64% | -28.04% | -27.17% | -17.90% | 69% | 68% |
| Class E |  |  |  |  |  | -28.35% | -29.92% | -29.05% | 84% | 67% |
| Overall | -27.89% | -33.54% | -23.94% | 69% | 64% | -29.47% | -33.49% | -24.50% | 78% | 68% |
| Class D | -21.31% | -22.23% | -16.48% | 70% | 66% | -27.32% | -28.98% | -20.95% | 69% | 67% |
| Class F | -5.02% | -7.79% | -5.54% | 76% | 67% | -6.86% | -8.14% | -5.09% | 81% | 72% |

## Simulation results (PSNR2) with configuration 1:

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.32% | 25.09% | 20.87% | 26% | 28% |
| Class A2 | 18.88% | 81.32% | 48.85% | 26% | 27% |
| Overall | 9.60% | 53.20% | 34.86% | 26% | 28% |

## Simulation results (PSNR2) with configuration 2:

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 16.52% | 22.69% | 22.94% | 68% | 64% |
| Class A2 | 30.65% | 39.72% | 39.23% | 70% | 65% |
| Overall | 23.58% | 31.21% | 31.09% | 69% | 64% |

## Simulation results (PSNR2) with configuration 1, using selected QPs:

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ECM-3.1 RPR enabling Over ECM-3.1 CTC |  |  |  |  | JVET-Y0128 RPR enabling Over ECM-3.1 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.66% | 3.40% | 5.11% | 71% | 61% | -1.66% | 3.42% | 5.11% | 71% | 61% |
| Class A2 | -0.37% | 15.96% | 10.96% | 79% | 73% | -0.37% | 15.95% | 10.95% | 79% | 73% |
| Overall | -1.01% | 9.68% | 8.03% | 75% | 67% | -1.02% | 9.68% | 8.03% | 75% | 67% |

# Reference

[1] H. Huang, Z. Zhang, V. Seregin, W.-J. Chien, C.-C. Chen, M. Karczewicz, “EE2: Adaptive decoder side motion vector refinement (test 3.4)”, JVET-X0049, Oct. 2021

[2] Z. Zhang, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz, “EE2: Bilateral and template matching AMVP-merge mode (test 3.3)”, JVET-X0083, Oct. 2021

[3] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/47, “Fix RPR_ENABLE for TMVP when all ref are scaled and with X0049, X0083.”

[4] J. Luo, V. Sergin, “JVET functionality confirmation test conditions for reference picture resampling”, JVET-Q2015, Jan. 2020

[5] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

[6] E. Alshina, J. Sauer, “EE1-2.1: Super Resolution with existing VVC functionality”, JVET-Y0061, Jan. 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).