| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0129-v1 |
| --- | --- |

| Title: | Non-EE2: MVD and merge index signaling of AMVP-merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang Han Huang Chun-Chi Chen Vadim Seregin Marta Karczewicz | Tel: Email: | zhizhang@qti.qualcomm.com hanhuang@qti.qualcomm.com chunchic@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to not signal MVD when AMVP-merge mode is used for the current block and template matching is enabled. It also proposes to add one pair of AMVP-merge MVPs. The proposed method was implemented on top of ECM-3.1, and it reports the BD-rate -0.08% (Y), -0.14 (U), -0.12% (V) for random access configuration with 100% encoding and 100% decoding run time compared to ECM3.1.

# Introduction

In ECM3.1 the AMVP part of bilateral matching (BM) AMVP-merge mode [1] is signaled as a regular uni-directional AMVP, i.e., reference index, MVP index and MVD are signaled. When template matching is enabled, the AMVP MVP is first refined by template matching, and then refined by bilateral matching. The final MV of the AMVP part is derived by applying TM refined MVD, BM refined MVD and signal MVD to the AMVP MVP.

The merge index is not signaled, and the merge part of BM AMVP-merge mode selects one merge candidate from the candidate list with smallest template or bilateral matching cost.

# Proposed method

We propose to not signal the MVD when BM AMVP-merge mode is used for the current block and template matching is enabled.

We also propose to add one pair of AMVP-merge MVPs. The merge candidate list is sorted based on the BM cost in increase order. An index (0 or 1) is signaled to indicate which merge candidate in the sorted merge candidate list to use. When there is only one candidate in merge candidate list, the pair of AMVP MVP and merge MVP without bilateral matching MV refinement is padded.

# Simulation results

The proposed method was implemented on top of ECM-3.1. The performance was evaluated using the common test conditions as defined in [3]. We report the test results by using ECM3.1 as anchor:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.23% | -0.25% | 99% | 100% |
| Class A2 | -0.10% | -0.13% | -0.15% | 99% | 100% |
| Class B | -0.07% | -0.17% | -0.06% | 100% | 100% |
| Class C | -0.06% | -0.04% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.14% | -0.12% | 100% | 100% |
| Class D | -0.04% | -0.16% | 0.00% | 100% | 100% |
| Class F (optional) | -0.02% | -0.05% | -0.08% | 100% | 100% |

# Conclusion

In this contribution, BM AMVP-merge mode without MVD signaling and increase the pair of AMVP-merge MVP from one to two are proposed. A coding efficiency of {-0.08%, -0.14%, -0.12%} for YUV in RA configuration has been achieved without additional encoding and decoding time increase. It is suggested to adopt the proposed method into ECM.

# Reference

[1] ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] Z. Zhang, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz, “EE2: Bilateral and template matching AMVP-merge mode (test 3.3)”, JVET-X0083, Oct. 2021.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).