| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0130-v2 |
| --- | --- |

| Title: | EE2-related : Unification of availability check for intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Hyeongmun Jang Junghak Nam Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil, Seocho-gu Seoul 06772 South Korea Seung Hwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this proposal, the unification of usage for neighbor intra modes above the CTU boundary is suggested. Currently, in ECM, the neighbor intra modes can be used in MPM and TIMD process. In VVC, neighboring intra modes above the CTU boundary are not exploited for MPM process to avoid the necessity of storing the intra modes in line buffer memory. However, in ECM, the above mentioned restriction still remains in some parts, but above neighbor intra modes are used as the candidates of the current block for some processes, such as TIMD and in the case of the intra mode propagation from the inter blocks (IPM). Therefore, it is proposed to unify the usage for neighboring intra modes above the CTU boundary. Two unified methods are introduced. In Method 1, the above intra modes in the different CTUs are always restricted as VVC. Method 2 allows to use the intra modes or intra modes from the IPM buffer from the above, above-right, or above-left neighbor blocks, regardless of the CTU boundaries.  The experimental results are shown below.

- Method 1 (to restrict intra modes above the CTU boundary)

  - AI: 0.01% / -0.05% / -0.05% for Y/U/V

  - RA: 0.01 % / 0.04% / 0.11% for Y/U/V

- Method 2 (no restriction for intra modes above the CTU boundary)

  - AI: -0.01% / -0.01% / -0.03% for Y/U/V

  - RA: 0.00% / 0.04% / 0.09% for Y/U/V

Introduction

In the latest video codec standard, VVC, it is not available for the MPM construction to derive intra prediction modes from the above neighbor blocks, if these adjacent blocks are not in the same CTU of the current coding block. This restriction helps to prevent the intra prediction mode from being stored in the line buffer. In ECM, however, the intra modes from the above blocks in the other CTUs are not restricted in some conditions. The cases are listed below:

- Above/Left neighbor intra mode derivation when the current block is rectangular in the MPM process

- Intra modes from the intra mode propagation buffer in the inter blocks in the MPM process

- Above-left and above-right neighbor intra modes when TIMD process

Firstly, in the current MPM construction of ECM, the intra mode in the above neighbor can be considered even though it is from the different CTU when the height of the block is larger than or equal to the width of the block. Moreover, for the intra mode from the left neighbor, it is restricted to exploit when this block is in the different CTU under the same block size condition, which is not desired.

Secondly, the above, above-right and above-left intra modes in the IPM buffer from the other CTUs are not checked at all. If it is allowed for the current block to use the IPM information above the CTU boundary, it is necessary to keep this information in the line buffer memory.

Lastly, only the above intra mode is checked in the TIMD process. The above-right and above-left neighbor modes are not restricted, unlikely the way for the neighbor mode derivation in the MPM construction process.

Therefore, in this proposal, it is suggested to unify the restriction on the derivation of intra modes from the above adjacent blocks in the other CTUs.

Proposed method

Method1: Consistently restrict to refer the above neighbor modes from the different CTU

It is proposed for the MPM or TIMD candidates from the above neighbor blocks to be consistently restricted. For the MPM construction, if the current block which has smaller width than height, the above intra mode is restricted and the left mode can be derived when the neighbor intra modes are from the different CTUs. Then, intra modes of the IPM buffer in the above, above-left, and above-right neighbors are not used as the candidates of MPM if those modes are from the different CTUs. Lastly, the above-right and above-left neighbor intra modes in the TIMD process are not exploited when they are not in the same CTU.

Method2: No restriction for the above neighbor blocks

It is suggested to use the intra modes of the intra or inter neighbor blocks from the above, above-right, or above-left neighbor blocks, regardless of the CTU boundaries. The CTU boundary check conditions for the second candidate, above-left and above-right candidates in the MPM construction process, and the one for the above neighbor candidate of the TIMD process are removed.

Experimental results

The proposed methods are implemented in ECM-3.1. The experiments are conducted as described in the common test conditions [1] with all intra (AI) and random access (RA) configurations.

Unified restriction to the above neighbor modes

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.14% | 0.02% | 98% | 99% |
| Class A2 | -0.01% | 0.06% | 0.03% | 99% | 99% |
| Class B | 0.00% | -0.03% | -0.14% | 99% | 100% |
| Class C | 0.00% | 0.03% | -0.09% | 98% | 99% |
| Class E | 0.04% | -0.22% | -0.01% | 99% | 98% |
| Overall | 0.01% | -0.05% | -0.05% | 98% | 99% |
| Class D | 0.02% | 0.07% | -0.06% | 99% | 99% |
| Class F | 0.03% | 0.04% | -0.06% | 101% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.09% | 99% | 101% |
| Class A2 | 0.00% | 0.06% | 0.07% | 100% | 101% |
| Class B | 0.02% | 0.02% | 0.08% | 99% | 100% |
| Class C | 0.02% | 0.11% | 0.32% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.11% | 99% | 100% |
| Class D | 0.00% | 0.01% | -0.14% | 99% | 99% |
| Class F | 0.02% | 0.19% | 0.02% | 97% | 99% |

No restriction for the above neighbor blocks

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | -0.04% | 98% | 99% |
| Class A2 | -0.01% | 0.02% | 0.05% | 98% | 99% |
| Class B | -0.01% | -0.03% | -0.11% | 98% | 99% |
| Class C | -0.01% | -0.02% | -0.01% | 97% | 98% |
| Class E | 0.00% | -0.09% | 0.03% | 97% | 97% |
| Overall | -0.01% | -0.01% | -0.03% | 98% | 98% |
| Class D | 0.01% | 0.04% | -0.11% | 98% | 98% |
| Class F | 0.01% | 0.15% | 0.01% | 101% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | -0.17% | 98% | 98% |
| Class A2 | -0.02% | 0.01% | -0.06% | 99% | 99% |
| Class B | 0.00% | 0.05% | 0.21% | 100% | 99% |
| Class C | -0.01% | 0.07% | 0.25% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.09% | 99% | 99% |
| Class D | 0.00% | 0.20% | -0.09% | 100% | 100% |
| Class F | 0.02% | 0.01% | -0.25% | 100% | 100% |

Conclusion

It is proposed to unify the restriction of neighbor intra modes when those are in the above CTU boundaries. This method can be less costly when the line buffer is implemented, since the intra modes are not necessary to be stored. Moreover, it is confirmed that the advantage of using the neighbor intra modes above the CTU boundary is negligible. Therefore, it is recommended to restrict the above neighbor intra modes in the different CTUs.

Reference(s)

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-X2017, 24th Meeting, by teleconference, 6–15 October 2021.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).