| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0135-v2 |
| --- | --- |

| Title: | Non-EE2: Template matching based reordering for GPM split modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chun-Chi Chen, 2Han Huang, Yan Zhang, Zhi Zhang, Yao-Jen Chang, 3Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | 1chunchic@qti.qualcomm.com 2hanhuang@qti.qualcomm.com 3vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the result of a reordering method for GPM split modes using template matching (TM) cost. The method computes the TM cost of each GPM split mode with the GPM motion information of the current block, and then reorders all GPM split modes in ascending order through their respective TM costs. An index is signaled using Golomb-Rice code to indicate the use of split mode in the list of reordered GPM split modes. Compared with ECM-3.1, it is reported that the contribution achieves BD-rate saving of {0.21% (Y), 0.27% (U), 0.24% (V)} for Random Access and {0.29% (Y), 0.31% (U), 0.25% (V)} for Low-delay B.

# Introduction

In VVC spec (JVET-T2001) and ECM-3.1, there are 64 GPM split modes and the use of split mode of each GPM coding unit (CU) is signaled using fixed-length binary code. This coding method could imply that all GPM split modes are treated as equal-probable events, and thus a fixed-length code could be used accordingly for signaling.

Alternatively, this contribution infers the respective template matching (TM) cost of each GPM split mode, trying to rank the effectiveness of each GPM split mode for reordering. In the reordering process, a split mode with a lower TM cost could be assigned to a smaller index, while another one with a relatively higher TM cost could be assigned to a larger index. Accordingly, GPM split modes are no longer treated as equal-probable events after reordering, and a variable-length code could thus be applied to signaling the use of aforementioned index of each GPM split mode. As compared with ECM-3.1 under common test conditions of Exploration Experiments 2 (JVET-X2024), the proposed method could achieve BD-rate saving of {0.21% (Y), 0.27% (U), 0.24% (V)} for Random Access and {0.29% (Y), 0.31% (U), 0.25% (V)} for Low-delay B.

# Proposed reordering method

The proposed reordering method for GPM split modes is a two-step process after the respective reference templates of the two GPM partitions in a coding unit are generated, as follows:

- blending the reference templates of the two GPM partitions using the respective weights of split modes (i.e., resulting in 64 blended reference templates) and computing the respective TM costs of these blended reference templates;

- reordering the TM costs in ascending order and marking the best N candidates as available split modes.

The edge on the template is extended from that of the current CU, as Figure 1 illustrates. The corresponding weights used in the blending process of templates are computed similar to the GPM weight derivation process (i.e., subclause 8.5.7.2 in VVC spec JVET-T2001). The only difference is as follows:

- the sample positions (relative to the original of the CU) on the template are used to derive weights;

- weights are mapped to 0 and 8 before use depending on whichever is closer and thus the edge on templates is clean cut for computational simplification in the blending process of templates.

- The edge on templates

After ascending reordering using TM cost, the best N GPM split modes are assigned to their respective indices, according to their TM cost from small to large. Golomb-Rice code is used to signal this index.

# Test results

The proposed method was implemented on top of the ECM-3.1 software and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-X2024). The test results are summarized below.

- Performance results of proposed method relative to ECM-3.1 anchor.

|  | Random Access Main 10 |  |  |  |  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.33% | -0.33% | 102% | 101% | x | x | x | x | x |
| Class A2 | -0.23% | -0.29% | -0.33% | 102% | 100% | x | x | x | x | x |
| Class B | -0.16% | -0.27% | -0.11% | 102% | 101% | -0.22% | -0.26% | -0.24% | 101% | 100% |
| Class C | -0.28% | -0.21% | -0.27% | 103% | 101% | -0.39% | -0.65% | -0.51% | 101% | 99% |
| Class E | x | x | x | X | X | -0.27% | 0.06% | 0.08% | 101% | 99% |
| Overall | -0.21% | -0.27% | -0.24% | 102% | 101% | -0.29% | -0.31% | -0.25% | 101% | 99% |
| Class D | -0.17% | -0.18% | -0.12% | 104% | 102% | -0.20% | -0.46% | -0.30% | 101% | 100% |
| Class F | -0.16% | -0.29% | -0.19% | 102% | 101% | -0.26% | -0.40% | 0.04% | 102% | 100% |

# Conclusion

This contribution reports test results of a template matching based reordering method for GPM split modes. It also shows the effectiveness of the proposed method in improving the coding performance of ECM. Accordingly, it is suggested to further study the proposed method in the context of an EE.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).