| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0137-v1 |
| --- | --- |

| Title: | EE2-4.2: Enhanced sign prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Ning Yan, C.-W. Kuo, Hong-Jheng Jhu, W. Chen, Xianglin Wang | Email: | {xiaoyuxiu, yiwenchen, yanning03, cheweikuo, jhuhong-jheng chenwei06, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-4.2 on enhanced sign prediction. In the EE test, the sign prediction tool in ECM-3.1 is further enhanced by the following two methods: 1) adaptive selection of transform coefficient positions for the sign prediction by maximizing their impacts on reconstructed border samples; 2) extension of the sign prediction to LFNST mode. There are two sub-tests in EE2-4.2. The performance of adaptive selection of transform coefficient is investigated in EE2-4.2a while EE2-4.2b provides the results of the combination of adaptive coefficient selection and extension of sign prediction to LFNST. Simulation results are summarized as follows:

|  | AI |  |  | RA |  |  |  | LD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-BD | Enc | Dec | Y-BD | Enc | Dec | Y-BD |  | Enc | Dec |
| EE2-4.2a | -0.07% | 102% | 101% | -0.09% | 100% | 100% | -0.08% |  | 100% | 100% |
| EE2-4.2b | -0.31% | 103% | 104% | -0.22% | 101% | 100% | -0.11% |  | 100% | 100% |

# Proposal

In JVET-X0150, two enhancements are proposed further improve the coding efficiency of the sign prediction tool in the ECM:

- Instead of selecting the first M coefficients in raster-scan order, it is proposed to choose the non-zero transform coefficients based on their impacts on the quality of the reconstructed border samples of one TB. Specifically, to select the transform coefficients, the following cost function is applied which measures the L1-norm energy of the variations caused by one transform coefficient on the reconstructed border samples:

cost=Ci,j∙k=0N-1Ti,jk

where Ci,j represents the transform coefficient at coordinate i,j in the TB; Ti,j represents the template that corresponds to the transform coefficient, which consists of N reconstructed samples at the top and left border of the TB.

- It is proposed to extend the sign prediction to the LFNST mode. Additionally, to control the computational complexity, a maximum of 4 coefficients in the top-left CG are allowed to be predicted for one LFNST TB.

At JVET-X meeting, the above two methods were included in exploration experiment (EE)2-4.2 for further evaluation.

# Experimental results

The proposed methods are implemented on top of the reference software ECM-3.1 [1] and tested using the common test conditions as specified in [2]. There are two sub-tests in EE2-4.2. The performance of adaptive selection of transform coefficient is investigated in EE2-4.2a while EE2-4.2b provides the results of the combination of adaptive coefficient selection and extension of sign prediction to LFNST. Table 1 and Table 2 illustrates the corresponding BD-rate performance of EE2-4.2a and EE2-4.2b, respectively.

Table 1 The BD-rate performance of EE2-4.2a, adaptive coefficient selection

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.03% | -0.01% | 101% | 102% |
| Class A2 | -0.09% | 0.02% | -0.07% | 103% | 100% |
| Class B | -0.08% | -0.13% | -0.04% | 103% | 101% |
| Class C | -0.06% | -0.10% | -0.01% | 102% | 101% |
| Class E | -0.06% | -0.08% | -0.03% | 102% | 102% |
| Overall | -0.07% | -0.07% | -0.03% | 102% | 101% |
| Class D | -0.03% | 0.09% | -0.20% | 101% | 102% |
| Class F | -0.03% | 0.04% | 0.05% | 102% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.18% | -0.22% | 104% | 99% |
| Class A2 | -0.10% | -0.08% | 0.02% | 98% | 98% |
| Class B | -0.08% | -0.12% | 0.00% | 100% | 99% |
| Class C | -0.04% | 0.00% | 0.19% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.09% | 0.01% | 100% | 99% |
| Class D | -0.02% | 0.33% | -0.25% | 98% | 100% |
| Class F | -0.10% | -0.05% | -0.27% | 98% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.50% | -0.15% | 102% | 98% |
| Class C | -0.09% | 0.15% | 0.01% | 98% | 101% |
| Class E | -0.03% | -0.54% | -1.24% | 100% | 100% |
| Overall | -0.08% | -0.29% | -0.37% | 100% | 100% |
| Class D | 0.01% | -0.66% | -0.05% | 98% | 103% |
| Class F | 0.25% | -0.33% | -0.67% | 103% | 100% |

Table 2 The BD-rate performance of EE2-4.2b, adaptive coefficient selection + extension of sign prediction to LFNST

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.26% | -0.35% | 106% | 103% |
| Class A2 | -0.28% | -0.12% | -0.15% | 107% | 103% |
| Class B | -0.29% | -0.17% | -0.23% | 101% | 104% |
| Class C | -0.26% | -0.09% | -0.09% | 103% | 104% |
| Class E | -0.41% | -0.29% | -0.30% | 100% | 104% |
| Overall | -0.31% | -0.18% | -0.21% | 103% | 104% |
| Class D | -0.22% | 0.01% | -0.31% | 102% | 110% |
| Class F | -0.15% | -0.03% | -0.05% | 105% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.27% | -0.44% | 100% | 100% |
| Class A2 | -0.21% | -0.06% | -0.18% | 101% | 99% |
| Class B | -0.22% | -0.10% | -0.11% | 101% | 100% |
| Class C | -0.21% | -0.17% | 0.11% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.14% | -0.13% | 101% | 100% |
| Class D | -0.13% | -0.01% | -0.12% | 103% | 102% |
| Class F | -0.12% | -0.10% | -0.29% | 102% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.37% | -0.06% | 100% | 101% |
| Class C | -0.13% | -0.17% | -0.19% | 100% | 100% |
| Class E | -0.09% | -0.28% | -0.73% | 100% | 98% |
| Overall | -0.11% | -0.28% | -0.27% | 100% | 100% |
| Class D | -0.04% | -1.01% | 0.26% | 102% | 98% |
| Class F | 0.01% | -0.87% | -0.23% | 101% | 99% |

# Conclusion

This contribution reports the results of EE2-4.2 on enhanced sign prediction which includes two aspects of modifications. In the first aspect, adaptive selection of transform coefficient positions for the sign prediction is proposed by maximizing their impacts on reconstructed border samples. In the second aspect, it is proposed to extend the sign prediction to LFNST mode. Simulation results demonstrates the combination of the two modifications provides interesting BD-rate gains for all coding configurations with negligible encoding and decoding run-time impacts. It is recommended to adopt EE2-4.3b into the next version of ECM software.

# References

ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct., 2021.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).