| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0138-v1 |
| --- | --- |

| Title: | EE2-4.3: Combined Test Results of EE2-4.1 and EE2-4.2 on Sign Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Yan Ye Ru-Ling Liao Xinwei Li Xiaoyu Xiu, Yi-Wen Chen, Ning Yan, C.-W. Kuo, Hong-Jheng Jhu, W. Chen, Xianglin Wang | Tel: Email: | jiechen.cj@alibaba-inc.com xiaoyuxiu@kwai.com |
| Source: | Alibaba Group. Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports the combined test results of test 4.3 of EE2. Two combined tests are conducted in test 4.3. Test 4.3.a is on top of test 4.1.c[1] and additionally applies sign prediction to LFNST block. Test 4.3.b is on top of test 4.2.b[2] and additionally extends sign prediction area to maximum 32x32 and the area is signalled in SPS. Following results are reported:

- Test 4.3.a

  - AI: -0.35% (Y), -0.22%(U), -0.22%(V), 107%(EncTime), 108% (DecTime)

  - RA: -0.0x% (Y), -0.0x%(U), -0.0x%(V), 10x%(EncTime), 10x% (DecTime)

  - LB: -0.20% (Y), -0.22%(U), -0.05%(V), 102%(EncTime), 102% (DecTime)

- Test 4.3.b

  - AI: -0.34% (Y), -0.24%(U), -0.22%(V), 105%(EncTime), 105% (DecTime)

  - RA: -0.28% (Y), -0.16%(U), -0.17%(V), 102%(EncTime), 100% (DecTime)

  - LB: -0.21% (Y), -0.28%(U), -0.33%(V), 101%(EncTime), 100% (DecTime)

Introduction

In ECM2.0, signs of some transform coefficient levels are encoded using a system of prediction. A sign that has been predicted is no longer equiprobable (EP) signaled in the bitstream, but is replaced by a “residue” (signaled using an associated CABAC context) which indicates if the prediction was correct or not. In ECM2.0, at most the first M (which is set to 8) non-zero coefficients in the raster-scan order of the top-left 4x4 area of a TB are selected for the sign prediction.

In last meeting, JVET-X0120[3] and JVET-X0150[4] were proposed to improve the sign prediction in ECM2.0 and both of these two contributions were included EE2 for further study. JVET-X0120 proposed to 1) adaptively select the signs to be predicted based on the absolute value of qIdx (qIdx is the transform coefficient level after compensating the impact of the multiple quantizers in DQ) and 2) extend the area for sign prediction from 4x4 to maximum 32x32. JVET-X0150 proposed to 1) adaptively select the signs to be predicted based on their impacts on the quality of the reconstructed border samples of one TB and 2) also applied sign prediction to LFNST blocks.

Test 4.3.a

Test 4.3.a is on top of test 4.1.c and additionally applies sign prediction to LFNST blocks. So test 4.3.a includes the following three improvements:

- The transform coefficients with the largest K qIdx value are selected for sign prediction. qIdx value is the transform coefficient level after compensating the impact from the multiple quantizers in DQ. A larger qIdx value will produce a larger de-quantized transform coefficient level.

- The sign prediction area is extended. The signs of top-left MxN block are predicted. The value of M and N is computed as M=minw, 32, N=min⁡(h, 32), where w and h are the width and height of the transform block

- The sign prediction is also applied to LFNST block. And for LFNST block, a maximum of 4 coefficients in the top-left 4x4 area are allowed to be sign predicted.

Test 4.3.b

Test 4.3.b is on top of test 4.2.b and additionally extends the sign prediction area to maximum 32x32. Specifically, test 4.3.c includes the following three improvements:

- Adaptively select the signs to be predicted based on their impacts on the reconstructed border samples of one TB.

- The sign prediction is applied to LFNST blocks and for LFNST block, a maximum of 4 coefficients in the top-left 4x4 area are allowed to be sign predicted.

- The maximum area for sign prediction is extended. Encoder selects the maximum area size from four allowed values 4, 8, 16 and 32 based on configuration, sequence class and QP, and signaled it in SPS.

Simulation results

The two combined tests are implemented on top of ECM3.1. The performance is evaluated based on EE2 test conditions suggested in JVET-X2017[5]. The following results are reported.

The simulation results are shown in Table 1, Table 2.

Table 1: Results of test 4.3.a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.24% | -0.28% | 110% | 106% |
| Class A2 | -0.33% | -0.13% | -0.17% | 110% | 107% |
| Class B | -0.35% | -0.18% | -0.20% | 106% | 109% |
| Class C | -0.28% | -0.19% | -0.12% | 105% | 109% |
| Class E | -0.46% | -0.41% | -0.37% | 103% | 106% |
| Overall | -0.35% | -0.22% | -0.22% | 107% | 108% |
| Class D | -0.27% | 0.02% | -0.28% | 106% | 112% |
| Class F | -0.17% | 0.06% | -0.16% | 107% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.07% | -0.52% | 103% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.29% | -0.17% | -0.21% | 103% | 102% |
| Class C | -0.21% | -0.09% | 0.17% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.24% | 0.39% | -0.11% | 103% | 101% |
| Class F | -0.21% | -0.12% | -0.34% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -0.35% | 0.16% | 103% | 103% |
| Class C | -0.22% | -0.45% | 0.08% | 101% | 102% |
| Class E | -0.19% | 0.32% | -1.39% | 102% | 101% |
| Overall | -0.20% | -0.22% | -0.25% | 102% | 102% |
| Class D | -0.04% | -0.19% | -0.19% | 100% | 102% |
| Class F | 0.02% | 0.18% | -0.24% | 103% | 101% |

Table 2: Results of test 4.3.b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.27% | -0.28% | 108% | 104% |
| Class A2 | -0.32% | -0.22% | -0.19% | 109% | 104% |
| Class B | -0.33% | -0.20% | -0.23% | 102% | 106% |
| Class C | -0.28% | -0.12% | -0.14% | 105% | 107% |
| Class E | -0.45% | -0.44% | -0.31% | 101% | 105% |
| Overall | -0.34% | -0.24% | -0.22% | 105% | 105% |
| Class D | -0.25% | -0.20% | -0.23% | 103% | 109% |
| Class F | -0.11% | 0.13% | -0.01% | 106% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.37% | -0.40% | 100% | 100% |
| Class A2 | -0.32% | -0.10% | -0.15% | 103% | 99% |
| Class B | -0.28% | -0.19% | -0.18% | 101% | 100% |
| Class C | -0.22% | 0.01% | 0.02% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.16% | -0.17% | 102% | 100% |
| Class D | -0.16% | 0.21% | -0.18% | 103% | 102% |
| Class F | -0.17% | -0.15% | -0.45% | 103% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -0.52% | -0.29% | 102% | 101% |
| Class C | -0.22% | -0.50% | 0.20% | 101% | 100% |
| Class E | -0.23% | 0.40% | -1.12% | 98% | 98% |
| Overall | -0.21% | -0.28% | -0.33% | 101% | 100% |
| Class D | -0.03% | -0.17% | -0.58% | 99% | 98% |
| Class F | -0.01% | -0.88% | 0.08% | 100% | 99% |

Conclusion

This contribution reports the combined test results of EE2-4.3. Two tests are conducted in this contribution. Both of these two tests adopt an adaptive selection of signs to be predicted, extend the sign prediction area and additionally applies sign prediction on LFNST blocks. With these improvements, more than 0.3% luma coding gain in AI and more than 0.2% luma coding gain in RA are observed with not much running time increase. Thus, it is suggested to adopt one of the combined tests into next version of ECM software.

References

J. Chen, Y. Ye, R.-L. Liao, X. Li, “EE2-4-1:Test results on Sign Prediction improvement”, JVET-Y0138, Teleconference, Jan. 2022 update later

X. Xiu, Y.-W. Chen, N. Yan, C.-W. Kuo, H.-J. Jhu, W. Chen, X. Wang, “EE2-4.2: Enhanced sign prediction”, JVET-Y0137, Teleconference, Jan. 2022.

M. G. Sarwer, Y. Ye, J. Chen, R.-L, Liao, “AHG12: On sign prediction”, JVET-X0120, Teleconference, Oct 2021.

X. Xiu, J.-H. Jhu, W. Chen, C.-W. Kuo, N. Yan, Y.-W. Chen, X. Wang, “AHG12: Teleconference, Enhanced sign prediction”, JVET-X0150, Oct. 2021.

M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, Teleconference, JVET-X2017, Oct. 2021

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).