| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0141-v2 |
| --- | --- |

| Title: | EE2-4.3 related: More combined test results for sign prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Yan Ye Ru-Ling Liao Xinwei Li Xiaoyu Xiu, Yi-Wen Chen, Ning Yan, C.-W. Kuo, Hong-Jheng Jhu, W. Chen, Xianglin Wang | Tel: Email: | jiechen.cj@alibaba-inc.com xiaoyuxiu@kwai.com |
| Source: | Alibaba Group. Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports more combined test results for sign prediction. EE2-4.3.a[1] combines three improvements for sign prediction proposed in last meeting: 1) the signs to be predicted are selected based on the value of qIdx; 2) the area of the sign prediction is extended to maximum 32x32; 3) sign prediction is applied to LFNST block. To reduce the complexity of EE2-4.3.a, two more combined tests are conducted on top of EE2-4.3.a in this contribution. In test 1, maximum area for sign prediction is not always set to 32x32 but is set by the encoder and signaled in SPS; in test 2, the sign prediction area is not extended (keep 4x4 as the anchor). Results of following two tests are reported:

- Test 1: adaptive maximum sign prediction area + selection of signs to be predicted based on qIdx + apply sign prediction to LFNST blocks

  - AI: -0.33% (Y), -0.20%(U), -0.22%(V), 105%(EncTime), 106% (DecTime)

  - RA: -0.29% (Y), -0.11%(U), -0.19%(V), 102%(EncTime), 100% (DecTime)

  - LB: -0.22% (Y), -0.17%(U), -0.15%(V), 101%(EncTime), 101% (DecTime)

- Test 2: selection of signs to be predicted based on qIdx + apply sign prediction to LFNST blocks

  - AI: -0.31% (Y), -0.17%(U), -0.18%(V), 103%(EncTime), 105% (DecTime)

  - RA: -0.22% (Y), -0.03%(U), -0.16%(V), 101%(EncTime), 100% (DecTime)

  - LB: -0.10% (Y), -0.07%(U), -0.26%(V), 101%(EncTime), 102% (DecTime)

In v2 version, full results are provided.

Introduction

JVET-X0120[2] and JVET-X0150[3] were proposed to improve the sign prediction in ECM2.0 in last meeting. JVET-X0120 proposed to 1) adaptively select the signs to be predicted based on the absolute value of qIdx (qIdx is the transform coefficient level after compensating the impact of the multiple quantizers in DQ) and 2) extend the area for sign prediction from 4x4 to maximum 32x32. JVET-X0150 proposed to 1) adaptively select the signs to be predicted based on their impacts on the quality of the reconstructed border samples of one TB and 2) also applied sign prediction to LFNST blocks.

Both of these two contribution were included in EE2 for further study. Two combined tests were included in test 4.3[1]. In combined test 4.3.a, the three improvements proposed were combined and tested: 1) the signs to be predicted are selected based on the value of qIdx; 2) the area of the sign prediction is extended to maximum 32x32; 3) sign prediction is applied to LFNST block. Test 4.3.a achieves quite interesting coding gain in AI, RA and LD, but with some increase of running time, especially in AI. To reduce the complexity of test 4.3.a and make a better trade off, two more tests are conducted in this contribution.

Test 4.3.a

Test 4.3.a includes three improvements of sign prediction as follows

- The transform coefficients with the largest K qIdx value are selected for sign prediction. qIdx value is the transform coefficient level after compensating the impact from the multiple quantizers in DQ. A larger qIdx value will produce a larger de-quantized transform coefficient level.

- The sign prediction area is extended. The signs of top-left MxN block are predicted. The value of M and N is computed as M=minw, 32, N=min⁡(h, 32), where w and h are the width and height of the transform block

- The sign prediction was also applied to LFNST block. And for LFNST block, a maximum of 4 coefficients in the top-left 4x4 area are allowed to be sign predicted.

Test 1: adaptive sign prediction area setting

In test 1, the maximum area for sign prediction is not always set to 32x32. Encoder sets the maximum area based on configuration, sequence class and QP, and signaled the area in SPS which align with EE2-4.3.b. The range of maximum area is from 4x4 to 32x32. The other two improvements (selecting the signs to be predicted based on qIdx and applying sign prediction to LFNST block) are the same as EE2-4.3.a.

Test 2: keep sign prediction area as 4x4

To further reduce the running time, in test 2, the area of sign prediction is not extended but keep 4x4 as the anchor. So only two improvements (selecting the signs to be predicted based on qIdx and applying sign prediction to LFNST block) which are the same as EE2-4.3.a are included in test 2.

Simulation results

The tests are implemented on top of ECM3.1. The performance is evaluated based on EE2 test conditions suggested in JVET-X2017[4]. Following two tests are conducted.

- Test 1: adaptive maximum sign prediction area + selection of signs to be predicted based on qIdx + apply sign prediction to LFNST blocks

- Test 2: selection of signs to be predicted based on qIdx + apply sign prediction to LFNST blocks

The simulation results are shown in Table 1, Table 2.

Table 1: Results of test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.24% | -0.36% | 108% | 103% |
| Class A2 | -0.32% | -0.10% | -0.18% | 108% | 105% |
| Class B | -0.33% | -0.22% | -0.15% | 104% | 107% |
| Class C | -0.28% | -0.14% | -0.11% | 104% | 109% |
| Class E | -0.43% | -0.31% | -0.36% | 102% | 105% |
| Overall | -0.33% | -0.20% | -0.22% | 105% | 106% |
| Class D | -0.25% | -0.11% | -0.41% | 103% | 112% |
| Class F | -0.20% | -0.02% | -0.14% | 105% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.18% | -0.49% | 102% | 100% |
| Class A2 | -0.31% | -0.01% | -0.13% | 102% | 100% |
| Class B | -0.28% | -0.13% | -0.21% | 102% | 101% |
| Class C | -0.24% | -0.09% | 0.02% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.11% | -0.19% | 102% | 100% |
| Class D | -0.18% | 0.21% | -0.14% | 103% | 101% |
| Class F | -0.22% | -0.07% | -0.33% | 102% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.30% | 0.14% | 101% | 102% |
| Class C | -0.22% | -0.45% | 0.08% | 102% | 102% |
| Class E | -0.24% | 0.43% | -0.94% | 100% | 100% |
| Overall | -0.22% | -0.17% | -0.15% | 101% | 101% |
| Class D | -0.09% | 0.41% | -0.49% | 102% | 102% |
| Class F | -0.15% | -0.42% | -0.15% | 101% | 100% |

Table 2: Results of test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.21% | -0.32% | 107% | 103% |
| Class A2 | -0.29% | -0.12% | -0.15% | 107% | 103% |
| Class B | -0.30% | -0.14% | -0.12% | 102% | 106% |
| Class C | -0.28% | -0.10% | -0.10% | 103% | 108% |
| Class E | -0.39% | -0.31% | -0.24% | 100% | 105% |
| Overall | -0.31% | -0.17% | -0.18% | 103% | 105% |
| Class D | -0.23% | 0.07% | -0.35% | 102% | 111% |
| Class F | -0.17% | 0.00% | -0.30% | 104% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.02% | -0.55% | 101% | 100% |
| Class A2 | -0.24% | -0.10% | -0.13% | 101% | 100% |
| Class B | -0.22% | 0.01% | -0.18% | 101% | 100% |
| Class C | -0.17% | -0.05% | 0.12% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.03% | -0.16% | 101% | 100% |
| Class D | -0.17% | 0.01% | -0.20% | 101% | 101% |
| Class F | -0.17% | -0.21% | -0.38% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.04% | 0.21% | 101% | 102% |
| Class C | -0.14% | 0.27% | 0.20% | 100% | 102% |
| Class E | -0.07% | -0.72% | -1.65% | 100% | 100% |
| Overall | -0.10% | -0.07% | -0.26% | 101% | 102% |
| Class D | 0.00% | 0.15% | -0.46% | 100% | 101% |
| Class F | 0.13% | 0.08% | 0.43% | 99% | 100% |

Conclusion

This contribution reports results of two more combined tests on sign prediction improvement. Compared with test 4.3.a of EE2, test 1 uses adaptive maximum area for sign prediction and test 2 keeps the maximum area of sign prediction as 4x4. Both of these two tests reduce the running time compared with test 4.3.a with negligible coding gain drop. Based on the coding performance and the complexity, it is suggested to adopt one of the tests into next version of ECM software.

References

J. Chen, Y. Yan, R.-L Liao, X. Li, X. Xiu, Y.-W. Chen, N. Yan, C.-W. Kuo, H.-J. Jhu, W. Chen, X. Wang, “EE2-4.3: Combined Test Results of EE2-4.1 and EE2-4.2 on sign prediction”, JVET-Y0138, Jan. 2022

M. G. Sarwer, Y. Ye, J. Chen, R.-L, Liao, “AHG12: On sign prediction”, JVET-X0120, Oct 2021.

X. Xiu, J.-H. Jhu, W. Chen, C.-W. Kuo, N. Yan, Y.-W. Chen, X. Wang, “AHG12: Enhanced sign prediction”, JVET-X0150, Oct. 2021.

M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).