| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0141-v3 |
| --- | --- |

| Title: | EE2-4.3 related: More combined test results for sign prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Yan Ye Ru-Ling Liao Xinwei Li Xiaoyu Xiu, Yi-Wen Chen, Ning Yan, C.-W. Kuo, Hong-Jheng Jhu, W. Chen, Xianglin Wang | Tel: Email: | jiechen.cj@alibaba-inc.com xiaoyuxiu@kwai.com |
| Source: | Alibaba Group. Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports more combined test results for sign prediction. EE2-4.3.a[1] combines three improvements for sign prediction proposed in last meeting: 1) the signs to be predicted are selected based on the value of qIdx; 2) the area of the sign prediction is extended to maximum 32x32; 3) sign prediction is applied to LFNST block. To reduce the complexity of EE2-4.3.a, two more combined tests are conducted on top of EE2-4.3.a in this contribution. In test 1, maximum area for sign prediction is not always set to 32x32 but is set by the encoder and signaled in SPS; in test 2, the sign prediction area is not extended (keep 4x4 as the anchor). Results of following two tests are reported:

- Test 1: adaptive maximum sign prediction area + selection of signs to be predicted based on qIdx + apply sign prediction to LFNST blocks

  - AI: -0.33% (Y), -0.20%(U), -0.22%(V), 105%(EncTime), 106% (DecTime)

  - RA: -0.29% (Y), -0.11%(U), -0.19%(V), 102%(EncTime), 100% (DecTime)

  - LB: -0.22% (Y), -0.17%(U), -0.15%(V), 101%(EncTime), 101% (DecTime)

- Test 2: selection of signs to be predicted based on qIdx + apply sign prediction to LFNST blocks

  - AI: -0.31% (Y), -0.17%(U), -0.18%(V), 103%(EncTime), 105% (DecTime)

  - RA: -0.22% (Y), -0.03%(U), -0.16%(V), 101%(EncTime), 100% (DecTime)

  - LB: -0.10% (Y), -0.07%(U), -0.26%(V), 101%(EncTime), 102% (DecTime)

In v2 version, full results are provided.

In v3 version, based on the meeting discussion, a new combined test (test 3) are provided to address the issues proposed by the experts during the EE2 review. The following results are reported for test 3

  - AI: -0.31% (Y), -0.21%(U), -0.18%(V), 104%(EncTime), 104% (DecTime)

  - RA: -0.2x% (Y), -0.x%(U), -0.x%(V), 10x%(EncTime), 10x% (DecTime)

  - LB: -0.2x% (Y), -0.x%(U), -0.x%(V), 10x%(EncTime), 10x% (DecTime)

Introduction

JVET-X0120[2] and JVET-X0150[3] were proposed to improve the sign prediction in ECM2.0 in last meeting. JVET-X0120 proposed to 1) adaptively select the signs to be predicted based on the absolute value of qIdx (qIdx is the transform coefficient level after compensating the impact of the multiple quantizers in DQ) and 2) extend the area for sign prediction from 4x4 to maximum 32x32. JVET-X0150 proposed to 1) adaptively select the signs to be predicted based on their impacts on the quality of the reconstructed border samples of one TB and 2) also applied sign prediction to LFNST blocks.

Both of these two contribution were included in EE2 for further study. Two combined tests were included in test 4.3[1]. In combined test 4.3.a, the three improvements proposed were combined and tested: 1) the signs to be predicted are selected based on the value of qIdx; 2) the area of the sign prediction is extended to maximum 32x32; 3) sign prediction is applied to LFNST block. Test 4.3.a achieves quite interesting coding gain in AI, RA and LD, but with some increase of running time, especially in AI. To reduce the complexity of test 4.3.a and make a better trade off, two more tests are conducted in this contribution.

Test 4.3.a

Test 4.3.a includes three improvements of sign prediction as follows

- The transform coefficients with the largest K qIdx value are selected for sign prediction. qIdx value is the transform coefficient level after compensating the impact from the multiple quantizers in DQ. A larger qIdx value will produce a larger de-quantized transform coefficient level.

- The sign prediction area is extended. The signs of top-left MxN block are predicted. The value of M and N is computed as M=minw, 32, N=min⁡(h, 32), where w and h are the width and height of the transform block

- The sign prediction was also applied to LFNST block. And for LFNST block, a maximum of 4 coefficients in the top-left 4x4 area are allowed to be sign predicted.

Test 1: adaptive sign prediction area setting

In test 1, the maximum area for sign prediction is not always set to 32x32. Encoder sets the maximum area based on configuration, sequence class and QP, and signaled the area in SPS which align with EE2-4.3.b. The range of maximum area is from 4x4 to 32x32. The other two improvements (selecting the signs to be predicted based on qIdx and applying sign prediction to LFNST block) are the same as EE2-4.3.a.

Test 2: keep sign prediction area as 4x4

To further reduce the running time, in test 2, the area of sign prediction is not extended but keep 4x4 as the anchor. So only two improvements (selecting the signs to be predicted based on qIdx and applying sign prediction to LFNST block) which are the same as EE2-4.3.a are included in test 2.

Test 3: a combined test of Test 1 and EE2-4.3b

In test 1, the qIdx based sign selection is performed in the parsing process as the decoder needs to know which transform coefficients are sign predicted and the order of these signs to be predicted when parsing the sign pred flag for these transform coefficients. It gives a burden to the entropy decoder.

In EE2-4.3b[1], although the sign selection is performed in the reconstruction process, the selection is based on the sum of the boundary reconstructed residuals. So the decoder has to calculate the templates for all the transform coefficients before determining which transform coefficients to be sign predicted. It introduces a lot of amount of computation.

Thus, a combination of test 1 and EE2-4.3b are provided in this contribution as Test 3 to further simplify the design. In test 3, the context modelling of sign pred flag is no longer based on the transform coefficient for which the sign pred flag is being parsed, and by doing this, the sign selection is moved to the reconstruction process as how it does in EE2-4.3b. Moreover, the sign selection is still based on qIdx as that in test 1 to so that there is no need to calculate the templates for all the coefficients. So the above mentioned issues are addressed in test 3.

Simulation results

The tests are implemented on top of ECM3.1. The performance is evaluated based on EE2 test conditions suggested in JVET-X2017[4]. Following two tests are conducted.

- Test 1: adaptive maximum sign prediction area + selection of signs to be predicted based on qIdx + apply sign prediction to LFNST blocks

- Test 2: selection of signs to be predicted based on qIdx + apply sign prediction to LFNST blocks

- Test 3: Test 1 + EE2-4.3b

The simulation results are shown in Table 1, Table 2 and Table 3.

Table 1: Results of test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.24% | -0.36% | 108% | 103% |
| Class A2 | -0.32% | -0.10% | -0.18% | 108% | 105% |
| Class B | -0.33% | -0.22% | -0.15% | 104% | 107% |
| Class C | -0.28% | -0.14% | -0.11% | 104% | 109% |
| Class E | -0.43% | -0.31% | -0.36% | 102% | 105% |
| Overall | -0.33% | -0.20% | -0.22% | 105% | 106% |
| Class D | -0.25% | -0.11% | -0.41% | 103% | 112% |
| Class F | -0.20% | -0.02% | -0.14% | 105% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.18% | -0.49% | 102% | 100% |
| Class A2 | -0.31% | -0.01% | -0.13% | 102% | 100% |
| Class B | -0.28% | -0.13% | -0.21% | 102% | 101% |
| Class C | -0.24% | -0.09% | 0.02% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.11% | -0.19% | 102% | 100% |
| Class D | -0.18% | 0.21% | -0.14% | 103% | 101% |
| Class F | -0.22% | -0.07% | -0.33% | 102% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.30% | 0.14% | 101% | 102% |
| Class C | -0.22% | -0.45% | 0.08% | 102% | 102% |
| Class E | -0.24% | 0.43% | -0.94% | 100% | 100% |
| Overall | -0.22% | -0.17% | -0.15% | 101% | 101% |
| Class D | -0.09% | 0.41% | -0.49% | 102% | 102% |
| Class F | -0.15% | -0.42% | -0.15% | 101% | 100% |

Table 2: Results of test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.21% | -0.32% | 107% | 103% |
| Class A2 | -0.29% | -0.12% | -0.15% | 107% | 103% |
| Class B | -0.30% | -0.14% | -0.12% | 102% | 106% |
| Class C | -0.28% | -0.10% | -0.10% | 103% | 108% |
| Class E | -0.39% | -0.31% | -0.24% | 100% | 105% |
| Overall | -0.31% | -0.17% | -0.18% | 103% | 105% |
| Class D | -0.23% | 0.07% | -0.35% | 102% | 111% |
| Class F | -0.17% | 0.00% | -0.30% | 104% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.02% | -0.55% | 101% | 100% |
| Class A2 | -0.24% | -0.10% | -0.13% | 101% | 100% |
| Class B | -0.22% | 0.01% | -0.18% | 101% | 100% |
| Class C | -0.17% | -0.05% | 0.12% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.03% | -0.16% | 101% | 100% |
| Class D | -0.17% | 0.01% | -0.20% | 101% | 101% |
| Class F | -0.17% | -0.21% | -0.38% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.04% | 0.21% | 101% | 102% |
| Class C | -0.14% | 0.27% | 0.20% | 100% | 102% |
| Class E | -0.07% | -0.72% | -1.65% | 100% | 100% |
| Overall | -0.10% | -0.07% | -0.26% | 101% | 102% |
| Class D | 0.00% | 0.15% | -0.46% | 100% | 101% |
| Class F | 0.13% | 0.08% | 0.43% | 99% | 100% |

Table 3: Results of test 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.27% | -0.32% | 104% | 106% |
| Class A2 | -0.29% | -0.16% | -0.13% | 104% | 106% |
| Class B | -0.30% | -0.18% | -0.16% | 105% | 106% |
| Class C | -0.26% | -0.15% | -0.07% | 104% | 107% |
| Class E | -0.42% | -0.30% | -0.27% | 102% | 104% |
| Overall | -0.31% | -0.21% | -0.18% | 104% | 106% |
| Class D | -0.23% | -0.02% | -0.35% | 104% | 108% |
| Class F | -0.15% | 0.09% | -0.11% | 105% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.28% | -0.09% | -0.03% | 102% | 100% |
| Class C | -0.22% | -0.11% | -0.04% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.18% | 0.14% | -0.14% | 102% | 99% |
| Class F | -0.15% | -0.15% | -0.21% | 102% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.24% | -0.21% | 0.04% | 100% | 101% |
| Class E | -0.21% | -0.34% | -1.06% | 99% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.07% | -0.47% | 99% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

This contribution reports results of three more combined tests on sign prediction improvement. Compared with test 4.3.a of EE2, test 1 uses adaptive maximum area for sign prediction and test 2 keeps the maximum area of sign prediction as 4x4.. Test 3 is a combined test of test 1 and EE2-4.3b. It solves the parsing issue and the computation issue proposed by the experts during the EE review with negligible coding gain drop. Thus, it is suggested to adopt test 3 to the next version of ECM

References

J. Chen, Y. Yan, R.-L Liao, X. Li, X. Xiu, Y.-W. Chen, N. Yan, C.-W. Kuo, H.-J. Jhu, W. Chen, X. Wang, “EE2-4.3: Combined Test Results of EE2-4.1 and EE2-4.2 on sign prediction”, JVET-Y0138, Jan. 2022

M. G. Sarwer, Y. Ye, J. Chen, R.-L, Liao, “AHG12: On sign prediction”, JVET-X0120, Oct 2021.

X. Xiu, J.-H. Jhu, W. Chen, C.-W. Kuo, N. Yan, Y.-W. Chen, X. Wang, “AHG12: Enhanced sign prediction”, JVET-X0150, Oct. 2021.

M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).