| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0143-v2 |
| --- | --- |

| Title: | EE1-1.2: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang Li Zhang (Bytedance) Hongtao Wang Kevin Reuze Anand Meher Kotra Marta Karczewicz (Qualcomm) | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com hongtaow@qti.qualcomm.com |
| Source: | Bytedance Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents the EE test results of JVET-X0066. In this test, convolutional neural network-based models are utilized for in-loop filtering and tested on top of both VTM software (EE1-1.2.1) and ECM software (EE1-1.2.2).

In EE1-1.2.1, compared with VTM-11.0 + NewMCTF, the test results reportedly show on average {9.80%, 21.20%, 21.17%}, {8.52%, 14.33%, 14.24%}, and {7.39%, 19.28%, 19.37%} BD-rate reductions for {Y, Cb, Cr} under RA, LDB, and AI configurations, respectively.

In EE1-1.2.2, compared with ECM-3.0, the test results reportedly show on average {7.24%, 17.12%, 18.63%}, {6.79%, 17.35%, 17.50%}, and {4.76%, 14.71%, 15.69%} BD-rate reductions for {Y, Cb, Cr} under RA, LDB, and AI configurations, respectively. Compared with VTM-11.0 + new MCTF, a combination of the NN-based coding methods and the non-NN-based coding methods beyond VVC reportedly shows on average {21.27%, 32.38%, 34.40%}, {11.34%, 25.74%, 27.36%}, and {17.81%, 31.59%, 30.67%} BD-rate reductions for {Y, Cb, Cr}, under RA and AI configurations, respectively.

# Introduction

In JVET-X0066 [1], a deep learning-based in-loop filtering method was tested on top of the VTM software. This EE test is to study the performance of the deep filtering method presented in JVET-X0066 on top of different softwares. Specifically, EE1-1.2.1 and EE1-1.2.2 use VTM anchor and ECM anchor respectively, to evaluate the deep filter presented in JVET-X0066.

# Proposed method

The proposed CNN model takes reconstruction, partitioning, boundary strength and QP as input. In addition, residual scaling is adopted, i.e., the differences (a.k.a. residues) between the input samples and the NN filtered samples may be further scaled by the corresponding scaling factors before being added to input samples. The CNN-architecture is conditioned on QP, leading to a unified model to handle different quality levels. Section 2.1 - 2.3 present the CNN structure, the parameter selection, and the residue scaling, respectively. In addition, section 2.4 presents the inference and section 2.5 describes the training process.

## Architecture

The backbone of the proposed CNN filter is shown in Fig. 1. The calculation process in the attention module can be written as:

F_out = F_in × f (Rec, Pred, BS, QP) + F_in

where F_in and F_out denote the input and the output of the attention module, respectively. Rec, Pred, BS, and QP stand for the reconstruction, the prediction, the boundary strengths, and the sequence-level input quantization parameter respectively. f comprises 2 convolutional layers, where an activation function is applied after the first convolutional layer. The objective of f is to generate a spatial attention map from external information, which then recalibrates the feature maps F_in.

The input of the network comprises reconstruction, prediction, boundary strengths, and QP (split is additionally taken as input in intra slice models). The numbers of feature maps and attention residual blocks are set as 96 and 8, respectively.

Fig. 1. (a) Architecture of the proposed CNN filtering method. M denotes the number of feature maps. N stands for the number of samples in one dimension. (b) Construction of Attention Residual Block in (a)

## Parameter Selection

In the proposed technique, each slice or block could determine whether to apply the CNN-based filter or not. When the CNN-based filter is determined to be applied to a slice/block, which conditional parameter from a candidate list including three candidates derived from QP could be further decided. Denote the sequence level QP as q, the candidate list includes conditional parameters {q, q - 5, q - 10}. The selection process is based on the rate-distortion cost at the encoder side. Indication of on/off control as well as the conditional parameter index, if needed, are signalled in the bitstream. Other details regarding the parameter selection are provided as below.

- The granularity of the filter determination and the parameter selection is dependent on resolution and QP. In the proposed scheme, given a higher resolution and a larger QP, the determination and selection will be performed in a larger region.

- The candidate parameter list is different across different temporal layers. For a high temporal layer, the third candidate q - 10 is replaced by q + 5.

- For all-intra configuration, parameter selection is disabled while on/off control is still preserved.

## Residue Scaling

When a NN filter is being applied to reconstructed pictures, a scaling factor is signaled for each color component in the picture header. The difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples.

## Inference

PyTorch is used for performing the inference of the proposed CNN filters in VTM. The network information in the inference stage is provided in Table 1 as suggested by [2].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | ~1.56M/model (4 models) |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | ~6.3MB/model, 4 models in total |
|  | MAC (Giga) | 539K/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 25+16 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

## Training

PyTorch is used as the training platform. The DIV2K [3] and BVI-DVC [4] datasets are adopted to train the CNN filters of I slices and B slices, respectively. Different CNN models are trained to adapt to different QP points. The network information in the training stage is provided in Table 2 as suggested by [2].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 90 |
|  | Batch size: | 64 |
|  | Training time: | 60h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF in EE1-1.2.1, ECM-3.0 in EE1-1.2.2, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

In EE1-1.2.1, the proposed CNN-based in-loop filtering method is tested on top of VTM-11.0 + new MCTF [5] according to the common test conditions defined in [2], where the new MCTF patch is from [6]. Deblocking filtering and SAO are disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering. Test results are shown in Table 3 ~ Table 5. It can be observed that the proposed method could bring on average {9.80%, 21.20%, 21.17%}, {8.52%, 14.33%, 14.24%}, and {7.39%, 19.28%, 19.37%} BD-rate reductions for {Y, Cb, Cr} under RA, LDB, and AI configurations, respectively.

In EE1-1.2.2, the proposed CNN-based in-loop filtering method is tested on top of ECM-3.0 [7] according to the common test conditions defined in [2]. Deblocking filtering is disabled while SAO (and CCSAO) and ALF (and CCALF) are placed after the proposed CNN-based filtering. Test results are shown in 6 ~ Table 8. It can be observed that the proposed method could bring on average {7.24%, 17.12%, 18.63%}, {6.79%, 17.35%, 17.50%}, and {4.76%, 14.71%, 15.69%} BD-rate reductions for {Y, Cb, Cr} under RA, LDB, and AI configurations, respectively. In addition, Table 9 and Table 10 provide the coding efficiency of a combination of the NN-based coding methods and the non-NN-based coding methods beyond VVC compared to VVC.

Table 3. Performance of the proposed method on top of VTM-11.0 + new MCTF (RA)

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -11.77% | -9.23% | -17.16% | -19.98% | 197% | 45290% |
| Class A2 | -12.50% | -10.55% | -20.48% | -17.58% | 192% | 42723% |
| Class B | -12.21% | -9.38% | -23.11% | -22.54% | 189% | 42241% |
| Class C | -12.74% | -10.18% | -22.40% | -23.06% | 153% | 33606% |
| Class E |  |  |  |  |  |  |
| Overall | -12.32% | -9.80% | -21.20% | -21.17% | 181% | 40391% |
| Class D | -14.30% | -11.74% | -24.15% | -24.74% | 147% | 30414% |
| Class F | -6.44% | -4.83% | -12.57% | -12.40% | 241% | 18250% |

Table 4. Performance of the proposed method on top of VTM-11.0 + new MCTF (LDB)

|  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -9.13% | -7.96% | -14.20% | -14.72% | 189% | 44519% |
| Class C | -10.22% | -9.46% | -13.91% | -14.13% | 157% | 35947% |
| Class E | -9.51% | -8.21% | -15.11% | -13.58% | 315% | 41646% |
| Overall | -9.59% | -8.52% | -14.33% | -14.24% | 202% | 40770% |
| Class D | -12.06% | -10.82% | -18.21% | -17.21% | 146% | 29523% |
| Class F | -5.33% | -5.13% | -7.58% | -5.08% | 245% | 20325% |

Table 5. Performance of the proposed method on top of VTM-11.0 + new MCTF (AI)

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -9.66% | -6.53% | -17.59% | -17.98% | 175% | 32189% |
| Class A2 | -8.90% | -6.63% | -19.47% | -17.09% | 142% | 25792% |
| Class B | -9.21% | -6.68% | -20.45% | -20.46% | 133% | 24330% |
| Class C | -10.00% | -7.61% | -18.40% | -20.71% | 117% | 15970% |
| Class E | -11.95% | -9.86% | -19.99% | -19.43% | 136% | 26349% |
| Overall | -9.86% | -7.39% | -19.28% | -19.37% | 137% | 23756% |
| Class D | -9.91% | -7.64% | -18.71% | -20.92% | 114% | 14040% |
| Class F | -5.64% | -4.16% | -11.84% | -11.53% | 116% | 17595% |

Table 6. Performance of the proposed method on top of ECM-3.0 (RA)

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -9.43% | -7.20% | -14.38% | -16.43% | 118% | 9879% |
| Class A2 | -10.42% | -8.03% | -17.17% | -20.01% | 117% | 8834% |
| Class B | -9.32% | -6.82% | -18.64% | -19.31% | 116% | 8626% |
| Class C | -9.36% | -7.21% | -17.22% | -18.37% | 107% | 7110% |
| Class E |  |  |  |  |  |  |
| Overall | -9.57% | -7.24% | -17.12% | -18.63% | 114% | 8458% |
| Class D | -10.86% | -8.62% | -18.80% | -20.37% | 107% | 6078% |
| Class F | -4.99% | -3.70% | -9.97% | -10.15% | 137% | 4629% |

Table 7. Performance of the proposed method on top of ECM-3.0 (LDB)

|  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -8.54% | -6.65% | -16.36% | -17.49% | 124% | 12522% |
| Class C | -8.34% | -6.53% | -16.76% | -17.42% | 110% | 10302% |
| Class E | -9.67% | -7.36% | -19.78% | -17.62% | 160% | 15564% |
| Overall | -8.75% | -6.79% | -17.35% | -17.50% | 127% | 12389% |
| Class D | -10.07% | -7.99% | -19.41% | -20.29% | 109% | 8649% |
| Class F | -5.45% | -4.17% | -11.48% | -10.44% | 144% | 6412% |

Table 8. Performance of the proposed method on top of ECM-3.0 (AI)

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -9.66% | -4.45% | -13.15% | -15.40% | 113% | 13151% |
| Class A2 | -8.90% | -4.32% | -14.89% | -14.61% | 101% | 10946% |
| Class B | -9.21% | -4.19% | -15.56% | -15.48% | 101% | 8880% |
| Class C | -10.00% | -4.51% | -13.79% | -16.43% | 97% | 6085% |
| Class E | -11.95% | -6.77% | -15.92% | -16.44% | 101% | 9248% |
| Overall | -9.86% | -4.76% | -14.71% | -15.69% | 102% | 9087% |
| Class D | -9.91% | -4.78% | -13.72% | -16.61% | 98% | 5493% |
| Class F | -5.64% | -2.97% | -8.58% | -8.54% | 102% | 5420% |

Table 9. Performance of a combination of the proposed method and non-NN-based tools presented in ECM-3.0 compared with VTM-11.0 + new MCTF (RA)

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -24.71% | -22.36% | -27.69% | -34.85% | 548% | 47643% |
| Class A2 | -25.96% | -23.15% | -33.67% | -36.06% | 522% | 46178% |
| Class B | -22.95% | -19.56% | -35.03% | -34.96% | 482% | 42187% |
| Class C | -23.36% | -21.18% | -31.64% | -32.13% | 429% | 33079% |
| Class E |  |  |  |  |  |  |
| Overall | -24.02% | -21.27% | -32.38% | -34.40% | 487% | 41250% |
| Class D | -24.86% | -22.75% | -32.96% | -33.44% | 413% | 29582% |
| Class F | -19.15% | -16.81% | -27.22% | -27.21% | 471% | 18068% |

Table 10. Performance of a combination of the proposed method and non-NN-based tools presented in ECM-3.0 compared with VTM-11.0 + new MCTF (AI)

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -15.55% | -11.34% | -24.39% | -29.97% | 447% | 34865% |
| Class A2 | -13.58% | -10.57% | -26.38% | -24.89% | 387% | 28132% |
| Class B | -13.54% | -10.19% | -27.91% | -28.23% | 369% | 23932% |
| Class C | -14.08% | -11.42% | -23.44% | -26.07% | 337% | 15583% |
| Class E | -16.62% | -13.90% | -25.90% | -27.45% | 356% | 26438% |
| Overall | -14.51% | -11.34% | -25.74% | -27.36% | 374% | 24194% |
| Class D | -12.81% | -10.57% | -21.39% | -23.83% | 328% | 13939% |
| Class F | -15.93% | -13.83% | -24.19% | -24.27% | 244% | 16615% |

# Conclusions

This contribution presents the test result of a CNN-based in-loop filtering method. The proposed CNN-based filtering method shows favorable trade-off in terms of the coding performance and complexity. It is suggested to adopt this contribution as the software base for NN-based tool evaluations in the future.

# References

- EE1-1.6: Combined Test of EE1-1.2 and EE1-1.4, JVET-X0066, Y. Li, K. Zhang, L. Zhang, H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

- https://vcgit.hhi.fraunhofer.de/jvet-x-ee1/ecm-jvet-x-ee-1/-/tree/ECM-3.0

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).