| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0144 |
| --- | --- |

| Title: | Non-EE2: DIMD Flag Signalling Clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung-Hwan Kim | Tel: Email: | +1 (858) 635-5300 jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In ECM3.1, the position of dimd_flag is separated from other intra mode related syntax elements, and it unnecessarily complicates the software and signalling logic. This contribution proposes to change the position of dimd_flag, and place it right before timd_flag. It is asserted that this change makes the code and signalling logic cleaner, and consistent with timd_flag signalling.

This change has little impact to coding performance. For class A to E, on average, impact on RD results are -0.01% -0.03%, 0.00% for AI, and 0.01%, 0.06%, 0.10% for RA configurations, no change for LDB.

For class F, the impact is slightly more noticeable, -0.04%, 0.11%, -0.11% for AI, -0.02%, 0.00%, -0.28% for RA and -0.10%, -1.13%, -0.52% for LDB.

# Introduction

In current ECM3.1 software, the position of dimd_flag is separated from other intra mode related syntax elements. All other intra mode related syntax elements are located inside function cu_pred_data() which includes the function intra_luma_pred_mode(). Only dimd_flag is located outside of cu_pred_data() , and before ACT and PLT.

CABACReader.cpp void CABACReader::coding_unit( CodingUnit &cu, Partitioner &partitioner, CUCtx& cuCtx ) { .. // prediction mode and partitioning data pred_mode ( cu ); cu_dimd_flag( cu ); if (CU::isIntra(cu)) { adaptive_color_transform(cu); } if (CU::isPLT(cu)) { .. cu_palette_info .. .. return; } // --> create PUs // prediction data ( intra prediction modes / reference indexes + motion vectors ) cu_pred_data( cu ); .. }

At the encoder side, during RD decision, because dimd_flag is not included in the rate estimation function cu_pred_data()which includes intra_luma_pred_mode(), therefore the dimd_flag has to be specially handled in the rate estimation. For example, in the code below, m_CABACEstimator->cu_dimd_flag( cu ) has to be called separately. There is more than one such place in ECM3.1. In addition, it seems rate of dimd_flag bit is missed in some other part of RD decision, such as xFracModeBitsIntra().

EncCU.cpp bool EncCu::xCheckRDCostIntra .. { .. m_CABACEstimator->pred_mode ( cu ); m_CABACEstimator->cu_dimd_flag ( cu ); m_CABACEstimator->adaptive_color_transform(cu); m_CABACEstimator->cu_pred_data ( cu ); .. }

Basically, there is no reason that the dimd_flag is at such a special location different from other intra mode syntax elements. Its current position unnecessarily complicates the code and signalling logic. Placing the dimd_flag inside intra_luma_pred_mode() would solve this issue and make code cleaner.

# Proposed Changes

We propose to move the dimd_flag signalling to be right before timd_flag. This simple change would make the code cleaner, and consistent with timd_flag signalling. Putting dimd_flag and timd_flag next to each other also have the potential benefit for easier DIMD and TIMD flag joint signalling if in the future DIMD and TIMD get harmonized.

| Signaling Order in ECM3.1 | Proposed signaling order |
| --- | --- |
| dimd_flag ACT PLT BDPCM TM (template matching) MIP timd_flag MRL … | dimd_flag ACT PLT BDPCM TM MIP dimd_flag timd_flag MRL … |

The corresponding syntax table with such change is as below. The blue highlighted texts are deletions and the green highlighted texts are additions. Since DIMD and tools such as MIP, BDPCM TM etc. are mutual exclusive, moving dimd_flag to a lower position also changes the signalling logic of some related syntax elements. The number of places that needs to check dimd_flag is reduced. The signalling logic appear to be cleaner.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_palette_enabled_flag && … ) ) |  |
| pred_mode_plt_flag | ae(v) |
| if (CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_dimd_enable_flag ) |  |
| intra_dimd_flag | ae(v) |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && … ) |  |
| cu_act_enabled_flag | ae(v) |
| … |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) |  |
| else { |  |
| if( sps_bdpcm_enabled_flag && && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize && && !intra_dimd_flag ) |  |
| intra_bdpcm_luma_flag | ae(v) |
| if( intra_bdpcm_luma_flag ) |  |
| intra_bdpcm_luma_dir_flag | ae(v) |
| else { |  |
| if( sps_tmp_enable_flag && cbWidth <= TMP_MaxSize && cbHeight <= TMP_MaxSize && !intra_dimd_flag ) |  |
| intra_tmp_flag | ae(v) |
| if ( !intra_tmp_flag && !intra_dimd_flag) { |  |
| if( sps_mip_enabled_flag ) |  |
| intra_mip_flag | ae(v) |
| if( intra_mip_flag ) { |  |
| intra_mip_transposed_flag[ x0 ][ y0 ] | ae(v) |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if (sps_dimd_enable_flag ) |  |
| intra_dimd_flag |  |
| if (!intra_dimd_flag) { |  |
| if ( sps_timd_enable_flag && (sh_slice_type != I ¦¦ (sh_slice_type == I && cbWidth * cbHeight <=1024 ))) |  |
| intra_timd_flag | ae(v) |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx | ae(v) |
| if( sps_isp_enabled_flag && …) |  |
| intra_subpartitions_mode_flag | ae(v) |
| } |  |
| if (!intra_timd_flag && !intra_dimd_flag) { |  |
| if( intra_luma_ref_idx = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |
| } |  |

# Results

The proposed changes are implemented on top of ECM3.1. In CTC, ACT and PLT are not used, and BDPCM and TM are only enabled for screen content. Therefore, for class A to E, the only impact of the proposed change is due to that dimd_flag is moved after MIP. For low delay, MIP is not enabled either. Therefore there is no RD result change to class A to E in low delay.

For screen content, class F and TGM, BDPCM and TM are enabled. Now that dimd_flag is moved after BDPCM and TM, therefore the impact of the proposed change is a slightly more noticeable for screen content. For class F, there is a small gain of -0.04% for AI, -0.02% for RA and -0.1% for LDB.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.04% | 101% | 100% |
| Class A2 | -0.01% | 0.01% | 0.01% | 100% | 99% |
| Class B | -0.01% | -0.07% | -0.01% | 100% | 100% |
| Class C | -0.01% | -0.03% | 0.01% | 100% | 99% |
| Class E | 0.00% | -0.06% | -0.07% | 101% | 101% |
| Overall | -0.01% | -0.03% | 0.00% | 100% | 100% |
| Class D | 0.02% | -0.03% | -0.22% | 101% | 100% |
| Class F | -0.04% | 0.11% | -0.11% | 100% | 101% |
| Class TGM | 0.00% | -0.02% | -0.02% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.04% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.14% | 0.11% | 100% | 100% |
| Class B | 0.01% | 0.08% | 0.09% | 100% | 100% |
| Class C | 0.00% | 0.04% | 0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | 0.10% | 100% | 100% |
| Class D | 0.01% | 0.19% | -0.22% | 100% | 100% |
| Class F | -0.02% | 0.00% | -0.28% | 100% | 99% |
| Class TGM | -0.04% | -0.05% | 0.02% | 100% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -0.10% | -1.13% | -0.52% | 101% | 99% |
| Class TGM | 0.03% | -0.10% | 0.15% | 100% | 101% |

# Results

The contribution proposes to change the position of dimd_flag to be right before timd_flag. It is asserted that this change make the code and signalling logic cleaner, and also consistent with time_flag signalling. There is little impact on coding performance. It’s proposed to adopt this change into ECM.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).