| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0148-v1 |
| --- | --- |

| Title: | Non-EE2: Spatial-Temporal Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yue Li2, Hongbin Liu1, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

In the current ALF design of ECM, the online-trained filters for Luma component only have one fixed filter shape only with the spatial neighbouring samples inside the current picture for filtering. In this contribution, a spatial-temporal adaptive loop filter (ST-ALF) is proposed. With ST-ALF, a sample is filtered with both its spatial and temporal neighbouring samples.

On top of ECM-3.1, simulation results (BD-rate changes, encoding time and decoding time) of ST-ALF are reported as below:

AI: 0.00%, 0.00%, 0.00%, 101%, 100%;

RA: %, %, %, %, %;

LB: %, %, %, %, %;

Introduction

The existing filter shape of ALF in ECM3-1 is a diamond shape with a size of nine as shown in Figure 2-1(a). It only contains the spatial taps on spatial neighbouring samples inside the current picture for filtering, lacking the information from temporal neighbouring samples.

Proposed methods

Spatial-temporal adaptive loop filter (ST-ALF) is proposed to utilize the temporal information in ALF. In the proposed method, several extended temporal taps are introduced together with the spatial taps to form a new filter shape. The newly introduced temporal taps, as shown in Figure 2-1(b), involve the collocated samples inside a forward/backward reference frame. The spatial taps and temporal taps are jointly trained to generate the corresponding coefficients and clipping parameters.

The proposed filter shape works along with the existing filter shape in a switchable way. The new shape is only trained/enabled for the luma component in non-intra slices and each CTB could select the optimal filter shape based on the available APSs.

- The existing filter shape of ALF in ECM-3.1

- Filter shape of the proposed ST-ALF

Figure 2-1 Filter shapes for ALF.

Simulation Results

The proposed method is implemented on top of ECM-3.1 [1]. The simulation results are tested under common test condition [2] and the anchor is ECM-3.1.

Table 3-1 shows the simulation results of the proposed method.

It is observed that the performance on Luma component is promising.

TABLE 3-1 Performance of the proposed ST-ALF

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 101% | -0.16% | 0.16% | 0.16% | 111% | 102% |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 101% | -0.08% | 0.10% | 0.09% | 112% | 102% |  |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 99% |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% | -0.13% | 0.03% | 0.02% | 118% | 101% | -0.02% | 0.33% | 0.07% | 125% | 101% |

- Conclusions

ST-ALF is proposed in this contribution. The proposed method can achieve promising coding performance. It is recommended to further study it in EE2.

References

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ECM/-/tree/ECM-3.1.

- M. Karczewicz, Y.Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, October. 2021.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).