| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0151 |
| --- | --- |

| Title: | Non-EE2: Adaptive re-ordering of merge candidates with refined motion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Na Zhang1, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to refine motion information before invoking the adaptive re-ordering of merge candidates (ARMC) process. Specifically, instead of re-ordering the merge candidates using the initial motion information obtained from spatial/temporal/pairwise candidates, template matching (TM) and multi-pass decoder motion vector refinement (DMVR) are applied first to derive the refined motion of each merge candidate. Then the refined motion is used as inputs to the ARMC process. Compared to ECM-3.1, simulation results of the proposed method are reported as below:

RA: ;

LDB: .

Introduction

In ECM-3.1, adaptive re-ordering of merge candidates (ARMC) [1] is used to code the merge index more efficiently, in which the template matching cost of each merge candidate is evaluated for the re-ordering. As shown in Figure 1, a merge candidate list is constructed first. Then ARMC is used to re-order the merge candidate list. After that, the selected merge candidate indicated by merge index may be further refined using TM or multi-pass DMVR. Finally, the refined motion is used in motion compensation. The motion information used in ARMC is not aligned with that used in motion compensation. Therefore, the refined motion could also be used in ARMC to achieve better coding performance.

Figure 1. ARMC in ECM-3.1.

Proposed method

This contribution proposes to use the refined motion in ARMC. Specifically, instead of re-ordering the merge candidate list directly, each merge candidate in the merge candidate list is refined using TM/multi-pass DMVR first and the refined motion will be used in ARMC to re-order the merge candidate list. Compared to ARMC in ECM-3.1, the only difference is highlighted by the red box in Figure 2. The proposed ARMC with refined motion is applied to template matching merge (TM merge) mode, adaptive DMVR mode, and template matching for advanced motion vector prediction (TM-AMVP) mode. When template matching is used to derive the refined motion, the template size is set equal to 1. When multi-pass DMVR is used to derive the refined motion, only the first pass (i.e., PU level) of multi-pass DMVR is involved. The selected merge candidate is further refined using the same process as that in ECM-3.1.

Figure 2. ARMC with refined motion.

Simulation results

The proposed method was implemented on top of the ECM-3.1 [2]. Simulations were performed following the common test conditions [3]. More detailed results could be found in the attached excel sheet. The results compared to ETM-3.1 are shown in Table 1.

Table 1: Coding performance compared to ECM-3.1

|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.12% | -0.19% | -0.08% | 103% | 98% |  |  |  |  |  |
| Class C | -0.13% | -0.20% | -0.13% | 105% | 102% | -0.12% | 0.15% | 0.14% | 108% | 103% |
| Class E |  |  |  |  |  | -0.05% | 0.27% | -0.22% | 104% | 87% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.16% | 0.05% | 0.00% | 106% | 103% | 0.04% | -0.21% | -0.26% | 109% | 105% |
| Class F (Optional) | -0.07% | -0.10% | -0.07% | 103% | 88% | 0.08% | 0.41% | -0.12% | 104% | 82% |

Conclusions

In this contribution, adaptive re-ordering of merge candidates with refined motion is proposed. It is recommended to study the proposed method in EE2.

Reference

- N. Zhang, K. Zhang, L. Zhang, H. Liu, Z. Deng, and Y. Wang, “EE2-3.1/EE2-3.2: Adaptive Reordering of Merge Candidates with Template/Bilateral Matching,” Document of Joint Video Experts Team, JVET-W0090, by teleconference, July 2021.

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-3.1.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-X2017, by teleconference, October 2021.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).