| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0152-v2 |
| --- | --- |

| Title: | AHG10: Fast skip of TT split partitioning on top of ECM reference software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lien-Fei Chen, Xiang Li, Shan Liu 2747 Park Boulevard Palo Alto, CA 94306, U.S.A | Email: | lienfeichen@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The contribution presents a fast method of conditionally skipping ternary-tree (TT) split on top of ECM-3.1 reference software. The overall simulation results on top of ECM-3.1 are reported as below.

On top of ECM-3.1 with 1.100 threshold value (ECM-3.1 as anchor):

- AI: xxx%/xxx%/xxx% with xxx% EncT and xxx% DecT

- RA: 0.09%/0.14%/0.14% with 95% EncT and 100% DecT

- LB: 0.05%/-0.20%/-0.05% with 96% EncT and 100% DecT

On top of ECM-3.1 with 1.075 threshold value (ECM-3.1 as anchor):

- AI: xxx%/xxx%/xxx% with xxx% EncT and xxx% DecT

- RA: 0.13%/0.19%/0.31% with 93% EncT and 100% DecT

- LB: 0.08%/-0.04%/0.00% with 94% EncT and 100% DecT

# Introduction

JVET-W0086[1] explored unsymmetric partitioning methods denoted as unsymmetric binary-tree (UBT) partitioning and unsymmetric quad-tree (UQT) In [1], a fast partitioning method was also proposed on top of ECM-1.0 to speed up the partition search for UBT and UQT. This method is also applicable for TT split on top of ECM.

# Proposal

In [1], the proposed method used the RD cost of the non-split partition and the binary-tree (BT) partition to early skip the RD process of the UBT and UQT. This fast method could be further utilized for ternary-tree (TT) partition. In this proposal, the fast method in [1] is also used to early skip the TT partitioning on top of ECM-3.1.

bool res = false;

if (split == CU_TRIH_SPLIT)

{

if (m_pcEncCfg->getFastTTskip() & FAST_METHOD_ENC_SPEEDUP_BT_BASED)

{

if (splitRdCostBest[CTU_LEVEL] < MAX_DOUBLE && splitRdCostBest[CU_HORZ_SPLIT] < MAX_DOUBLE)

{

if (splitRdCostBest[CU_HORZ_SPLIT] > l_TT_ENC_SPEEDUP_RATE * splitRdCostBest[CTU_LEVEL]) { res = true; }

}

}

if (m_pcEncCfg->getFastTTskip() & FAST_METHOD_HOR_XOR_VER)

{

if (splitRdCostBest[CU_HORZ_SPLIT] < MAX_DOUBLE && splitRdCostBest[CU_VERT_SPLIT] < MAX_DOUBLE)

{

if (splitRdCostBest[CU_HORZ_SPLIT] > l_HOR_XOR_VER_RATE * splitRdCostBest[CU_VERT_SPLIT]) { res = true; }

}

}

}

if (split == CU_TRIV_SPLIT)

{

if (m_pcEncCfg->getFastTTskip() & FAST_METHOD_ENC_SPEEDUP_BT_BASED)

{

if (splitRdCostBest[CTU_LEVEL] < MAX_DOUBLE && splitRdCostBest[CU_VERT_SPLIT] < MAX_DOUBLE)

{

if (splitRdCostBest[CU_VERT_SPLIT] > l_TT_ENC_SPEEDUP_RATE * splitRdCostBest[CTU_LEVEL]) { res = true; }

}

}

if (m_pcEncCfg->getFastTTskip() & FAST_METHOD_HOR_XOR_VER)

{

if (splitRdCostBest[CU_HORZ_SPLIT] < MAX_DOUBLE && splitRdCostBest[CU_VERT_SPLIT] < MAX_DOUBLE)

{

if (splitRdCostBest[CU_VERT_SPLIT] > l_HOR_XOR_VER_RATE * splitRdCostBest[CU_HORZ_SPLIT]) { res = true; }

}

}

}

return res;

The fast skip checking mechanism will be performed before TT split partition searching process. For horizontal TT split partition, the RD cost comparison between non-split partition and horizontal BT split partition and the RD cost comparison between horizontal BT split and vertical BT split are calculated to decided whether the horizontal TT split partition search procedure should be performed or not. For vertical TT split partition, the above similar process is also utilized to early skip the vertical TT split partition search procedure. The relative source code on top of ECM-3.1 is shown as follows. The typical value of 1_TT_ENC_SPEEDUP_RATE and l_HOR_XOR_VER_RATE within the source code could be set as 1.100 or 1.075. When we choose the 1.1 as threshold value, we could get less coding loss but also have less runtime saving. Otherwise, we could get higher runtime saving with higher coding loss for the setting with 1.075 threshold value.

The overall performance impact and runtime saving of the fast skip of TT partitioning on top of EMC-3.1 is shown in Table 1 and Table 2. Two different threshold values, 1.1 and 1.075, are tested for 1_TT_ENC_SPEEDUP_RATE and l_HOR_XOR_VER_RATE. Table 1 shows the results with the 1.10 threshold value for both of the 1_TT_ENC_SPEEDUP_RATE and l_HOR_XOR_VER_RATE. The results with 1.075 threshold value is shown in Table 2.

Table 1: Simulation Results of fast skip for TT partition on top of ECM-3.1 with threshold value = 1.100

| Fast TT Skip thr=1.1 | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | 0.17% | 93% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.02% | -0.01% | 0.00% | 94% | 99% |
| Class C | 0.03% | -0.05% | 0.23% | 96% | 100% |
| Class E | 0.07% | -0.02% | 0.21% | 93% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.04% | 0.16% | 0.00% | 97% | 100% |
| Class F | 0.20% | 0.19% | 0.32% | 93% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Fast TT Skip thr=1.1 | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.15% | 0.01% | 93% | 100% |
| Class A2 | 0.08% | 0.14% | 0.16% | 96% | 100% |
| Class B | 0.08% | 0.11% | 0.11% | 95% | 100% |
| Class C | 0.12% | 0.15% | 0.28% | 94% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.14% | 0.15% | 95% | 100% |
| Class D | 0.03% | 0.32% | -0.05% | 96% | 100% |
| Class F | 0.22% | 0.22% | 0.10% | 93% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Fast TT Skip thr=1.1 | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.25% | -0.16% | 95% | 99% |
| Class C | -0.01% | -0.12% | 0.13% | 97% | 99% |
| Class E | 0.16% | -0.22% | -0.10% | 96% | 102% |
| Overall | 0.05% | -0.20% | -0.05% | 96% | 100% |
| Class D | 0.09% | -0.05% | -0.92% | 97% | 100% |
| Class F | 0.14% | 0.69% | 0.63% | 95% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2: Simulation Results of fast skip for TT partition on top of ECM-3.1 with threshold value = 1.075

| Fast TT Skip thr=1.075 | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.12% | 0.09% | 90% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.03% | 0.07% | 0.14% | 92% | 99% |
| Class C | 0.07% | 0.07% | 0.25% | 94% | 101% |
| Class E | 0.12% | 0.17% | 0.17% | 90% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.05% | 0.27% | 0.18% | 94% | 100% |
| Class F | 0.21% | 0.29% | 0.30% | 90% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Fast TT Skip thr=1.075 | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.09% | 0.13% | 91% | 100% |
| Class A2 | 0.14% | 0.30% | 0.28% | 94% | 100% |
| Class B | 0.11% | 0.19% | 0.33% | 93% | 100% |
| Class C | 0.17% | 0.18% | 0.44% | 92% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | 0.19% | 0.31% | 93% | 100% |
| Class D | 0.13% | 0.66% | -0.04% | 94% | 100% |
| Class F | 0.28% | 0.54% | 0.01% | 91% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Fast TT Skip thr=1.075 | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.39% | 0.25% | 94% | 100% |
| Class C | 0.03% | 0.17% | -0.09% | 94% | 100% |
| Class E | 0.20% | 0.26% | -0.31% | 95% | 101% |
| Overall | 0.08% | -0.04% | 0.00% | 94% | 100% |
| Class D | 0.13% | 0.88% | 0.09% | 95% | 99% |
| Class F | 0.32% | -0.39% | 0.50% | 93% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

In [1], the fast method for UBT and UQT is applicable for TT split partitioning on top of ECM. Based on the fast method in [1], we test the early skip method which has been used for the UQT and UBT in EE Test 1.1-1.4, for TT split partition on top of EMC-3.0. According to the simulation results, this fast method with 1.075 threshold value has 0.08% coding loss and about 6% runtime reduction for ECM-3.1 in LB case. In order to speed up the ECM software especially for the LB case, we recommend adopting this fast method with 1.075 threshold value on ECM software.

# Reference

- K. Zhang, L. Zhang, Z. Deng, N. Zhang, and Y. Wang, “EE2-1.3/EE2-1.4: Unsymmetric partitioning methods in video coding”, ISO/IEC JTC1/SC29/WG11 JVET-W0086, July 2021.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.