| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0153-v1 |
| --- | --- |

| Title: | EE2-3.11: Non-adjacent spatial neighbors for affine merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Che-Wei Kuo, Xianglin Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | { chenwei06, xiaoyuxiu, yiwenchen, jhuhong-jheng, cheweikuo, xianglinwang } @kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-3.11 on using non-adjacent spatial neighbors for affine merge and AMVP modes. In the proposed scheme, the motion information of non-adjacent spatial neighbors, which is obtained in a fixed pattern, is utilized to derive additional inherited and constructed affine merge candidates. The derived candidates are inserted into the existing candidate lists of affine merge mode and affine AMVP mode. The proposed method was implemented on top of ECM-3.1 [3], and it reports the average {Y, U, V} BD-rate savings of {-0.35%, -0.12%, -0.22%} and {-0.46%, -0.29%, -0.53%} for RA and LD configurations, respectively. The corresponding encoding and decoding time are 100% and 100% for RA, and 101% and 101% for LD, respectively.

Introduction

In the VVC [1], for both regular merge mode and affine merge mode, only adjacent spatial neighbors above and left to one current block are utilized to form its corresponding merge candidate list. In ECM-3 [2], an enhanced regular merge mode, which additionally inserts a plurality of non-adjacent spatial merge candidates to the existing regular merge candidate list, is applied to improve the inter coding efficiency. Similar to the enhanced regular merge mode, in JVET-X0151 [5], it proposed to use non-adjacent spatial neighbors for affine merge mode (NSAM).

Proposed method

In this contribution, the NSAM method proposed in [5] is tested as EE2-3.11 according to the EE description [6].

In NSAM, the pattern of obtaining non-adjacent spatial neighbors is shown in Figure 1. Same as the existing non-adjacent regular merge candidates [2], the distances between non-adjacent spatial neighbors and current coding block in the NSAM are also defined based on the width and height of current CU.

The motion information of the non-adjacent spatial neighbors in Figure 1 is utilized to generate additional inherited and constructed affine merge candidates. Specifically, for inherited candidates, the same derivation process of the inherited affine merge candidates in the VVC is kept unchanged except that the CPMVs are inherited from non-adjacent spatial neighbors. The non-adjacent spatial neighbors are checked based on their distances to the current block, i.e., from near to far. At a specific distance, only the first available neighbor (that is coded with the affine mode) from each side (e.g., the left and above) of the current block is included for inherited candidate derivation. As indicated by the red dash arrows in Figure 1 (a), the checking orders of the neighbors on the left and above sides are bottom-to-up and right-to-left, respectively. For constructed candidates (namely “the first type of constructed affine candidates from non-adjacent neighbors”), as shown in the Figure 1 (b), the positions of one left and above non-adjacent spatial neighbors are firstly determined independently; After that, the location of the top-left neighbor can be determined accordingly which can enclose a rectangular virtual block together with the left and above non-adjacent neighbors. Then, as shown in the Figure 2, the motion information of the three non-adjacent neighbors is used to form the CPMVs at the top-left (A), top-right (B) and bottom-left (C) of the virtual block, which is finally projected to the current CU to generate the corresponding constructed candidates.

Figure 1 Spatial neighbors for deriving affine merge candidates: (a) for deriving inherited affine merge candidates (b) for deriving constructed affine merge candidates

Figure 2 From non-adjacent neighbors to constructed affine merge candidates

Within the same scope of such EE test (i.e., non-adjacent spatial neighbors for affine mode), the following two modifications are further introduced in EE2-3.11 compared to [5]:

- Besides the affine merge mode, the NSAM candidates are also applied to affine AMVP mode.

- One second type of constructed affine candidates that are derived from non-adjacent neighbors are introduced for affine merge mode. Those new constructed affine candidates are derived based on the same affine candidate construction scheme in [8]. However, instead of using history-based look-up table, the non-translational affine parameters are inherited from the non-adjacent spatial neighbors. Specifically, the second type of affine constructed candidates are generated from the combination of 1) the translational affine parameters of adjacent neighboring 4x4 blocks; and 2) the non-translational affine parameters inherited from the non-adjacent spatial neighbors as defined in Figure 1 (a).

In EE2-3.11, the NSAM candidates are inserted into the existing affine merge candidate list and affine AMVP candidate list according to the following orders:

Affine merge mode:

- SbTMVP candidate, if available

- Inherited from adjacent neighbors

- Inherited from non-adjacent neighbors

- Constructed from adjacent neighbors

- The second type of constructed affine candidates from non-adjacent neighbors

- The first type of constructed affine candidates from non-adjacent neighbors

- Zero MVs

Affine AMVP mode:

- Inherited from adjacent neighbors

- Constructed from adjacent neighbors

- Translational MVs from adjacent neighbors

- Translational MVs from temporal neighbors

- Inherited from non-adjacent neighbors

- Zero MVs

Due to the inclusion of the additional candidates generated by NSAM, in this contribution, the size of the affine merge candidate list is increased from 5 to 15. The subgroup size of the affine merge mode for the adaptive reordering method [7] is increased from 3 to 15.

Simulation results

The proposed NSAM was implemented on top of the ECM-3.1 [3] and tested by following the common test conditions defined in [4]. The performance results are shown in Table 1.

Table 1 Performance of the test EE2-3.11

|  | Over ECM-3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -0.26% | -0.35% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.57% | -0.25% | -0.40% | 101% | 101% |  |  |  |  |  |
| Class B | -0.33% | -0.08% | -0.13% | 100% | 100% | -0.57% | -0.66% | -0.69% | 102% | 101% |
| Class C | -0.22% | 0.02% | -0.10% | 100% | 101% | -0.49% | -0.39% | -0.41% | 101% | 102% |
| Class E |  |  |  |  |  | -0.23% | 0.45% | -0.45% | 101% | 100% |
| Overall | -0.35% | -0.12% | -0.22% | 100% | 100% | -0.46% | -0.29% | -0.53% | 101% | 101% |
| Class D | -0.25% | -0.18% | -0.02% | 101% | 101% | -0.74% | -0.22% | -0.97% | 101% | 101% |
| Class F | -0.23% | -0.27% | -0.28% | 100% | 102% | -0.55% | -0.53% | 0.21% | 101% | 102% |

As complementary information, the performance of increasing the size of the affine merge candidate list and the subgroup size of the adaptive reordering to 15 without the proposed method is shown in Table 2.

Table 2 Performance of only increasing the size of affine merge candidate list and subgroup size of adaptive reordering method

|  | Over ECM-3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.11% | -0.05% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.01% | 0.03% | 0.03% | 101% | 100% |  |  |  |  |  |
| Class B | 0.00% | -0.06% | 0.12% | 100% | 100% | -0.04% | -0.27% | -0.23% | 101% | 101% |
| Class C | 0.01% | 0.05% | 0.14% | 100% | 99% | -0.02% | 0.03% | 0.02% | 101% | 101% |
| Class E |  |  |  |  |  | 0.05% | 0.56% | -0.26% | 101% | 101% |
| Overall | 0.00% | -0.02% | 0.07% | 100% | 100% | -0.01% | 0.04% | -0.16% | 101% | 101% |
| Class D | -0.02% | -0.03% | 0.14% | 101% | 100% | 0.17% | -0.03% | -0.58% | 101% | 100% |
| Class F | 0.02% | 0.06% | -0.01% | 100% | 100% | 0.22% | 0.10% | 0.15% | 101% | 100% |

Table 3 illustrates the BD-rate performance of the proposed NSAM scheme where the second type of constructed affine candidates are completely removed.

Table 3 Performance of the test EE2-3.11 without the second type of constructed affine candidates

|  | Over ECM-3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.19% | -0.27% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.46% | -0.29% | -0.32% | 101% | 102% |  |  |  |  |  |
| Class B | -0.27% | -0.12% | -0.09% | 100% | 100% | -0.47% | -0.48% | -0.45% | 101% | 101% |
| Class C | -0.16% | -0.11% | -0.04% | 100% | 99% | -0.37% | -0.33% | -0.26% | 101% | 100% |
| Class E |  |  |  |  |  | -0.25% | 0.33% | -0.56% | 101% | 100% |
| Overall | -0.28% | -0.16% | -0.16% | 100% | 100% | -0.39% | -0.23% | -0.42% | 101% | 101% |
| Class D | -0.18% | -0.13% | -0.02% | 101% | 100% | -0.50% | -0.02% | -0.87% | 102% | 102% |
| Class F | -0.19% | -0.08% | -0.21% | 100% | 100% | -0.40% | -0.56% | -0.44% | 101% | 102% |

Conclusions

This contribution reports the test results of EE2-3.11 on using non-adjacent spatial neighbors for affine merge and AMVP modes. Compared to the ECM3.1 anchors, the proposed method demonstrates elegant coding gains without obvious encoding/decoding run-time increase. It is recommended to adopt the proposed scheme into the next version of ECM software.

References

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10)”, JVET-S2001, June 2020, Teleconference.

M. Coban, F. Le Léannec, M. Sarwer, J. Ström, “Algorithm description of Enhanced Compression Model 3 (ECM3)”, JVET-X2025, October, 2021, Teleconference.

ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/jvet-x-ee2/ECM.

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, October, 2021.

W. Chen, X. Xiu, Y.-W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang, “AHG12: Non-adjacent spatial neighbors for affine merge mode”, JVET-X0151, October, 2021, Teleconference.

V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-X2024, October, 2021, Teleconference.

N. Zhang, K. Zhang, L. Zhang, H. Liu, Z. Deng, Y. Wang, “EE2-3.1/EE2-3.2: Adaptive Reordering of Merge Candidates with Template/Bilateral Matching”, JVET-W0090, July, 2021, Teleconference.

K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang, “Non-EE2: History-based Affine Model Inheritance”, JVET-X0088, October, 2021, Teleconference.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).