| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0155-v1 |
| --- | --- |

| Title: | AHG10: Fixes and clean up for temporal prefilter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada, Ltd. |  |  |

_____________________________

# Abstract

Multiple bugs and inconsistencies in the temporal prefilter are reported and remedies are proposed. These relate mainly to boundary conditions at the beginning and end of sequences and to the computation of a noise estimate. The combined impact of the remedies under VTM SDR CTC is nil for the All Intra configuration, about -0.2% BD-rate difference for the Random Access configuration, and about -0.1% BD-rate difference for Low Delay configurations.

# Problem Statement

The following issues were found in the temporal prefilter code:

- In the random access configuration, for each frame P to be encoded, the prefilter considers 8 frames Qi=P-4..P-1,P+1..P+4. If one or more of the considered frames Qi does not exist, the behaviour depends on their position: if Qi precedes the first frame in a sequence, Qi is not considered in the filtering process; if Qi follows the last frame in a sequence, no prefilter is applied to frame P. This inconsistency is undesirable as it leads to some frames not being filtered.

- When deriving a noise estimate for each motion-compensated block in a frame, the vertical position inside a target block is ignored (see code in section 1.1). This results in an incorrect calculation.

- When deriving a noise estimate for each 8x8 motion-compensated block in a frame, not all samples within the block are considered. A bias towards the top-left corner exists and is undesirable. For example, the variable variance is computed as i=06j=06δij2 and values for i=7 and j=7 are ignored.

- Three sets of frame weightings are provided in the array m_refStrengths. The first set is used when 8 reference frames are available, the second set when 4 reference frames are available, and the third set otherwise. This arrangement seems to have outlived its original purpose with the addition of more reference frames and of one-sided filtering (e.g., for low-delay configurations). It should thus be updated.

- The number of frames considered by the temporal prefilter is hardcoded and hidden from the user. It would be desirable to expose this information and make it configurable.

## Noise estimation code

In the following, the derivation of pix, pixR, and pixD does not consider y1 (note that srcPel remains constant throughout the loop), whereas the derivation of ref, refR, and refD does consider y1:

double variance = 0, diffsum = 0;

for (int y1 = 0; y1 < blockSizeY - 1; y1++)

{

for (int x1 = 0; x1 < blockSizeX - 1; x1++)

{

int pix  = *(srcPel + x1);

int pixR = *(srcPel + x1 + 1);

int pixD = *(srcPel + x1 + srcStride);

int ref  = *(correctedPics[i].bufs[c].buf + ((y + y1) * correctedPics[i].bufs[c].stride + x + x1));

int refR = *(correctedPics[i].bufs[c].buf + ((y + y1) * correctedPics[i].bufs[c].stride + x + x1 + 1));

int refD = *(correctedPics[i].bufs[c].buf + ((y + y1 + 1) * correctedPics[i].bufs[c].stride + x + x1));

int diff  = pix  - ref;

int diffR = pixR - refR;

int diffD = pixD - refD;

variance += diff * diff;

diffsum  += (diffR - diff) * (diffR - diff);

diffsum  += (diffD - diff) * (diffD - diff);

}

}

srcFrameInfo[i].mvs.get(x / blockSizeX, y / blockSizeY).noise = (int) round((300 * variance + 50) / (10 * diffsum + 50));

# Proposed solution

It is proposed to treat the beginning and the end of a sequence in a same manner: if any of the considered frames is not within a valid range, it is not used, but the filter is applied to all coded frames. The definition of which frames are considered “in range” for filtering purposes can be refined to exclude frames that contains copyright notices. Parameters FirstValidFrame and LastValidFrame are introduced for each test sequence to define a valid frame range which may differ from the actual number of frames present in the source file.

It is further proposed to modify the noise estimation code to correctly consider the vertical position y1 within a block and to consider all samples within a block. The variable variance is computed as i=07j=07δij2 instead of i=06j=06δij2, and the variable diffsum as i=07j=06δij-δi,j+12+i=06j=07δij-δi+1,j2 instead of i=06j=06δij-δi,j+12+i=06j=06δij-δi+1,j2. The variance is further weighted by a factor 78 to account for the change in the number of summed elements.

The proposed modified code is shown below:

double variance = 0, diffsum = 0;

const ptrdiff_t refStride = correctedPics[i].bufs[c].stride;

const Pel *     refPel    = correctedPics[i].bufs[c].buf + y * refStride + x;

for (int y1 = 0; y1 < blockSizeY; y1++)

{

for (int x1 = 0; x1 < blockSizeX; x1++)

{

const Pel pix  = *(srcPel + srcStride * y1 + x1);

const Pel ref  = *(refPel + refStride * y1 + x1);

const int diff = pix - ref;

variance += diff * diff;

if (x1 != blockSizeX - 1)

{

const Pel pixR  = *(srcPel + srcStride * y1 + x1 + 1);

const Pel refR  = *(refPel + refStride * y1 + x1 + 1);

const int diffR = pixR - refR;

diffsum += (diffR - diff) * (diffR - diff);

}

if (y1 != blockSizeY - 1)

{

const Pel pixD  = *(srcPel + srcStride * y1 + x1 + srcStride);

const Pel refD  = *(refPel + refStride * y1 + x1 + refStride);

const int diffD = pixD - refD;

diffsum += (diffD - diff) * (diffD - diff);

}

}

}

const int cntV = blockSizeX * blockSizeY;

const int cntD = 2 * cntV - blockSizeX - blockSizeY;

srcFrameInfo[i].mvs.get(x / blockSizeX, y / blockSizeY).noise =

(int) round((15.0 * cntD / cntV * variance + 5.0) / (diffsum + 5.0));

It is further proposed to reduce the number of entries in the array m_refStrengths such that one set of weights is used for random-access configurations (when past and futures frames are used for filtering) and one set for the low-delay configurations (when only past frames are used for filtering). This change impacts only a few frames in corner cases towards the start and end of test sequences.

It is further proposed to make the number of frames used by the temporal filter configurable using parameters such as TemporalFilterPastRefs and TemporalFilterFutureRefs that indicate the number of past and future frames used by the temporal filter. These can replace TemporalFilter and TemporalFilterFutureReference.

Constants used in the prefilter may need retuning with the proposed fixes. No such effort was carried out here.

# Results

The biggest impact comes from fixing the use of the vertical position in the noise computation. By itself, this fix yields the following results under JVET SDR CTC.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.41% | -0.58% | 101% | 103% |
| Class A2 | -0.19% | -0.49% | -0.45% | 103% | 104% |
| Class B | -0.37% | -0.78% | -0.72% | 99% | 100% |
| Class C | -0.05% | -0.41% | -0.30% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.55% | -0.53% | 101% | 101% |
| Class D | 0.04% | -0.38% | -0.09% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.46% | -0.42% | 105% | 103% |
| Class C | 0.05% | -0.33% | -0.22% | 104% | 106% |
| Class E | 0.04% | -0.37% | -0.24% | 101% | 103% |
| Overall | -0.02% | -0.40% | -0.31% | 103% | 104% |
| Class D | 0.14% | -0.51% | -0.16% | 97% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 104% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | -0.31% | -0.78% | 104% | 104% |
| Class C | 0.02% | 0.03% | -0.29% | 100% | 102% |
| Class E | -0.08% | -0.14% | 0.56% | 95% | 97% |
| Overall | -0.12% | -0.16% | -0.28% | 101% | 102% |
| Class D | 0.03% | -0.59% | -0.27% | 99% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |

The combination of all proposed modification yields the following results under JVET SDR CTC.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 46% | 56% |
| Class A2 | 0.00% | 0.00% | 0.00% | 47% | 57% |
| Class B | 0.00% | 0.00% | 0.00% | 48% | 56% |
| Class C | 0.00% | 0.00% | 0.00% | 50% | 58% |
| Class E | 0.00% | 0.00% | 0.00% | 48% | 54% |
| Overall | 0.00% | 0.00% | 0.00% | 48% | 56% |
| Class D | 0.00% | 0.00% | 0.00% | 51% | 55% |
| Class F | 0.00% | 0.00% | 0.00% | 46% | 57% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.58% | -0.72% | 51% | 57% |
| Class A2 | -0.16% | -0.62% | -0.49% | 54% | 59% |
| Class B | -0.38% | -0.87% | -0.83% | 53% | 59% |
| Class C | -0.03% | -0.54% | -0.28% | 53% | 60% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.67% | -0.59% | 53% | 59% |
| Class D | 0.00% | -0.58% | -0.21% | 53% | 63% |
| Class F | 0.00% | 0.00% | 0.00% | 50% | 56% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.37% | -0.51% | 56% | 59% |
| Class C | 0.03% | -0.10% | -0.24% | 56% | 63% |
| Class E | -0.07% | -0.10% | -0.45% | 53% | 57% |
| Overall | -0.06% | -0.21% | -0.40% | 55% | 60% |
| Class D | 0.10% | -0.44% | 0.03% | 54% | 63% |
| Class F | 0.00% | 0.00% | 0.00% | 54% | 58% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-15.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.23% | -0.43% | -0.80% | 55% | 60% |
| Class C | -0.01% | 0.25% | -0.31% | 53% | 59% |
| Class E | -0.19% | 0.46% | 0.63% | 50% | 54% |
| Overall | -0.15% | 0.02% | -0.28% | 53% | 58% |
| Class D | 0.01% | 0.01% | -0.12% | 52% | 59% |
| Class F | 0.00% | 0.00% | 0.00% | 51% | 55% |

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).