| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0156-v1 |
| --- | --- |

| Title: | AHG9: SEI message with sample phase indication for consistent rendering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen Andrew Segall | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

An SEI message containing a sample phase indication is proposed. It is asserted to enable more consistent rendering, in particular when rendering at a higher resolution and when switching between multiple resolutions as is common in streaming scenarios where bandwidth may fluctuate.

# Problem Statement

When downsampling video content to generate multiple versions suitable for various bandwidth constraints, a low-pass filter is typically used. A phase shift may be introduced during downsampling, and the relative location between samples at different resolutions may not be known to a client consuming the video. When switching between multiple resolutions a slight geometric shift may be noticeable if the phase shift used in the upsampler does not mirror the phase shift of the downsampler. This issue was previously reported in [1].

Even when content is made available in a single resolution to a client, the client may upsample the content when the display has a higher resolution. Knowing the intended position of samples relative to the video surface also allows for more consistent rendering, which may be useful when multiple videos are rendered to a same screen and require precise alignment.

When rendering a decoded frame using a shader, for example using OpenGL, an offset may be added to texture coordinates to compensate for any phase shift. This is similar to offsets that may be added when accessing chroma samples to account for the position of chroma samples relative to luma samples.

# Proposed solution

An SEI message is proposed with the syntax and semantics described below.

| phase_indication( payloadSize ) { | Descriptor |
| --- | --- |
| hor_phase_num | u(8) |
| ver_phase_num | u(8) |
| phase_den | u(8) |
| } |  |

hor_phase_num and phase_den specify the horizontal position of luminance sampling locations relative to a display window. If phase_den is equal to 0, then the horizontal position is undefined. Otherwise, if phase_den is larger than 0, the horizontal position hor_phase_num ÷ phase_den is expressed in units of the horizontal distance between two horizontally adjacent luminance sampling locations. hor_phase_num shall be larger than or equal to 0 and smaller than or equal to phase_den.

ver_phase_num and phase_den specify the vertical position of luminance sampling locations relative to a display window. If phase_den is equal to 0, then the vertical position is undefined. Otherwise, if phase_den is larger than 0, the vertical position ver_phase_num ÷ phase_den is expressed in units of the vertical distance between two vertically adjacent luminance sampling locations. ver_phase_num shall be larger than or equal to 0 and smaller than or equal to phase_den.

Note: the phase indicators may be used during the rendering process. For example, if using a shader as in OpenGL, texture coordinates may be offset by an amount proportional to the signaled horizontal and vertical phase indicators.

The mapping between the cases in [1] and the proposed syntax is given below:

| JVET-X0092 | hor_phase_num | ver_phase_num | phase_den |
| --- | --- | --- | --- |
| Unknown (0) | 0 | 0 | 0 |
| Co-sited (1) | b | b | 2a |
| Centered (2) | 1 | 1 | 2 |

Where the fraction ab represents the ratio between an original frame size where samples are “centered” relative to the image surface and a current frame size (downsampled).

# References

- J.Samuelsson, A.Tourapis, D.Podborski, K.Rapaka, D.Singer, “Down-sample phase indication (SEI message),” JVET-X0092, October 2021.

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).