| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0159-v2 |
| --- | --- |

| Title: | EE2-related: inter MTS refinement on adaptive intra MTS (EE2-4.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Tomohiro Ikai | Tel: Email: | tomonori.hashimoto@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp corporation |  |  |

_____________________________

- Abstract

This contribution proposes to improve inter MTS aspect of Adaptive intra MTS (AMTS) in EE2-4.4a by restricting AMTS’s candidate number increase in the case of inter MTS. It is noted that AMTS (EE2-4.4a) extends the number of MTS (Multiple transform selection) candidate up to 6:

- VVC         4

- ECM         4 (but the MTS candidate can be chosen from more transforms)

- AMTS        6

It is reported the simulation results of proposed method compared to the ECM-3.1 with inter MTS is:

- RA: BD-rate YUV: -0.06%, -0.12%, -0.02%; EncT: 99%; DecT: 100%

- LDB: BD-rate YUV: -0.23%, -0.22%, -0.47%; EncT: 103%; DecT: 100%

It is recommended to adopt method1 because it is asserted to show best performance without any encoder time increase.

In v2, results are updated.

Introduction

In EE2, Adaptive intra MTS (AMTS) [1] has been studied, which switches the number of MTS candidate up to 6 depends on the sum of coefficient values.

In EE, intra MTS has been studied but inter MTS has not been tested since it is disabled in CTC. Figure 1 shows the current situation of MTS in EE2-4.4a (ATMS). It is noted the provided ATMS software only supports intra MTS and the software causes an encoder-decoder mismatch when inter MTS is enabled.

Figure 1. Current situation of MTS

This contribution investigates an optimal method for inter MTS and proposes to leave the number of inter MTS candidates as four (fixed number). i.e. No changes on inter MTS compared to the current ECM.

The following table shows the performance of inter MTS in ECM-3.1. It is noted that other encoder-decoder mismatch problem (see ECM bug tracker Issue #10) is also investigated in the authors and a merge request !56 is also provided.

Performance of inter MTS

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | 0.42% | 0.54% | 122% | 100% |
| Class A2 | -0.54% | 0.60% | 0.55% | 121% | 100% |
| Class B | -0.30% | 0.63% | 0.66% | 119% | 100% |
| Class C | -0.21% | 0.70% | 0.64% | 120% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | 0.60% | 0.61% | 120% | 100% |
| Class D | -0.19% | 0.65% | 0.70% | 118% | 100% |
| Class F | -0.01% | 0.39% | 0.22% | 114% | 98% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.56% | 0.66% | 0.89% | 125% | 99% |
| Class C | -0.31% | 1.18% | 0.94% | 133% | 98% |
| Class E | -0.08% | 0.95% | 1.18% | 120% | 99% |
| Overall | -0.36% | 0.90% | 0.98% | 126% | 99% |
| Class D | -0.27% | 0.57% | 0.15% | 125% | 97% |
| Class F | -0.08% | 0.83% | 0.82% | 120% | 98% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |

Proposed method

We implemented and tested three methods to achieve the best performance and complexity balance and propose the method1 as an adoption candidate.

|  | Intra MTS | Inter MTS |
| --- | --- | --- |
| Method1 (Proposal) | #MTScands = {1,4,6} | #MTScands = 4 |
| Method2 | #MTScands = {1,4,6} | #MTScands = {1,4} |
| Method3 | #MTScands = {1,4,6} | #MTScands = {1,4,6} |

Method 1 proposes to leave the number of inter MTS candidates as four (Figure 2) It is noted that since inter MTS in ECM uses four candidates as VVC, no encoding time increase is required.

Figure 2. Leaving the number of inter MTS candidates as four

Method 2 applies adaptive MTS candidates for inter MTS without six MTS candidates (Figure 3).

Figure 3. Adaptive MTS for inter MTS without six MTS candidates

Method 3 applies adaptive MTS for both intra and inter, which also solves the mismatch problem (Figure 4).

Figure 4. Adaptive MTS for inter MTS

Experiment

The following table summarizes the number of MTS candidates in inter MTS for each methods and anchors.

|  | Intra MTS | Inter MTS |
| --- | --- | --- |
| Anchor#1 (CTC, --MTS=1) | #MTScands = {4} | off |
| Anchor#2 (--MTS=3) | #MTScands = {4} | #MTScands = {4} |
| Method1 | #MTScands = {1,4,6} | #MTScands = {4} |
| Method2 | #MTScands = {1,4,6} | #MTScands = {1,4} |
| Method3 | #MTScands = {1,4,6} | #MTScands = {1,4,6} |

The implementations are done on top of EE2-4.4a software. To conduct inter MTS simulation, a software fix is implemented by the authors (See merge request !56 and Issue #10 in ECM bug tracker).

The simulations are conducted following common test conditions in JVET-X2017[2] with following encoder option:

--MTS=3  // Enable both Intra and Inter

Simulation results

Anchor: ECM-3.1 (inter MTS is disabled as the same as CTC)

The following tables show results of three methods against ECM-3.1 (anchor#1).

Method1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | 0.33% | 0.45% | 122% | 100% |
| Class A2 | -0.58% | 0.56% | 0.52% | 119% | 100% |
| Class B | -0.39% | 0.54% | 0.62% | 119% | 100% |
| Class C | -0.22% | 0.48% | 0.71% | 118% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.37% | 0.48% | 0.59% | 120% | 100% |
| Class D | -0.24% | 0.87% | 0.31% | 113% | 100% |
| Class F | -0.01% | 0.24% | 0.18% | 112% | 99% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.73% | 0.31% | 0.78% | 130% | 99% |
| Class C | -0.53% | 0.80% | 0.51% | 136% | 98% |
| Class E | -0.44% | 1.16% | -0.01% | 123% | 99% |
| Overall | -0.59% | 0.68% | 0.49% | 130% | 99% |
| Class D | -0.28% | 0.92% | 0.50% | 129% | 98% |
| Class F | -0.23% | 0.21% | 0.94% | 123% | 99% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |

Method2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | 0.25% | 0.39% | 122% | 100% |
| Class A2 | -0.53% | 0.42% | 0.45% | 118% | 100% |
| Class B | -0.34% | 0.40% | 0.63% | 118% | 100% |
| Class C | -0.21% | 0.34% | 0.54% | 117% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | 0.36% | 0.52% | 118% | 100% |
| Class D | -0.24% | 0.71% | 0.41% | 114% | 101% |
| Class F | -0.07% | 0.27% | 0.13% | 111% | 99% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.71% | 0.25% | 0.55% | 128% | 99% |
| Class C | -0.50% | 0.67% | 0.56% | 131% | 98% |
| Class E | -0.44% | 0.95% | -0.14% | 121% | 99% |
| Overall | -0.57% | 0.57% | 0.38% | 127% | 99% |
| Class D | -0.30% | 0.84% | 0.55% | 126% | 98% |
| Class F | -0.02% | 0.02% | 1.38% | 119% | 99% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |

Method3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.47% | 0.39% | 124% | 100% |
| Class A2 | -0.53% | 0.45% | 0.41% | 120% | 100% |
| Class B | -0.33% | 0.42% | 0.58% | 121% | 100% |
| Class C | -0.19% | 0.41% | 0.59% | 120% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | 0.43% | 0.51% | 121% | 100% |
| Class D | -0.22% | 0.92% | 0.28% | 113% | 101% |
| Class F | -0.04% | 0.26% | 0.04% | 113% | 99% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.65% | 0.17% | 0.66% | 132% | 99% |
| Class C | -0.48% | 0.54% | 0.35% | 139% | 98% |
| Class E | -0.40% | 0.45% | 0.29% | 126% | 99% |
| Overall | -0.53% | 0.36% | 0.47% | 133% | 99% |
| Class D | -0.33% | 0.58% | 0.63% | 131% | 98% |
| Class F | -0.16% | -0.06% | 1.06% | 125% | 98% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |

Anchor: ECM-3.1 (inter MTS is enabled)

This section shows the same test results compared to different anchor (anchor#2).

The following tables show above results against inter MTS enabled in tests and anchor.

Method1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 + interMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.10% | -0.09% | 100% | 100% |
| Class A2 | -0.04% | -0.04% | -0.03% | 98% | 100% |
| Class B | -0.09% | -0.09% | -0.04% | 100% | 100% |
| Class C | -0.01% | -0.22% | 0.06% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.12% | -0.02% | 99% | 100% |
| Class D | -0.04% | 0.22% | -0.39% | 96% | 100% |
| Class F | 0.00% | -0.15% | -0.04% | 98% | 100% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 + interMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.35% | -0.10% | 104% | 100% |
| Class C | -0.22% | -0.37% | -0.43% | 102% | 100% |
| Class E | -0.36% | 0.21% | -1.16% | 103% | 100% |
| Overall | -0.23% | -0.22% | -0.47% | 103% | 100% |
| Class D | -0.01% | 0.35% | 0.35% | 103% | 101% |
| Class F | -0.15% | -0.61% | 0.12% | 103% | 101% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |

Method2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 + interMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.17% | -0.15% | 99% | 100% |
| Class A2 | 0.01% | -0.17% | -0.11% | 98% | 100% |
| Class B | -0.04% | -0.23% | -0.03% | 98% | 100% |
| Class C | 0.00% | -0.36% | -0.10% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.24% | -0.09% | 98% | 100% |
| Class D | -0.04% | 0.06% | -0.29% | 96% | 100% |
| Class F | -0.06% | -0.11% | -0.09% | 97% | 100% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 + interMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -0.40% | -0.33% | 103% | 100% |
| Class C | -0.20% | -0.51% | -0.38% | 99% | 100% |
| Class E | -0.36% | 0.00% | -1.28% | 101% | 100% |
| Overall | -0.22% | -0.34% | -0.58% | 101% | 100% |
| Class D | -0.03% | 0.27% | 0.40% | 101% | 101% |
| Class F | 0.06% | -0.79% | 0.57% | 100% | 101% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |

Method3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 + interMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | -0.15% | 101% | 100% |
| Class A2 | 0.02% | -0.15% | -0.14% | 99% | 100% |
| Class B | -0.03% | -0.21% | -0.08% | 101% | 100% |
| Class C | 0.02% | -0.28% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.17% | -0.10% | 101% | 100% |
| Class D | -0.03% | 0.26% | -0.42% | 96% | 100% |
| Class F | -0.03% | -0.12% | -0.18% | 99% | 100% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 + interMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.48% | -0.21% | 106% | 100% |
| Class C | -0.17% | -0.63% | -0.58% | 105% | 100% |
| Class E | -0.32% | -0.49% | -0.86% | 105% | 100% |
| Overall | -0.17% | -0.54% | -0.50% | 105% | 100% |
| Class D | -0.06% | 0.02% | 0.49% | 105% | 101% |
| Class F | -0.07% | -0.88% | 0.25% | 104% | 100% |
| Class TGM | ###### | ###### | ###### | #DIV/0! | #DIV/0! |

Conclusion

In this contribution, several inter MTS methods are tested on top of AMTS (EE2-4.4a). Simulation results show proposed method1 (four constant candidates) is better than the other methods. It is recommended to leave the number of inter MTS candidates as four (method1).

References

[1] B. Ray, V. Seregin, M. Karczewicz, "Non-EE2: Adaptive Intra MTS", JVET-X0135, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 24th Meeting, by teleconference

[2] M. Karczewicz, Y. Yen, "Common Test Conditions and evaluation procedures for enhanced compression tool testing", JVET-X2017.

[3] B. Ray, V. Seregin, M. Karczewicz, "EE2-4.4: adaptive intra MTS," JVET-Y0142.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).