| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0161-v2 |
| --- | --- |

| Title: | EE2-3.12-related: Extensions of history-parameter-based affine model inheritance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng Na Zhang, Yang Wang 8910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents additional results for history-parameter-based affine model inheritance in test EE2-3.12. Based on the code of EE2-3.12, some extensions are proposed as below:

Aspect #1: The size of sub-block-based merge candidate list is increased to 15, and all the affine merge candidates are involved in the adaptive reordering merge candidates (ARMC) process.

Aspect #2: The number of neighboring blocks, which are used to generate affine merge candidates jointly with affine models stored in tables, are increased from five to seven.

Aspect #3: A second history-parameter table with base MV information is appended. And the history-parameter tables stored in the neighbouring CTU above and above-right to the current CTU can also be used to generate history-affine-parameter-based candidates.

Aspect #4: Pair-wised affine merge candidates are generated by two affine merge candidates which are history-derived or not history-derived.

Two tests are conducted:

Test #1: EE2-3.12 + Aspect #1

Test #2: EE2-3.12 + Aspect #1~#4

On top of ECM-3.1, simulation results of the proposed methods are reported as below:

Test #1 :RA: {-0.26%, -0.19%, -0.15%, 100%, 100%}; LB: {-0.25%, -0.06%, -0.17%, 100%, 100%}.

Test #2 :RA: {}; LB: {}.

# Introduction

History-parameter-based affine model inheritance (HAMI) was proposed in JVET-X0088 [1] and tested in EE2-3.12a. In the test of EE2-3.12a, HAMI is implemented following the way as described in the EE description JVET-X2024 [2], which may limit the performance of HAMI.

# Proposed methods

This contribution presents additional results for history-parameter-based affine model inheritance in test EE2-3.12. Based on the code of EE2-3.12, some extensions are proposed:

Aspect #1:

The size of sub-block-based merge candidate list is increased to 15, and all the affine merge candidates are involved in the adaptive reordering merge candidates (ARMC) process.

Aspect #2:

The number of neighboring blocks, which are used to generate affine merge candidates jointly with affine models stored in tables, are increased from five to seven. As shown in Fig.1, A2 and B3 are the two additional neighbouring blocks.

Aspect #3:

A second history-parameter table (HPT) with base MV information is appended. And the history-parameter tables stored in the CTU above/above-right to the current CTU can be used by blocks in the current CTU.

There are nine entries in the second HPT, wherein an entry comprises a base MV, a reference index and four affine parameters for each reference list, and a base position, an additional history-affine-parameter-based candidates (HAPC) can be generated from the second HPT with the base MV information the corresponding affine models stored in an entry. The difference between the first HPT and the second HPT is illustrated in Fig. 2.

Besides HPTs updated online along the current CTU row, the HPTs stored in the neighbouring CTUs above and above-right to the current CTU can also be used to generate HAPCs for a block. After coding/decoding a CTU, the HPT will be stored for the usage in the next CTU row, as shown in Fig. 3.

Aspect #4:

Pair-wised affine merge candidates are generated by two affine merge candidates which are history-derived or not history-derived. A pair-wised affine merge candidates is generated by averaging the CPMVs of existing affine merge candidates in the list.

Fig. 1 Seven neighbouring blocks.

Fig. 2 The first HPT and the second HPT.

Fig. 3 The first HPT and the second HPT.

Two tests are conducted:

Test #1: EE2-3.12 + Aspect #1

Test #2: EE2-3.12 + Aspect #1~#4

# Experimental results

The proposed methods are tested on top of ECM-3.1 [3], according to the common test conditions defined in [4]. The simulation results are summarized in table 1 and table 2 for the two tests.

Table 1: Performance of the Test #1

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.28% | -0.28% | 102% | 102% |  |  |  |  |  |
| Class A2 | -0.43% | -0.23% | -0.31% | 102% | 103% |  |  |  |  |  |
| Class B | -0.24% | -0.15% | -0.07% | 100% | 101% | -0.34% | -0.29% | -0.10% | 100% | 100% |
| Class C | -0.16% | -0.12% | -0.03% | 98% | 97% | -0.23% | 0.09% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  | -0.14% | 0.13% | -0.44% | 100% | 99% |
| Overall | -0.26% | -0.19% | -0.15% | 100% | 100% | -0.25% | -0.06% | -0.17% | 100% | 100% |
| Class D | -0.17% | 0.03% | -0.04% | 100% | 96% | -0.32% | 0.23% | -0.14% | 101% | 100% |
| Class F | -0.11% | -0.10% | -0.11% | 99% | 97% | -0.31% | -0.30% | -0.12% | 100% | 96% |

Table 2: Performance of the Test #2

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.33% | -0.27% | -0.19% | 100% | 101% |  |  |  |  |  |
| Class C | -0.20% | -0.01% | -0.14% | 98% | 99% | -0.27% | -0.37% | -0.06% | 99% | 101% |
| Class E |  |  |  |  |  | -0.19% | 0.04% | -0.44% | 99% | 100% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.27% | -0.04% | -0.05% | 99% | 99% | -0.53% | 0.11% | -0.87% | 97% | 99% |
| Class F | -0.22% | -0.20% | -0.28% | 99% | 100% |  |  |  |  |  |

# Conclusions

This contribution presents additional EE-related test results for history-parameter-based affine model inheritance. The proposed method contributes promising coding gains with negligible complexity overhead. It is recommended to adopt it into ECM.

# References

- K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang, “Non-EE2: History-parameter-based Affine Model Inheritance,” JVET-X0088, Oct. 2021.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-X2024, Oct. 2021.

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-3.1.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).