| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0163-r1 |
| --- | --- |

| Title: | AHG 7: GDR without encoder constraints for ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang Seungwook Hong Krit Panusopone | Email: | limin.2.wang@nokia.com seungwook.hong@nokia.com krit.panusopone@nokia.com |
|  | Miska Hannuksela |  | miska.hannuksela@nokia.com |
|  | Jani Lainema |  | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents a new GDR implementation for ECM [1]. With this new GDR implementation, many constraints that have to be imposed on coding tools under the current VVC standard can be removed, which helps to reduce the code size for ECM significantly. In addition, this new GDR implementation provides great flexibility in future tool development for ECM.

Simulations were conducted under LDP configuration specified by LLCC AhG [2]. The results demonstrate that

- Without turning on ECM new coding tools, the new GDR performs better than VTM-like GDR [2] by x.xx%, and

- With ECM coding tools (except new loop-filter related coding tools), the new GDR has a small loss against the anchor by y.yy%.

# Introduction

Versatile Video Coding (VVC) is a new international video coding standard. One of functionalities that VVC supports is Gradual Decoding Refresh (GDR). As the latest video coding standard, VVC employs many new and advanced coding tools. Many of them however may cause leaks for GDR. Hence, constraints have to be imposed on those coding tools, which not only increases the complexity of coding process, but also affects the overall coding performance.

Enhanced Compression Model (ECM) introduces many new coding features and tools on top of VVC. It becomes even more challenging to enable GDR implementation for ECM because additional constraints will have to be imposed on those new coding tools.

# Proposed Solution

To remove the constraints on coding tools for ECM, alleviate the development burden and reduce the code size, a new GDR implementation is proposed as follows.

In the new GDR implementation,

- All types of partitions within a current CTU are allowed, but no CU should span both the clean and dirty areas,

- For CUs in clean area of a current picture, the reconstructed pixels in dirty areas of the current picture and reference pictures are considered as “not available”, and if needed, they are set to 2BD-1 , where BD is bit depth, which will give prediction for CUs in clean area of the current picture more freedom over the current picture and reference pictures.

- For CUs in clean area of a current picture, coding information in dirty areas of the current picture and reference pictures are considered “not available”, which will prevent CUs in clean area of the current picture from using coding information in dirty area of the current picture and reference pictures.

- CUs in dirty area of the current picture are allowed to use the reconstructed pixels and the coding information of both clean and dirty areas of the current picture and reference pictures.

.

# Experimental Results

The new GDR was implemented on ECM 3.0. Simulations were conducted over JVET test sequences using low-delay P configuration specified by LLCC AhG [2] with the following setting,

- The first picture is coded as an IDR picture,

- A GDR picture is inserted every (frame rate + 4) pictures,

- GDR period (between two consecutive GDR pictures) is set to (frame rate + 4),

- GOP interval (between GDR picture and associated recovery point picture) is set to frame rate,

- Each inter picture refers up to 4 immediately preceding pictures in decoding order,

- IntraQPOffset and QPoffset are set equal to 0.

Two sets of GDR tests are performed. One set of tests were conducted with turning off the ECM new coding tools and the other set of tests with enabling all the ECM new coding tools, except the following new loop-filter related coding tools,

- ALF_IMPROVEMENT

- JVET_V0094_BILATERAL_FILTER

- JVET_X0071_CHROMA_BILATERAL_FILTER

- JVET_W0066_CCSAO

- JVET_X0071_LONGER_CCALF

- JVET_X0071_ALF_BAND_CLASSIFIER

All the GDR test bitstreams were checked, and none of them shows leak.

For comparison purpose, VTM-like GDR and anchor bitstreams are also performed using low-delay P configuration specified by LLCC AhG [2]. The VTM-like GDR tests use the same setting as the new GDR without ECM new coding tools. The anchor bitstreams are generated with the following setting,

- The first picture is coded as an IDR picture,

- A CRA picture is inserted every (frame rate +4) pictures,

- CRA period (between two consecutive CRA pictures) is set to (frame rate + 4),

- Each inter picture refers up to 4 immediately preceding pictures in decoding order,

- IntraQPOffset and QPoffset are set equal to 0.

Table 1 summarizes the tests conducted.

Table 1 Tests

|  | With ECM coding tools off (Test 1) | With ECM coding tools on, except 7 new loop-filter tools (Test 2) |
| --- | --- | --- |
| Anchor |  |  |
| VTM-like GDR |  |  |
| New GDR |  |  |

Table 2 shows the performance comparison of new GDR over the VTM-like GDR in terms of BD rate, where ECM new coding tools are off for both VTM-GDR and new GDR. As seen, the new GDR performs better than the VTM-like GDR by an overall performance gain of x.xx %.

Table 2. Simulation Results of new GDR over VTM-like GDR (Test 1)

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-like GDR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.65% | -3.76% | -3.50% | 104% | 106% |
| Class E | -1.11% | -3.05% | -1.84% | 101% | 109% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.86% | -2.62% | -2.94% | 106% | 114% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3 shows the performance comparison of the new GDR over the anchor, where ECM new coding tools, except new loop-filter coding tools, are on for both anchor and new GDR. The overall performance loss of the new GDR against the anchor is xx.%. Note that a loss for GDR should be expected because CUs in clean area do not use any coding information from dirty areas.

Table 3. Simulation Results of new GDR over anchor (Test 2)

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Anchor (ECM 3.0) on Linux |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 1.50% | 3.56% | 4.34% | 103% | 110% |
| Class E | 6.35% | 13.18% | 14.18% | 101% | 113% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 2.08% | 6.28% | 5.70% | 101% | 113% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

The new GDR implementation is presented. With this new GDR implementation, the constraints imposed on coding tools will no longer be necessary, which helps reduce the code size and future development of ECM. The simulation results demonstrate that the new GDR implementation preforms better than the current VTM-like GDR implementation. It is therefore proposed to include the new GDR implementation in ECM.

# References

- M. Coban, F. L. Léannec, and J. Ström, “Algorithm description of Enhanced Compression Model 3 (ECM 3)”, JVET-X2025, Oct. 2021.

- S. Hong, L. Wang, K. Panusopone, “GDR Software”, JVET-U0097, Jan. 2021.

- G. Martin-Cocher, S. Puri, T. Poirier, K. Naser, J. Xu, D. Nicholson, M. Sychev, L. Wang, S. Liu, W. Yang, J.M. Tiesse, and M. Karczewicz, “AHG-7, LLCC Scenarios and baseline configurations”, JVTE -Y0043, Jan. 2022.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).