| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0165 |
| --- | --- |

| Title: | Crosscheck of JVET-Y0053 (AHG9/AHG13: Film grain blending process for film grain characteristics SEI message) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Milos Radosavljevic | Tel: Email: | milos.radosavljevic@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports a cross-check of contribution JVET-Y0053 ”AHG9/AHG13: film grain blending process for film grain characteristics SEI message”. JVET-Y0053 contribution proposes a film grain blending process for film grain characteristics SEI message. The blending process is identical to the process specified in SMPTE RDD5 except the modifications to reduce line buffers and on-chip memory storage. It is reported that the proposed method produces visually the same output compared to the initial implementation, however with the reduced implementation cost.

# Introduction

JVET-Y0053 proposes to change the grain block size from fixed 8x8-size in RDD5 to 8x1 for picture resolutions less than or equal to 1920x1080, 16x1 for resolution greater than 1920x1080 and up to 3840x2160, and 32x1 for resolutions greater than 4K including 8K. According to the proposal, using Nx1 block size eliminates the need of line buffers to compute block average in the existing methods.

# Experiments

Bitstreams provided by CE package are used (Core Experiment on Film Grain Synthesis [1]). Proposed modifications are tested on the CE subjective test set, and on the CE Complexity test set, configuration I10C3.

# Results

Subjective evaluation of the reconstructed sequences with a synthetized film grain does not show visual difference compared to the anchor. The modification of grain block size does not impact the subjective quality of a film grain.

# Conclusions

The results reported in JVET-Y0053 are confirmed. Proposed implementation does not affect visual appearance of a film grain.

# References

[1] CE software package: https://vcgit.hhi.fraunhofer.de/jvet-w-ce/ce-fgs/-/tree/CE-FGS