| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0174-v2 |
| --- | --- |

| Title: | EE2-1.1–related: additional tests on partitioning flexibility |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fabrice Urban, Karam Naser, Franck Galpin Kai Zhang, Li Zhang, Zhipin Deng Na Zhang, Yang Wang 975 Avenue des Champs Blancs, 35510 Cesson-Sévigné, France 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | fabrice.urban@interdigital.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. InterDigital |  |  |

_____________________________

# Abstract

To bring more compression gain, it is possible to relax constraints about partitioning, for example by playing with the split depth. Additional gains can be therefore obtained at the expense of increased encoding run-time. Another track consists in adding new partitioning types, such as ABT or UQT.

This contribution reports experiments results based on these two options, and reportedly shows that enabling new partitions, namely ABT or UQT, leads to better compression gain/encoding run-time trade-offs in use cases where encoding run-time is less constrained.

# Introduction

Compression gain in a video encoder often comes at the cost of an increased number of coding tools that implies a greater rate-distortion search space. Consequently, if the encoder uses an exhaustive Rate-Distortion Optimization (RDO) algorithm, compression gain comes at the cost of an increased encoder complexity. However, such coding tools are often designed to have a limited run-time or complexity impact on the decoder side. Moreover, depending on the use-case, encoder complexity can be of second concern when coding gain is the priority. For example, [1] is trying to improve the encoding performance of the VTM encoder. Furthermore, in most of encoder applications, RDO is a key feature and improves with time [2] when new heuristics are developed to find better encoder gain-complexity trade-off. In this context, encoder complexity does not sum-up to rate-distortion search space size.

During VVC development, it was shown that flexible partitioning was the tool bringing the highest gains on top of HM. Typically, adding BT on top of QT allowed VTM to perform all partitioning done by HM plus some new ones. In the same spirit, ABT also brings new ones.

For each partition type, the complexity point for which the trade-off is the best is indeed different. For example, in VVenC, the fastest configuration only uses QT partition because BT does not lead to a good tradeoff for very low complexity. On the opposite, the slower configuration removes some QT flexibility and increases the MTT flexibility.

Flexible partitioning is a performing solution to bring compression gain. However, even by relaxing CTC’s split depth constrains, it is observed that the compression gain saturates, even for higher encoding time.

For example, in [3] it has been shown that for higher complexity, new partitioning types result in a better trade-off for AI configuration on JEM7 with a CNN based fast split mode decision algorithm (Figure 1).

Figure 1. BD-rate gains in AI configuration, as a function of the relative encoding time (in % of HM) for QTBT and QTBT+ABT. (From  [3])

# Study of compression performances versus partitioning flexibility

The experiment studies the effect of partitioning flexibility on compression performance. Encoding times are also monitored. The parameter MaxMTTHierarchyDepthByTiD has been set to different values for the ECM-3.1 encoder [4]. The configuration parameters are described in Table 1. For obvious reasons of encoding time, the results are presented only for class B sequences for the highest values of MaxMTTHierarchyDepthByTiD. Simulation results summary is presented in Table 2.

Table 1:encoder configuration options

|  | Class A-B |  | Class C-D |  |
| --- | --- | --- | --- | --- |
|  | MaxMTTHierarchyDepth | MaxMTTHierarchyDepthByTid | MaxMTTHierarchyDepth | MaxMTTHierarchyDepthByTid |
| ECM-3.1 | 4 | 3 3 3 3 2 2 | 4 | 3 3 3 3 3 3 |
| ECM-3.1 444433 | 4 | 4 4 4 4 3 3 | 4 | 4 4 4 4 3 3 |
| ECM-3.1 444444 | 4 | 444444 |  |  |
| ECM-3.1 554444 | 4 | 554444 |  |  |
| ECM-3.1 555544 | 4 | 555544 |  |  |
| ABT 333222 | 4 | 333222 | 4 | 333332 |
| ABT | 4 | 333322 | 4 | 333333 |
| ABT 333333 | 4 | 333333 |  |  |
| ABT 443333 | 4 | 443333 |  |  |
| ABT 444433 | 4 | 444433 | 4 | 444433 |
| ABT 444444 | 4 | 444444 |  |  |

By increasing the split hierarchy depth, some compression gain can be obtained, but the gain saturates regardless the complexity allowed for split search algorithm. If we look at Figure 2, we see that compression performances curve has an asymptote when encoding time increases.

We performed the similar experiment with Asymmetric Binary Tree (ABT) partitioning as described in [5]. The configuration parameters are described in Table 1. Simulation results summary is presented in Table 2. In Figure 2, we can see from the rate-distortion curves that compression gain saturation is much less strong than in previous case (without new partitioning type).

Note that these compression performance gains are obtained with no impact on decoder complexity, which just has to be able to handle additional block sizes.

Table 2: compression performances versus encoding time for several configurations

|  | Overall |  | Class B |  |
| --- | --- | --- | --- | --- |
| ECM-3.1 | 0.00% | 100% | 0.00% | 100% |
| ECM-3.1 444433 | -0.91% | 174% | -0.98% | 186% |
| ECM-3.1 444444 | -1.16% | 219% | -1.23% | 226% |
| ECM-3.1 554444 |  |  | -1.37% | 273% |
| ECM-3.1 555544 |  |  | -1.52% | 326% |
| ABT 333222 (AB) | -0.38% | 137% | -0.43% | 133% |
| ABT | -0.68% | 158% | -0.71% | 155% |
| ABT 333333 |  |  | -1.23% | 200% |
| ABT 443333 |  |  | -1.46% | 240% |
| ABT 444433 | #VALUE! | #NUM! | -1.78% | 303% |
| ABT 444444 |  |  | #VALUE! | #NUM! |

- Results for all classes

- Results for class B sequences only

Figure 2: compression gain versus encoding time for ECM-3.1 and ABT in RA configuration. By relaxing the constraints on the partitioning, compression gain saturates even if we increase partition search time in ECM (more visible on c)). Enabling new partitions, permits more gain to be achieved.

# Conclusion

Enabling more partitioning types allows getting more compression performance and has a very limited impact on decoder development and complexity. More partitioning flexibility should be enabled in ECM.

# References

- Dmytro Rusanovskyy, Marta Karczewicz, (Qualcomm), Kenneth Andersson, Rickard Sjöberg, Lukasz Litwic (Ericsson), P. Nikitin, G. Martin-Cocher (InterDigital), Adam Wieckowski, Jens Brandenburg, Benjamin Bross (HHI), “AHG10: VTM encoder configurations for tests targeting improved coding performance”, JVET-Y0126, Jan. 2022.

- A. Wieckowski, J. Brandenburg, C. Bartnik, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, C. Stoffers, I. Zupancic, B. Bross, H. Schwarz, D. Marpe, T. Schierl (HHI), “Update on open, optimized VVC implementations VVenC and VVdeC”, JVET-Y0136,  Jan.2022.

- F. Galpin, F. Racapé, P. Bordes, F. Le Léannec, E. François, “AHG9: CNN-based driving of block partitioning for intra slices encoding”, JVET-J0034, Apr.2018.

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-3.1.

- K. Zhang, L. Zhang, Z. Deng, N. Zhang, Y. Wang, F. Urban, K. Naser, F. Galpin, EE2-1.1: “Tests on unsymmetric partitioning methods,” JVET-Y0150, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).